3GPP TSG-SA WG2 Meeting #16z4 	S2-2408257

# Maastricht, NL, August 19 – 23, 2024	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1161 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General Support for Vertical Federated Learning in TS 23.288. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on several principles listed in conclusion part in clause 8.2 of TR 23.700-84, the proposal specifies the procedure of Vertical Federated Learning among NWDAF(s) and/or AF in TS 23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the general procedure of Vertical Federated Learning among NWDAF(s) and/or AF. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure for Vertical Federated Learning will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X(All new text) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (All new text)

5.X		5GC support Vertical Federated Learning (VFL) among NWDAF and AF

Vertical Federated learning among NWDAF(s) and/or AF(s) is a machine learning technique where no raw data need to be exchanged but some level of coordination is still required when training and inference are performed on local models. In particular, datasets used for each local model need to share the same samples while holding different features.

5GC supports VFL among NWDAFs in a single PLMN and VFL between AF and NWDAF(s) in a single PLMN.

For Vertical Federated Learning supported by 5GC, there is one NWDAF or AF acting as the VFL server and NWDAF(s) and/or AF (s) acting as VFL client. The main functionality includes:

VFL Server:

-        integrates local training results for the local ML model update in VFL training process.

-        coordinates the VFL training process by discovering and selecting VFL clients.

-        (In the VFL inference process) aggregates local inference results from VFL clients to generate the final VFL inference result and sends the final VFL inference result to the consumer.

VFL Client:

-        holds the local dataset and performs local training and inference as asked by VFL Server.

When NWDAF is acting as VFL Server, the NWDAF determines, based on internal logic and the operator's policy, whether or not to use VFL to provide a particular analytics ID. When AF is acting as VFL server, VFL process is associated with internal AF process (i.e., VFL training of models for internal AF process, VFL inference for internal AF process).

Before VFL procedure is initiated by VFL server, the VFL server discovers appropriate VFL client NFs and determines appropriate sample set as described in clause 6.X.2.1 and 6.X.2.4.

When starting a VFL procedure, the VFL server NF is to provide an initial model to each VFL client NF, and then each VFL client NFs computes the intermediate results for their local ML models involved in the VFL training and provides reports with the intermediate results to VFL server. The VFL server aggregates intermediate results from VFL clients, trains a local model, computes intermediate results based on its local ML model, and sends the intermediate results to VFL clients involved in the joint VFL training process. The detailed procedure for VFL is described in clause 6.X.

END of CHANGES