SA WG2 Meeting #S2-164	S2-2408262

19 - 23 August, Maastricht, Netherlands	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5552 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MWAB Authorization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR2 |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the FS_VMR2 conclusion (clause 8.2 of 23.700-06.), this CR proposes to add the MWAB Authorization to 23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding MWAB Authorization clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MWAB Authorization is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.X.Y	MWAB authorization

#### 5.X.Y.1	General

There are two MWAB authorizations, MWAB-UE authorization and MWAB-gNB service authorization which are managed by different entities in the networks. MWAB-UE authorization is based in the MWAB-UE subscription and MWAB-gNB service authorization is controlled by OAM of MWAB-gNB owner PLMN/SNPN.

#### 5.X.Y.2	MWAB-UE authorization

For a MWAB-UE, the subscription information stored in the HPLMN (or SNPN or CH) of MWAB-UE indicates whether it is authorized to operate as MWAB-UE, and the corresponding location and time information as specified in TS 23.502 [3].

The MWAB-UE registers to the Backhaul PLMN/SNPN defined in clause 4.2.2.2 of TS 23.502 [3]. During registration, the MWAB-UE serving PLMN/SNPN (called Backhaul PLMN/SNPN) authorizes the MWAB-UE based on the MWAB-UE subscription and local policy.

The MWAB-UE authorization supports time based or location-based control. For time-based control, existing slice subscription control defined in clause 5.15 can be reused. For location-based control, existing mechanism, e.g. service area restriction, DNN/LADN based control can be reused.

If the MWAB-UE is authorized to operate as MWAB-UE, the AMF store this status in the UE context. If the MWAB-UE is not authorized to operate as MWAB-UE, the MWAB-UE is not allowed to establish the backhaul PDU session.

There may be Dedicated S-NSSAI(s)/DNN for MWAB operation in the MWAB-UE subscription. The MWAB-UE may be configured with dedicated S-NSSAI(s)/DNNs for MWAB operation. When the MWAB-UE is in roaming, the Backhaul PLMN/SNPN may provide mapped S-NSSAI(s)/DNNs (used for MWAB operation) as defined in the clause 5.15.

When the MWAB-UE's authorization status changes from "authorized" to "unauthorized", if the AMF recognized the dedicated S-NSSAI/DNN used by the MWAB-UE, e.g. a dedicated S-NSSAI(s)/DNNs for Backhaul PDU session, the AMF triggers the Backhaul PDU session handling based on existing mechanism e.g. the AMF sends an UCU to MWAB-UE and may determine to delay the corresponding PDU session release based on an operator configuration (e.g. local configured timer).

#### 5.X.Y.3	MWAB-gNB service authorization

The OAM of MWAB-gNB owner PLMN/SNPN determines whether the MWAB-gNB can operated as MWAB

NOTE 1: 	The OAM can make the decision based on the time and location of MWAB.

When the MWAB-gNB is no longer authorized to operate by OAM, or all the Backhaul PDU sessions are released by the MWAB-UE, the MWAB-gNB should hand over the UE(s) it serves to other cells and performs shut-down steps.

NOTE 2: 	The interaction between MWAB-gNB and OAM is not in the scope of this specification.

NOTE 3: 	MWAB-gNB's OAM is assumed to be aware of the MWAB-UE authorization limitations (pre-determined authorizations) and may (pre-)configure/trigger MWAB-gNB to turn on/shut down accordingly

END OF CHANGES