3GPP TSG-WG SA2 Meeting #164	S2-2408271

Masstricht, NL, 19-23 August, 2024	(revision of S2-240xxxx)

Source:	Samsung

Title:	Final Conclusion for KI #3

Document for:	Approval

Agenda Item:	19.10

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract: This pCR add final conclusion for KI#3 in TR 23.700-59

# Introduction

This paper proposes conclusion for solutions of KI#3.

# Discussion

Based on the NWM responses and the LS response from RAN, this contribution proposes final conclusion by suggesting solutions for the pending scenarios which are following.

Scenario 1: Provisioning of NTZ information to UAV UE.

There are various options which have been documented in the interim conclusion on how the NTZ information is provided to UAV UE. As the NTZ information comes from USS to UAV need to remain same, it will be complete waste of resource to keep one NF like PCF in the path. IF there is some policy which need to be generated by PCF using the received NTZ information from USS as input then it makes sense to use PCF which is not the case here.

Proposal: It is proposed that at the entry point to the network AMF and MME can directly send the received NTZ information from USS to UAV

Scenario 2: Provisioning of NTZ information to 5GC.

There are various options which have been documented in the TR as part of different solutions but this is a simple transparent information which is only meant for AMF, MME and UAV because using these information both UAV and network (i.e AMF and MME) applies the NTZ enforcement. Hence it is not recommended to have NFs unnecessarily in the path and instead directly OAM can configure these NTZ information to AMF and MME directly.

Proposal: It is proposed that OAM configures the NTZ information in AMF and MME.

Scenario 3: NTZ enforcement by network.

In the interim conclusion it is already agreed that supporting UAV UE using the available NTZ information shall apply the NTZ enforcement. For the connected mode it was expected that source NG-RAN would have not selected target NG-RAN if the target falls inside the NTZ similar to the existing CAG cell principle during handover. But as per the LS response from RAN WG, no work will be done by them on this topic. Hence from the core network perspective if AMF finds that UAV UE has entered to NTZ area, AMF informs SMF to release the PDU and then deregister the UAV.

Also for non-supporting UAV UE even if the NTZ information is provided over application layer, it cant apply the NTZ enforcement as it does not have required capability. Otherwise there would not have any difference between supporting UAV and non-supporting UAV. Hence in this scenario the network need to enforce by rejecting the registration by informing no suitable cells in the TA so that until unless moves out of that TA, UAV UE wont try registering with the network again.

Proposal: It is proposed that AMF informs SMF to release the PDU session and then deregister the UAV if UAV UE enters inside NTZ in connected mode. Also proposed to reject the registration for non-supporting UAV UE if network the request from inside NTZ area.

# 3. Text Proposal

It is proposed to capture the following changes to TR 23.700-59.

# * * * * First change * * * *

## 8.3	 Conclusion for Key Issue #3: Support of No Transmit Zones

### 8.3.1	General

The following principles are agreed to support NTZ.

1)	NTZ enforcement applies to the UE that is a UAV UE (aka an aerial UE) with aerial subscription.

2)	NTZs are supported for both LTE and NR.

3)	An NTZ may map to one or more cells or a fraction of a cell, or overlap different cells in a mobile operator network.

NOTE:	Replanning of existing tracking areas and cells based on the presence of NTZs is not assumed.

4)	No transmissions in the restricted frequency bands are allowed from a UAV UE located in the NTZ (in three dimensions to consider height restrictions), regardless of the service type.

5)	No UAV UE transmits or attempts to transmit in the restricted frequencies, location and height defined by NTZ. But UAV UEs can transmit/receive normally outside of these restrictions.

6)	UAV UEs supporting NTZ regulations are configured with NTZ information, so that these UAV UEs will be aware of the presence of NTZs.

7)	Reception of downlink data from the network by the UAV UE located in the NTZ is allowed given the regulatory requirements are not violated.

### 8.3.2	Configuring/Provisioning NTZ assistance information to UAV UE and NTZ enforcement by UAV UE

The following NTZ assistance information  is configured/provisioned in the UAV UE.

-	A list of the restricted frequency band(s) with Geographical Area (including coordinates with longitude, latitude and height restrictions).

NOTE 1:	It is assumed UAV UE is configured with NTZ assistance information before accessing to the operator's network, if the local regulation requires NTZ.

The above NTZ assistance information is made available to the UAV UE by using one or more following ways:

-	pre-configured;

-	provided/updated by the AMF during the registration procedure;

-	provided/updated by the AMF during the UE Configuration Update procedure specified in clause 4.2.4.2 of TS 23.502 [4];

-	provided/updated by the MME during the attach procedure;

-	provided/updated by the MME during the TAU procedure;

The UAV UE is responsible for NTZ enforcement independently of the UAV UE 3GPP Release.

The UAV UE supporting NTZ (i.e. Rel-19 and onward UAV UEs) performs the following:

a)	enforces NTZ based on the configured/provisioned NTZ assistance information.

b)	upon applying NTZ enforcement, the UAV UE does not transmit any signalling or data.

c)	does not select/reselect any cells impacted entirely by an NTZ.

NOTE 3:	Whether indication about UAV UE's NTZ support to the AMF/MME is needed will be decided during normative phase. This work does not have RAN impacts.

NOTE 4:	For the network to know about UAV presence inside NTZ, whether registration updates by the UAV UE before entering the NTZ and when exiting the NTZ are needed or the existing mechanism (e.g. AMF subscribing with NG-RAN by providing NTZ as AoI) can be applied will be decided during normative phase. This work does not have RAN impacts.

NOTE 5:	Whether extension of Service Restrictions for NTZs using Restricted Transmission Area (RTA) mapped from the NTZ information is needed will be discussed during normative phase. This work does not have RAN impacts.

### 8.3.3	Configuring/Provisioning NTZ assistance information to 5GC and NTZ enforcement by 5GC

The following NTZ assistance information is configured/provisioned to 5GC.

-	A list of the restricted frequency band(s) with Geographical Area (including coordinates with longitude, latitude and height restrictions).

The above NTZ assistance information is made available to 5GC/EPC by using one or more following ways:

-	pre-configured in the AMF;

-	AMF received as part of OAM configuration;

-	pre-configured in the AMF;

-	AMF received as part of OAM configuration provided/updated by the AMF during the registration procedure.

The AMF/MME is responsible for NTZ enforcement in 5GC and EPC respectively for the UAV UE which does not support the feature (i.e. has not provided the indication for the support of NTZ in GMM Capability) by following below behaviour:

-	reject the registration by AMF with existing cause code #15 "no suitable cells in tracking area" as specified in TS 24.501 [10];

-	in connected mode if UAV enters NTZ area then AMF informs SMF to release the PDU session and deregister the UAV.

-	reject the attach request by MME with existing cause code #15 "no suitable cells in tracking area" as specified in TS 24.501 [10];

-	in connected mode if UAV enters NTZ area then MME informs SMF+PGW-C to release the PDU session and detach the UAV.

# * * * * End of changes * * * *