SA WG2 Meeting #164	S2-2408309

# 19 - 23 August, 2024, Maastricht, Netherland	(was S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1166 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF assisted signalling storm analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TR 23.700-84, the conclusion of KI#4 is as below: - A new Analytics ID to support Signalling storm Mitigation and Prevention caused by NFs signalling will be defined. - It will be determined in the normative phase whether to define a new Analytics ID or enhance an existing Analytics ID to support Signalling storm Mitigation and Prevention caused by massive signalling of UEs. Considering Signalling storm mitigation and prevention is not about a single UE or even a single NF, and the existing Analytics IDs, e.g. Abnormal behaviour which is more for UE abnormal behaviour analytics, are not appropriate to be used for signalling storm related analytics, it is proposed to define a new analytics ID for Signalling storm mitigation and prevention. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Define a new analytics ID for signalling storm analytics. 2. Define the filter information in the signalling storm analytics request. 3. Define the input and output parameters of the new analytics 4. Define the procedure of the new analytics. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NWDAF assisted signalling storm mitigation and prevention cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (all texts are new)

## 6.X	Signalling Storm Analytics

### 6.X.1	General

This clause defines how to detect or predict signalling storm, with the help of NWDAF. The NWDAF provides signalling storm analytics, in the form of statistics and prediction.

The service consumer may be an NF (e.g. AMF, SMF, UDM, PCF, NRF) or an AF.

The consumer of these analytics may indicate in the request or subscription:

-	Analytics ID = "Signalling Storm Analytics";

-	Target of Analytics Reporting: any UE;

-	Analytics Filter Information optionally containing:

-	NF ID List: indicates instance IDs or NF Set IDs of target NF that signalling storm analytics should be executed for.

-	Area of Interest (AOI): restricts the scope of signalling storm analytics to the specific area;

-	Analytics target period: indicates the time period when the signalling storm analytics should be provided;

-	Time slot: indicates the time slot during which the analytics is derived, e.g. 10 seconds, 5 minutes, etc;

-	Analytics Reporting Information optionally containing:

-	Reporting threshold and filter:

-	Signalling frequency threshold: indicates the frequency of the received signalling within the time slot;

-	Signalling frequency growth rate threshold: indicates the threshold for the growth rate of the signalling frequency. For example, the signalling frequency received by NF is 1000 signalling per second at time slot#1 and 10000 signalling per second at next point of time slot. The Signalling frequency growth rate is equal to 10. When the Signalling frequency growth rate is above the threshold, the NWDAF provides the analytics output;

-	Failure signalling request type: indicates the message type of the failed signalling that the NWDAF is requested to report analytics result for;

-	Failure signalling request number threshold: indicates the threshold of the number of signalling corresponding to each message type. When the number of failures corresponding to the failure message type reaches the threshold, the NWDAF will provide the analytics result, including the number of signalling.

-	Total failure signalling request number: indicates the threshold that identifies the number of signalling that cause such a failure. When the total number of failure signalling reaches this threshold, NWDAF will provide analytics result;

-	UE Request Type: indicates the type of a request initiated by the UE. For example, a request type can be an initial registration request, or a periodic registration request, or a mobility registration request;

-	Frequency of UE requests threshold: identifies a frequency at which the UE initiates a request of one type of message. For example, it identifies a frequency at which the UE initiates an initial registration request, i.e. the number of request messages of this message type initiated per minute or per second;

-	UE number threshold: indicates a threshold of the amount of UEs of which the number of one type of request messages meets the Frequency of UE requests threshold. For example, if the number of UEs that initiate registration requests exceeds the pre-set threshold (10 times per hour) and the number of such UEs exceeds 100 (the threshold is set to 100), the NWDAF will provide the analytics.

### 6.X.2 Input data

To detect the signalling storm, the NWDAF needs to collect the signalling information from the NFs to anaylze the expected normal status of signalling and identify a significant deviation in abnormal scenarios, to determine the cause of the signalling storm, e.g. caused by massive signalling of UEs and/or caused by NF signalling.

The NWDAF may support to collect the signalling information from the NFs and the UE group information from the AF for signalling storm analytics. The detailed information collected from NFs by the NWDAF is defined in Table 6.X.2-1, and the detailed information about the UE group information collected from AF is defined in Table 6.X.2-2.

Table 6.X.2-1: Input data from 5GC for Signalling Storm Analytics

| Information | Source | Description |
| --- | --- | --- |
| Timestamp |  | A timestamp associated with the collected information. |
| Back-off timer | AMF | Applied MM back-off timer to the UEs. |
| Expected UE Behaviour parameters | UDR | Same as Expected UE Behaviour parameters specified in TS 23.502 [3]. |
| Signalling request statistics | NFs | The request signalling received by the NFs, e.g. AMF/SMF. |
| > Type | NFs | The type of the request (e.g. initial/mobility/periodic/update/ emergency registration request or PDU session request, NF services). |
| > Frequency | NFs | The total number of each type request per time unit (e.g. per second). |
| > Number of Request from UE (1 … Max) | NFs | The total number of each type request from the UEs per time unit (e.g. per second). |
| > UE Number | NFs | For each number of requests, the total number of UEs who sent that amount of request. |
| > Request result | NFs | The result of the request (e.g. accepted, rejected) |
| NF profile information | NFs | NF profile information related to a particular UE/Session context in the NFs (such as AMF/SMF) |
| NF Instance ID | NRF | Identification of the NF instance that the NF profile information corresponds to. |
| > NF load | NRF, SCP | The load of a specific NF instance(s) in the NF profile(s) as defined in the TS 29.510 [13]. |
| > NF status | NRF, SCP | The status of a specific NF instance(s) (registered, suspended, undiscoverable) as defined in the TS 29.510 [13]. |
| > Priority | NRF, SCP | Priority (relative to other NFs of the same type) to be used for NF selection. |
| > Capacity | NRF, SCP | Static capacity information expressed as a weight relative to other NF instances of the same type. |
| SCP signalling statistics | SCP | The number of different types of signalling received and sent by SCP during a target time period, etc. |

Table 6.X.2-2: UE group information per application to be collected from the AF for Signalling Storm Analytics

| Information | Source | Description |
| --- | --- | --- |
| UE group information configured in AF | AF | UE group information collected from AF. |
| > List of UE ID |  | The list of UE IDs (e.g., GPSIs or SUPIs) |
| > UE number |  | The total number of UE. |
| > Destination |  | Expected final location of UE based on the route planned. |
| > Time of arrival |  | Expected time of arrival to destination based on the route planned. |
| > priority of the UE group |  | The priority information of the UE group to access to the network during the time period. |

### 6.X.3	Output Analytics

The Signalling Storm analytics are shown in Table 6.X.3-1 for statistics and Table 6.X.3-2 for predictions.

Table 6.X.3-1: Signalling Storm Statistics

| Information | Description |
| --- | --- |
| Analytics Area | List of TA(s) or Cell ID(s) within the requested area of interest. |
| Time slot list | List of time slots during the Analytics target period. |
| NF type | The type of the NF instance (e.g. AMF). |
| NF instance ID (NOTE) | Identification of the NF instance. |
| CPU usage | The average usage of the CPU. |
| Received Signalling Analytics | Information of signalling received by the NF. |
| > Total number of received signalling | Indicates the statistics on the number of signalling messages received by the NF in the time slot. |
| >> Signalling request type | Indicates the signalling request type (e.g. registration request/ service request/PDU session request). |
| >> Received number of signalling | Received number of signalling of the specific signalling type. |
| > Growth rate of received signalling | Difference between the number of signalling messages received by the NF in the time slot and the number of signalling messages received by the NF in the previous time slot. |
| >> Signalling request Type | Indicates the signalling request type (e.g. registration request/ service request/PDU session request). |
| >> Growth rate of the signalling | Difference between the number of this specific type of signalling messages received by the NF in the time slot and the number of this specific type of signalling messages received by the NF in the previous time slot. |
| > Signalling analytics from UE | Indicates the statistics on the number of signalling messages received from UEs within the time slot. |
| >> Signalling request Type | Indicates the signalling request type (e.g. registration request/ service request/PDU session request). |
| >> number of signalling requests from the same UE (1…max) | Number of received signalling requests initiated by the same UE within the time slot. |
| >>> UE number | Number of UEs whose signalling requests sent by the same UE within the time slot. |
| Sending Signalling Analytics | Information of signalling sent by the NF. |
| > Total number of sending signalling | Indicates the statistics on the number of signalling messages sent by NFs in the time slot. |
| > Total number of successful sending signalling | Indicates the statistics on number of accepted signalling messages sent by the NF in the time slot. |
| >> Signalling Request type | The request type of successful signalling messages. |
| >> Number of the signalling | Number of successful signalling messages of each type. |
| > Total number of failure sending signalling | Indicates the statistics on the number of failure signalling messages sent by NFs in the time slot. |
| >> Failure request type | The request type of failure signalling messages. |
| >> Failure request number | The total number of failure request for each type. |
| Signalling Storm Probability | Probability of signalling storm. |
| Signalling Storm cause | The cause of the signalling storm. |
| NOTE: The NF is part of the NF instance list indicated in the Analytics Filter Information. |  |

Table 6.X.3-2: Signalling Storm Prediction

| Information | Description |
| --- | --- |
| Analytics Area | List of TA(s) or Cell ID(s) within the requested area of interest. |
| Time slot list | List of time slots during the Predictions target period. |
| NF type | The type of the NF instance (e.g. AMF). |
| NF instance ID (NOTE) | Identification of the NF instance. |
| CPU usage | The average usage of the CPU. |
| Received Signalling Analytics | Information of signalling received by NF. |
| > Total number of received signalling | Indicates the prediction on the number of signalling messages received by NFs in the time slot. |
| >> Signalling request type distribution | Indicates the signalling request type (e.g. registration request/ service request/PDU session request). |
| >> Received number of signalling | Received number of signalling of the specific signalling type. |
| > Growth rate of received signalling | Difference between the number of signalling messages received by the NF in the time slot and the number of signalling messages received by the NF in the previous time slot. |
| >> Signalling request Type | Indicates the signalling request type (e.g. registration request/ service request/PDU session request). |
| >> Growth rate of the signalling | Difference between the number of this specific type of signalling messages received by the NF in the time slot and the number of this specific type of signalling messages received by the NF in the previous time slot. |
| > Signalling analytics from UE | Indicates the prediction on the number of signalling messages received from UEs within the time slot. |
| >> Signalling request Type | Registration request/ service request/PDU session request. |
| >> number of signalling requests from the same UE (1…max) | Number of received signalling requests initiated by the same UE within the time slot. |
| >>> UE number | Number of UEs whose signalling requests sent by the same UE within the time slot. |
| Sending Signalling Analytics | Information of signalling sent by the NF. |
| > Total number of sending signalling | Indicates the prediction on the number of signalling messages sent by the NF in the time slot. |
| > Total number of successful sending signalling | Indicates the prediction on the number of accepted signalling messages sent by the NF in the time slot. |
| >> Signalling Request type | The request type of successful signalling messages. |
| >> Number of the signalling | Number of successful signalling messages of each type. |
| > Total number of failure sending signalling | Indicates the prediction on the number of failure signalling messages sent by NFs. |
| >> Failure request type | The request type of failure signalling messages. |
| >> Failure request number | The total number of failure request for each type. |
| Signalling Storm Probability | Prediction about the probability of signalling storm. |
| Signalling Storm cause | The cause of the predicted signalling storm. |
| Predicted Back-off timer range to the AMF | Indicates the Back-off timer range that may be selected by the AMF to resolve congestion. |
| Predicted candidate NF profile list to target NF | Indicates the candidate NFs with NF ID, type and corresponding priority that can be selected by the target NF to resolve congestion. |
| > Priority | Priority (relative to other NFs of the same type) of candidate NFs to be used for NF selection as defined in the TS 29.510 [18]. |
| > Capacity | Candidate NF capacity information, expressed as a weight relative to other NF instances of the same type as defined in the TS 29.510 [18]. |
| Confidence | Confidence of the prediction. |
| NOTE: The NF instance is part of the NF instance list indicated in the Analytics Filter Information. |  |

### 6.X.4	Procedures

Figure 6.X.4-1 depicts the procedure for an NF consumer to request analytics to the NWDAF for signalling storm detection.

Figure 6.X.4-1: Procedure for Signalling Storm Analytics

1.	The NF Consumer and/or the SCP sends an Analytics request (Analytics ID = Signalling Storm Analytics, Analytics Filter Information, Target of Analytics Reporting) to the NWDAF containing AnLF by invoking either Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe.

2.	The NWDAF may subscribe to the information as defined in Table 6.X.2-1 from the AMF.

3.	The NWDAF may subscribe to the data as defined in Table 6.X.2-1 from the NRF.

4.	The NWDAF may collect the data as defined in Table 6.X.2-1 from other 5GC NFs/SCP.

5.	For UE group information collection, if the request is authorized, the NWDAF may collect the data from  the AF as defined in Table 6.X.2-2.

6.	NWDAF derives the requested analytics as defined in Table 6.X.3-1 and 6.X.3-2.

7.	NWDAF sends the notification or response to the consumer using Nnwdaf_AnalyticsInfo_Request Response or Nnwdaf_AnalyticsSubscription_Subscribe_Notify, depending on the service request received in step 1.

8.	The consumer may determine whether the signalling storm occurs and take actions based on signalling storm analytics. If the consumer is the AMF, the consumer may adjust the proportion of 5G-AN nodes to be sent with NGAP OVERLOAD START message, and adjust the content of the overload start message, or adjust the back-off timer for the UE registration request. If the consumer is the PCF, the consumer may adjust the URSP policy of the UE or adjust the BDT policy stored in UDR or send BDT policy warning notification to the AF. If the consumer is NRF, the consumer may adjust the priority of the NF selection. If the consumer is the AF, the consumer may adjust the service request to the network (such as adjusting the frequency of sending data).

END OF CHANGES