3GPP TSG-WG SA2 Meeting #164	S2-2408318

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1169 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VFL Inference Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The new functionalinity of Vertical Federated Learning has been concluded in TR 23.700-84. The general principles of the VFL support have been agreed in Clause 8.2 TR 23.700-84. The agreed principles related to VFL Inference support need to be reflected in TS 23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of two new complete clauses in TS 23.288 to reflect the conclusions of VFL Inference with NWDAF acting as VFL Server and VFL Inference with AF acting as VFL server. It is proposed to organize the VFL Inference procedures according to which NF receives the request or subscription from the Analytics Consumer (e.g., PCF). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Concepts agreed in the Study Item phase for R19 FS_AIML_CN are not reflected at the NWDAF architecture and description in the TS 23.288. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X.2.y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Clause 6.2X.2.y(new) should be added under TS 23.288 CR1167, clause 6.2X.2. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2X.2.y	Vertical Federated Learning Inference

6.2X.2.y.1	General

NWDAFs are configure with the capability to identify whether a subscription for an analtyics ID requires the use of VFL Inference, which means that depending on certain conditions (as indicated in the procedures in Clause 6.2X.2.y.2), more than one NWDAF are involved in the analtyics generation.

An Analytics Consumer discovers NWDAF via NRF and subscribe to one of the candidate NWDAF unaware of any requirement for executing VFL Inference. Therefore, there are four possible procedures for the generation of analytics based on VFL Inference:

-	Case 1: The Analytic Consumer subscribes to a NWDAF and such NWDAF is determined to act as VFL Server for the VFL Inference process required for the generation of the required analtyics. In this case the procedure descripbed in Clause 6.2X.2.y.2.1.1 applies.

-	Case 2: The Analytic Consumer subscribes to a NWDAF and such NWDAF is determined to act as VFL Client for the VFL Inference process required for the generation of the required analtyics and for such VFL Inference process a NWDAF is acting as VFL Server.  In this case, the procedure defined in Clause 6.2X.2.y.2.1.2 applies.

-	Case 3: The Analytic Consumer subscribes to a NWDAF and such NWDAF has no VFL Capability itself. Such NWDAF is capable to determine that for the requested analtyics a VFL Inference process has to be used and discovers the approapriate NF to handle the subscription. In this case, when the new discovered NF is a NWDAF acting as VFL Server the procedure in Clause 6.2X.2.y.2.1.1 applies. When the new discovered NF is a AF acting as VFL Server the procedure in Clause 6.2X.2.y.2.2.1 applies.

-	Case 4: The Analytic Consumer subscribes to a NWDAF and such NWDAF is determined to act as VFL Client for the VFL Inference process required for the generation of the required analtyics and for such VFL Inference process a AF is acting as VFL Server.  In this case, the procedure defined in Clause 6.2X.2.y.2.2.2 applies.

6.2X.2.y.2	Procedures

6.2X.2.y.2.1	Procedures for Vertical Federated Learning Inference with NWDAF acting as VFL Server

6.2X.2.y.2.1.1	VFL Inference with Subscription at NWDAF Acting as VFL Server

NWDAF acting as VFL Server

Analytics

Consumer

NEF

2. Determine VFL Inference execution

NWDAF acting as VFL Client 1

…

NWDAF acting as VFL Client N

1.

Nnwdaf_AnalyticsSubscription_Subscribe

3. Generate

VFL inference output

4. Monitor accuracy of VFL inference

6. Determine change in VFL Inference

7

a

.

Nnwdaf_AnalyticsSubscription_Subscribe

(VFL Inference

Execution

request)

5.

Nnwdaf_AnalyticsSubscription_Notify

7c.

Nnef_AnalyticsExposure_Subscribe

(VFL Inference

Execution

request)

7d.

Naf_AnalyticsSubscription_Subscribe

(VFL Inference

Execution

request)

7b.

Naf_AnalyticsSubscription_Subscribe

(VFL Inference

Execution

request)

8b

.

Generate

local inference intermediary output

8c

.

Generate

local VFL inference intermediary output

9a.

Nnwdaf_AnalyticsSubscription_Notify

(VFL Inference intermediary results)

9d.

Nnef_AnalyticsExposure_Notify

(VFL Inference intermediary results)

9c.

Naf_AnalyticsSubscription_Notify

(VFL

Inference intermediary results)

9b.

Naf_AnalyticsSubscription_Notify

(VFL Inference intermediary results)

AF acting as VFL Client 1

…

AF acting as VFL Client N

10.

Generate VFL inference output

8a

.

Generate

local inference intermediary output

11.

Nnwdaf_AnalyticsSubscription_Notify

Figure 6.2X.2.y.2.1.1-1: VFL Inference with Subscription at NWDAF Acting as VFL Server

1.	An Analtyics Consumer invokes the Nnwdaf_AnalyticsSubscription _Subscribe service from an NWDAF.

2.	The NWDAF receiving the request from the Analtyics Consumer analyzes the information of the subscription and local VFL related configuration to verify whether VFL Inference has to be executed for generating the requested analytics. Additionally, such NWDAF also determines it is acting as VFL Server for such VFL inference process by configuration.

3.	When the NWDAF acting as VFL Server determines VFL Inference applies for the generation of analytics ID, the NWDAF acting as VFL Server starts the VFL Inference process based only on the execution of its local ML Model related to the VFL Correlation ID jointly trained with further ML Models by other entities that applies for the requested analtyics generation. The NWDAF acting as VFL Server generates the VFL inference output based only on its local jointly trained ML model.

4.	The NWDAF acting as VFL Server starts the VFL accuracy monitoring for the analytics ID based only on the inference output from the ML model being used by the NWDAF acting as VFL Server and determines the VFL accuracy information for the NWDAF acting as VFL Server.

5.	While the determined VFL accuracy information is equal or higher than required, the NWDAF acting as VFL Server provides the analytics output, based its generated VFL inference output, to the Analytics Consumer using the Nnwdaf_AnalyticsSubscription _Notify service.

6.	When the NWDAF acting as VFL Server determines that VFL accuracy information related only to the inference generated by the NWDAF acting as VFL Server is lower than required, the NWDAF acting as VFL Server decides to change the VFL Inference process and include the VFL Clients associated VFL Correlation ID. The NWDAF acting as VFL Server determines the appropriated NWDAFs or AFs acting VFL Clients. The NWDAF acting as VFL Server also determines the change in the VFL accuracy monitoring so that the input of all the related entities is also considered in the calculated VFL accuracy information.

7.	The NWDAF acting as VFL Server provides to the determined VFL Clients the request to join the VFL Inference process.

In case of NWDAFs acting as VFL Clients (Step 7a), the NWDAF acting as VFL Server provides via the Nnwdaf_AnalyticsSubscription_Subscribe request service including the VFL Inference Execution request which includes: Joint VFL Inference flag, VFL Correlation ID, sample alignment information, inference time information.

In case of trusted AFs acting as VFL Clients (Step 7b), the NWDAF acting as VFL Server provides via Naf_AnalyticsSubscription _Subscribe request service including the VFL Inference Execution request which includes: Joint VFL Inference flag, VFL Correlation ID, sample alignment information, inference time information.

In case of 3rd Party AFs acting as VFL Clients, the NWDAF acting as VFL Server provides to the NEF via Nnef_AnalyticsExposure_Subscribe request service the VFL Inference Execution request which includes: Joint VFL Inference flag, VFL Correlation ID, sample alignment information, and the AF information allowing the NEF to interact with the proper AFs related to the same VFL Correlation ID.

8.	The NWDAF acting as a VFL server and the NWDAFs or AFs acting as VFL Clients that accepted to join the VFL Inference process generate each their local intermediary inference output based on their  local ML Models jointly trained under the same VFL Correlation ID. The VFL Clients collects inference data based on the inference time information.

9.	The NWDAFs or AFs acting as VFL Clients provide to the NWDAF acting as  VFL Server the calculated intermediary inference results.

10. The NWDAF acting as VFL Server aggregates its own local calculated intermediary inference output with all information received in Step 9 to generate the final analytics output.

11.	The NWDAF acting as VFL Server provides the final analytics output based on the joint VFL inference to the Analtyics Consumer.

6.2X.2.y.2.1.2	VFL Inference with Subscription at NWDAF Acting as VFL Client

2. Determine VFL Inference execution

1.

Nnwdaf_AnalyticsSubscription_Subscribe

3. Determine VFL Server

6.

Nnwdaf_AnalyticsSubscription_Notify

8.

Nnwdaf_VFLInference_AccuracyMonitor_Notify

9.

Nnwdaf_AnalyticsSubscription_Transfer

Request

5. Generate

VFL inference output

4.

Trigger accuracy monitoring for VFL Client inference

NWDAF acting as VFL Client

Analytics

Consumer

NWDAF acting as VFL Server

7.

Determine change in VFL Inference

10.

VFL Inference (Steps 7

–

11) in Procedure for

NWDAF as

VFL Server based VFL Inference

Figure 6.2X.2.y.2.1.2-1: VFL Inference with Subscription at NWDAF Acting as VFL Server

1.	An Analtyics Consumer invokes the Nnwdaf_AnalyticsSubscription _Subscribe service from an NWDAF.

2.	The NWDAF receiving the request from the Analtyics Consumer analyzes the information of the subscription and local VFL related configuration to verify whether VFL Inference has to be executed for generating the requested analytics and is associated with a VFL correlation ID. Additionally, such NWDAF also determines it is acting as VFL Client for such VFL inference process by configuration.

3.	The NWDAF acting as VFL Client determines the NWDAF acting as VFL Server associated with the VFL correlation ID.

4.	When the NWDAF acting as VFL Client is configured with the capability to perform VFL Inference only based on its local ML model jointly trained under the VFL correlation ID, the NWDAF acting as VFL Client interact with the NWDAF acting as VFL Server in order to trigger the VFL accuracy monitoring of the inference output generated by the NWDAF acting as VFL Client.

5.	The NWDAF acting as VFL Client starts the VFL Inference process based only on the execution of its local ML Model related to the VFL Correlation ID jointly trained with further ML Models by other entities that applies for the requested analtyics generation. The NWDAF acting as VFL Client generates the VFL inference output based only on its local jointly trained ML model

6.	While the NWDAF acting as VFL Client does not receive any indication from the NWDAF acting as VFL Server that is monitoring the VFL inference process executed only by the NWDAF acting as VFL Client, the NWDAF acting as VFL Client provides the analytics output, based its generated VFL inference output, to the Analytics Consumer using the Nnwdaf_AnalyticsSubscription _Notify service.

7. 	The NWDAF acting as VFL Server determines that VFL accuracy information related only to the inference generated by the NWDAF acting as VFL Client is lower than the required.

8.	Based on the determination in Step 7, the NWDAF acting as VFL Server provides to the NWDAF acting as VFL Client the VFL Client Accuracy Information, which comprises an instruction to stop the VFL Inference based only on the NWDAF acting as VFL Client and change to the joint VFL Inference controlled by the NWDAF acting as VFL Server.

9.	The NWDAF acting as VFL client, based on the received instruction in Step 8, triggers the transfer of the analytics requested by the Analtyics Consumer to the NWDAF acting as VFL server. The NWDAF acting as VFL Client includes in the transfer request the VFL Inference Related Information comprising the Change to Joint VFL Inference flag.

10.	Based on the information in the transfer request from the NWDAF acting as VFL Client, the NWDAF acting as VFL Server executes the Steps 7 to 11 from Figure 6.2X.2.y.2.1.1-1, therefore performing the joint VFL inference with the further determined VFL Clients for the VFL correlation ID.

6.2X.2.y.2.2	Procedures for Vertical Federated Learning Inference with AF acting as VFL Server

6.2X.2.y.2.2.1	VFL Inference with Subscription at NWDAF without VFL Capability

NWDAF

Analytics Consumer

(

e.g

PCF)

NEF

2. Determine VFL Inference execution

NWDAF acting as VFL Client 1

…

NWDAF acting as VFL Client N

1.

Nnwdaf_AnalyticsSubscription_Subscribe

3. Determine VFL Server

12a

.

Generate

local inference intermediary output

12b

.

Generate

local VFL inference intermediary output

AF acting as VFL Server

4b.

Nnef_AnalyticsExposure_Subscribe

(VFL Inference Execution request)

4a.

Naf_AnalyticsSubscription_Subscribe

(VFL Inference Execution request)

4c.

Naf_AnalyticsSubscription_Subscribe

(VFL

Inference Execution request)

5

.

Determine VFL Inference execution

11a.

Nnwdaf_AnalyticsSubscription_Subscribe

(VFL Inference Execution request)

11b.

Nnef_AnalyticsExposure_Subscribe

(VFL Inference

Execution request)

11c.

Nnwdaf_AnalyticsSubscription_Subscribe

13a.

Nnwdaf_AnalyticsSubscription_Notify

13c.

Nnef_AnalyticsExposure_Notify

()

13b.

Nnwdaf_AnalyticsSubscription_Notify

14.

Generate VFL inference output

16.

Nnwdaf_AnalyticsSubscription_Notify

()

15c.

Nnef_AnalyticsExposure_Notify

()

15a.

Naf_AnalyticsSubscription_Notify

()

15b.

Naf_AnalyticsSubscription_Notify

()

6. Generate

VFL inference output

7.

Monitor accuracy of VFL inference

9.

Nnwdaf_AnalyticsSubscription_Notify

()

8b

.

Nnef_AnalyticsExposure_Notify

()

8a.

Naf_AnalyticsSubscription_Notify

()

8b.

Naf_AnalyticsSubscription_Notify

()

10.

Determine change in VFL Inference

Figure 6.2X.2.y.2.2.1-1: VFL Inference with Subscription at NWDAF without VFL Capability and AF acting as VFL Server

1.	An Analtyics Consumer invokes the Nnwdaf_AnalyticsSubscription _Subscribe service from an NWDAF.

2.	The NWDAF receiving the request from the Analtyics Consumer analyzes the information of the subscription and local VFL related configuration to verify whether VFL Inference has to be executed for generating the requested analytics.

3.	When the NWDAF determines it does not have a VFL Server nor a VFL Client capability for generating the requested analtyics, the NWDAF performs the discovery of an NWDAF or AF with the proper support.

4. 	The NWDAF determines an AF acting as VFL Server is the appropriated for the VFL Inference process, the NWDAF provides to the AF acting as VFL Server an analtyics subscriptions directly via  Naf_AnalyticsSubscription _Subscribe request service in case of trausted AFs (Step 4a); and via NEF in case of untrusted AFs (Step 4b and 4c). The input parameters of the invoked services include the VFL Inference Execution request comprising: VFL Inference flag, VFL Correlation ID, sample alignment information, inference time information.

5. 	The AF acting as VFL Server receives the subscription and identifies VFL Inference applies for the generation of analytics ID.

6.	The AF acting as VFL Server starts the VFL Inference process based only on the execution of its local ML Model related to the VFL Correlation ID jointly trained with further ML Models by other entities that applies for the requested analtyics generation. The AF acting as VFL Server generates the VFL inference output based only on its local jointly trained ML model.

7.	The AF acting as VFL Server starts the VFL accuracy monitoring for the analytics ID based only on the inference output from the ML model being used by the AF acting as VFL Server and determines the VFL accuracy information for the AF acting as VFL Server.

8.	While the determined VFL accuracy information is equal or higher than required, the AF acting as VFL Server provides the analytics output, based on its generated VFL inference output, to the NWDAF that requested the output, directly (Step 8a) or via NEF (Step 8b and 8c).

9.	The NWDAF that received the original request from the Analytics Consumer forwards the received anatyics output from AF acting as VFL Server to the Analytics Consumer.

10.	When the AF acting as VFL Server determines that VFL accuracy information related only to the inference generated by the AF acting as VFL Server is lower than required, the AF acting as VFL Server decides to change the VFL Inference process and include the VFL Clients associated VFL Correlation ID. The AF acting as VFL Server determines the appropriated NWDAFs acting as VFL Clients. The AF acting as VFL Server also determines the change in the VFL accuracy monitoring so that the input of all the related entities is also considered in the calculated VFL accuracy information.

11.	The AF acting as VFL Server provides to the determined VFL Clients the request to join the VFL Inference process.

In case of trusted AFs acting as VFL Server (Step 11a), the AF acting as VFL Server provides via Nnwdaf_AnalyticsSubscription _Subscribe request service including the VFL Inference Execution request which includes: Joint VFL Inference flag, VFL Correlation ID, sample alignment information, inference time information.

In case of 3rd Party AFs acting as VFL Server, the AF acting as VFL Server provides to the NEF via Nnef_AnalyticsExposure_Subscribe request service the VFL Inference Execution request which includes: Joint VFL Inference flag, VFL Correlation ID, sample alignment information. The NEF is capable to determine the NWDAFs associated with the VFL Correlation ID and invoke the analtyics subscription services including the received VFL Inference Execution request from AF acting as VFL Server.

12.	The AF acting as a VFL server and the NWDAFs acting as VFL Clients that accepted to join the VFL Inference process generate each their local intermediary inference output based on their  local ML Models jointly trained under the same VFL Correlation ID. The VFL Clients collects inference data based on the inference time information.

13.	The NWDAFs acting as VFL Clients provide to the AF acting as VFL Server the calculated intermediary inference results.

14. The AF acting as VFL Server aggregates its own local calculated intermediary inference output with all information received in Step 13 to generate the final analytics output.

15.	The AF acting as VFL Server provides the final analytics output based on the joint VFL inference to the NWDAF that requested the output, directly (Step 15a) or via NEF (Step 15b and 15c).

16.	The NWDAF that received the original request from the Analytics Consumer forwards the received anatyics output from the AF acting as VFL Server to the Analytics Consumer.

6.2X.2.y.2.2.2	VFL Inference with Subscription at NWDAF Acting as VFL Client

1.

Nnwdaf_AnalyticsSubscription_Subscribe

9b.

Nnef_VFLInference_Transfer

Request

9c.

Naf

_

VFLInference_Transfer

Request

9a.

Naf

_

VFLInference_Transfer

Request

10.

VFL Inference (Steps

11

-

16)

in Procedure for

AF VFL Server based VFL Inference

2. Determine VFL Inference execution

3. Determine VFL Server

5. Generate

VFL inference output

4.

Trigger accuracy monitoring for VFL Client inference

6.

Nnwdaf_AnalyticsSubscription_Notify

7.

Determine change in VFL Inference

8a.

Nnwdaf_VFLInference_AccuracyMonitor_Notify

8b.

Nnef_VFLInference_AccuracyMonitor_Notify

8c.

Nnwdaf_VFLInference_AccuracyMonitor_Notify

NWDAF acting as VFL Client

Analytics

Consumer

NEF

AF acting as VFL Server

Figure 6.2X.2.y.2.2.2-1: VFL Inference with Subscription at NWDAF Acting as VFL Client and AF acting as VFL Server

1.	An Analtyics Consumer invokes the Nnwdaf_AnalyticsSubscription _Subscribe service from an NWDAF.

2.	The NWDAF receiving the request from the Analtyics Consumer analyzes the information of the subscription and local VFL related configuration to verify whether VFL Inference has to be executed for generating the requested analytics and is associated with a VFL correlation ID. Additionally, such NWDAF also determines it is acting as VFL Client for such VFL inference process by configuration.

3.	The NWDAF acting as VFL Client determines the AF acting as VFL Server associated with the VFL correlation ID.

4.	When the NWDAF acting as VFL Client is configured with the capability to perform VFL Inference only based on its local ML model jointly trained under the VFL correlation ID, the NWDAF acting as VFL Client interact with the AF acting as VFL Server in order to trigger the VFL accuracy monitoring of the inference output generated by the NWDAF acting as VFL Client.

5.	The NWDAF acting as VFL Client starts the VFL Inference process based only on the execution of its local ML Model related to the VFL Correlation ID jointly trained with further ML Models by other entities that applies for the requested analtyics generation. The NWDAF acting as VFL Client generates the VFL inference output based only on its local jointly trained ML model

6.	While the NWDAF acting as VFL Client does not receive any indication from the AF acting as VFL Server that is monitoring the VFL inference process executed only by the NWDAF acting as VFL Client, the NWDAF acting as VFL Client provides the analytics output, based its generated VFL inference output, to the Analytics Consumer using the Nnwdaf_AnalyticsSubscription _Notify service.

7. 	The AF acting as VFL Server determines that VFL accuracy information related only to the inference generated by the NWDAF acting as VFL Client is lower than the required.

8.	Based on the determination in Step 7, the AF acting as VFL Server provides to the NWDAF acting as VFL Client the VFL Client Accuracy Information, which comprises an instruction to stop the VFL Inference based only on the NWDAF acting as VFL Client and change to the joint VFL Inference controlled by the NWDAF acting as VFL Server. The VFL Client Accuracy Information can be provided directly to the NWDAF acting as VFL Client (step 8a) or via NEF (Step 8b and 8c).

9.	The NWDAF acting as VFL client, based on the received instruction in Step 8, triggers the transfer of the control for the generation of the final output related to the analytics requested by the Analtyics Consumer to the AF acting as VFL server. The NWDAF acting as VFL Client includes in the transfer request the VFL Inference Related Information comprising the Change to Joint VFL Inference flag.

10.	Based on the information in the transfer request from the NWDAF acting as VFL Client, the AF acting as VFL Server executes the Steps 11 to 16 from Figure 6.2X.2.y.2.2.1-1, therefore performing the joint VFL inference with the further determined VFL Clients for the VFL correlation ID.

# * * * * End of changes * * * *