3GPP SA WG2 Meeting #S2-164	S2-2408322

Maastricht, NL, August 19 – 23, 2024

Source:	MediaTek Inc.

Title:	Update the conclusion of 8.3

Document for:	Agreement

Agenda Item:	19.4.1

Work Item / Release:	FS_EnergySys/Rel-19

Abstract of the contribution: This paper proposes to update the conclusion of 8.3

# Proposal

It proposes to include the below descriptions to TR 23.700-66

# Discussion

Based on the conclusion in 8.2:

-	The following information is stored as part of the subscription data in the UDM/UDR:

-	Energy saving subscription information per UE to assist the network to perform energy saving strategies for the UE.

-	The PCF may receive UE subscription data and notification related to the energy related information to trigger making policy decisions (reusing the existing parameters) based on operator policy.

The existing operations and procedures related to the QoS provisioning information to NG-RAN and UE can be enhanced to enforce the energy related QoS information (reusing the existing parameters) provisioning to NG-RAN and UE via SMF. SMF may also notify PCF to trigger PCF to make new PCC rules which will cause new value of QoS information to NG-RAN and UE.

*** Start of changes ***

## 8.3	Conclusion for Key Issue #3: 5GS enhancements for network energy saving and efficiency

It is proposed to procced to normative work based on the following principles:

1)	For Enhancements on NF discovery and (re-)selection based on energy related information:

-	NF profile may be extended (e.g. include the new energy related information or reuse existing NF profile parameters) to allow an operator to influence NF discovery and selection based on their energy strategy.

NOTE 1:	Which energy related information is included in the NF profile is to be further discussed in the normative phase. Only parameters captured in existing solutions will be considered. Coordinated with SA WG5 may be needed.

NOTE 2:	The information within NF profile is expected not to change frequently, e.g. constantly reflecting changes in energy consumption. The procedure for updating the information will be addressed in normative phase.

-	NF discovery and (re-)selection is enhanced to consider the energy related information from the NF profiles and/or discovery request from the NF consumer.

2)	For enhancements on existing operations and procedures for energy saving and energy efficiency:

-	UP path of PDU session may be adjusted.

-	QoS information provisioning to UE and NG-RAN may be adjusted

NOTE 3:	The energy related decision will also consider operator’s policy.