3GPP TSG-WG SA2 Meeting #164	S2-2408327

19 August - 23 August 2024, Maastricht, Netherlands	(revision of S2-240xxxx)

Source:	CATT

Title:	Initial conclusions for DualSteer KI#1.4

Document for:	Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract: Initial conclusions for DualSteer KI#1.4.

- Introduction

This paper provides an initial conclusion for KI#1.4.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-54 1.0.0.

# * * * * First change * * * *

8.1	Conclusions for DualSteer

8.1.4	Conclusion for KI#1.4: Policy enhancements for DualSteer

The following principles are proposed to be considered in normative work:

-	The PCF in HPLMN will provide policies to instruct the DualSteer Device when and how to trigger the second SUPI to perform the registration and when and how to establish connectivity to the second 3GPP access network.

NOTE:	The policy related to PLMN selection may depend on SA1/CT1 discussion.

-	The UE policy shall be enhanced to support the DualSteer service (e.g., to indicate the Access Type/RAT/PLMN selection for the second 3GPP access network and control the secondary PDU Session establishment).

NOTE:	The policy related to PLMN selection may depend on SA1/CT1 discussion.

-	The Access Type preference in Route Selection Descriptor shall be extended to include a Dual Steering indication or Multiple Accesses.

-	The PCC rules shall be extended to include the DualSteer policy which will control how to switch the service between two connected 3GPP access networks for DualSteer Device.

-	The DualSteer rules can be based on the principles of ATSSS rules with necessary extensions and steering mode limitations in place.

# * * * * End of changes * * * *