3GPP TSG-WG SA2 Meeting #164	S2-2408337

# Maastricht, Netherlands, Aug 19 – 23, 2024		(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4954 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UPF event exposure during UPF relocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, SK Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As concluded in clause 8.2 of TR 23.700-63: - NEF/NWDAF can use UE IP address to discover the PSA UPF and subscribe directly to PSA UPF for data collection. - For periodic reporting type of the UPF event exposure service with direct notifications when the UPF changes, the source UPF may report to the consumer any data collected between the time instance of the last notification to the consumer and the time instance of the UPF change. - When the PDU session is released, if the UPF event subscription is via the SMF, the SMF may send a notification of subscription termination. Based on the above conclusions for KI#2, the UPF event exposure can be enhanced as follows: 1. UPF event consumer discovers PSA UPF by using UE IP address and subscribes directly to the PSA UPF: The NWDAF may need to collect User Data Usage Measures (e.g., number of packet transmission/ retransmission) for Service Experience analytics based on analytics subscription/request from AF/NEF, and the Target of Analytics Reporting may contain a single UE, as described in clause 6.4 of TS 23.288. Therefore, NF consumer (e.g. NWDAF) may subscribe to the UPF Event Exposure service directly for data collected for a single UE, if the subscription is for User Data Usage Measures or User Data Usage Trends and the NF consumer can obtain the IP address of the UE, the NF consumer can use UE IP address to discover the PSA UPF and subscribe directly to PSA UPF for the data collection. 2. Source UPF sends the collected data to the consumer when UPF changes: The source UPF may have collected some data but not sent to the consumer yet when the UPF changes, and the consumer can decide how to handle this data, e.g., discard the collected data or send the collected data to the consumer. For example, when the consumer subscribes to the UPF event via SMF, the consumer can include a relocation reporting indication, which indicates how to handle the data collected by the source UPF when UPF relocation occurs, and when the UPF changes, the SMF can include in the unsubscription request a last notification indication which indicates how the collected data should be handled when receiving the last notification indication. The source UPF sends the collected data to the UPF event consumer if the last notification indication from SMF indicates that the source UPF should send the collected data to the UPF event consumer. 3. Send a notification with a subscription termination indication to the UPF event consumer when UPF changes: The AMF can inform the consumer NF that the subscription is terminated at the AMF, and the AMF can also indicate the reason for the subscription termination, as specified in TS 29.518. For the UPF event exposure, the subscription to UPF event may also be terminated, e.g., due to UPF relocation. However, currently, the SMF/UPF cannot indicate the subscription termination to the consumer. Therefore, the SMF/UPF event notification can be enhanced to include a subscription termination indication to indicate that the subscription to the UPF event has been terminated, and may also include a cause code indicating the reason for subscription termination, e.g., UPF relocation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new procedure of subscription to UPF event exposure service via SMF during UPF relocation. Add a new procedure of subscription directly to UPF event exposure service during UPF relocation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The enhancements on UPF event exposure during UPF relocation are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.X (new), 4.15.4.5.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.15.4.5.X	Information flow for subscription to UPF event exposure service via SMF during UPF relocation

Figure 4.15.4.5.X-1: Subscription to UPF event exposure service via SMF during UPF relocation

1.	The UPF event consumer (e.g. NWDAF) subscribes to the UPF data via SMF by invoking Nsmf_EventExposure_Subscribe service operation, and the Target of Event Reporting indicates a specific UE. The consumer may include a relocation reporting indication, which indicates how to handle the data collected by the source UPF when UPF relocation occurs, e.g., discard the collected data or send the collected data to the consumer.

2.	The SMF selects the source UPF and subscribes to the source UPF by invoking Nupf_EventExposure_Subscribe service operation.

3.	The source UPF sends the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

4.	The SMF determines that the UPF needs to be relocated.

5.	The SMF unsubscribes to the source UPF by invoking Nupf_EventExposure_Unsubscribe service operation. The SMF may include a last notification indication based on relocation reporting indication from the UPF event consumer, which indicates how the collected data should be handled when receiving the last notification indication, e.g., discard the collected data or send the collected data to the consumer.

6.	The source UPF sends the collected data to the UPF event consumer if the last notification indication from SMF indicates that the source UPF should send the collected data to the UPF event consumer. For example, 	for periodic reporting type of the UPF event exposure service when the UPF changes, the source UPF may report to the consumer any data collected between the time instance of the last notification to the consumer and the time instance of the UPF change.

7a-8a.	(Option A) The SMF may send the notification to the UPF event consumer by invoking Nsmf_EventExposure_Notify service operation, which includes a subscription termination indication to indicate that the subscription to the UPF event has been terminated, and may also include a cause code indicating the reason for subscription termination, e.g., UPF relocation. When receiving the notification of subscription termination, the UPF event consumer may determine to re-subscribe to the UPF event exposure.

7b-8b.	(Option B) To continue the subscription of the UPF event consumer, the SMF may select a target UPF, and re-subscribe to the target UPF based on the subscription from UPF event consumer in step 1. The target UPF will send the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

# * * * * Next change * * * *

4.15.4.5.Y	Information flow for subscription directly to UPF event exposure service during UPF relocation

Figure 4.15.4.5.Y-1: Subscription directly to UPF event exposure service during UPF relocation

1.	(For the case when the UPF event consumer is NWDAF) The analytics consumer (e.g. AF/NEF) sends a request to the NWDAF for analytics, and indicates a single UE (e.g. UE IP address) in the Target of Analytics Reporting. Analytics Filter Information optionally contains DNN, S-NSSAI, Area of Interest, Application server IP address/FQDN, Application ID, DNAI, etc.

2.	The UPF event consumer triggers the UPF discovery to NRF by Nnrf_NFDiscovery_Request providing the UE IP address, and optional DNN, S-NSSAI, DNAI, etc.

3.	The NRF provides Nnrf_NFDiscovery_Response that may refer to an UPF.

4.	If the subscribed UPF events allows to directly subscribe to UPF (as defined in clause 5.8.2.17 of TS 23.501 [2]), the UPF event consumer (e.g. NWDAF) triggers the Nupf_EventExposure_Subscribe to the discovered UPF, and the Target of Event Reporting contains a single UE.

5.	The UPF invokes Nupf_EventExposure_Notify service operation directly to the UPF event consumer (e.g. NWDAF).

6.	When the PDU session corresponding to the UE is released, e.g., due to UPF relocation, the UPF determines that the subscription to the UPF event has been terminated.

7.	If step 6 took place, the UPF sends the notification to the UPF event consumer by invoking Nupf_EventExposure_Notify service operation, which includes a subscription termination indication to indicate that the subscription to the UPF event has been terminated, and may also include a cause code indicating the reason for subscription termination, e.g., PDU session release.

# * * * * End of changes * * * *