3GPP TSG-SA2 Meeting #164	S2-2408346

# Maastricht, NL,  19th - 23rd Auguest, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4955 | rev | - | Current version: | V19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update NEF services to support the conclusion of KI#1 and KI#2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA2 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-07-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 8 in 3GPP TS 23.700-77 concludes KI#1 on "Extensible IMS mechanism supporting IMS events in the context of DC communication" and KI#2 on "Impact on IMS architecture, interfaces and procedures to support IMS capability exposure in the context of IMS data channel session". The corresponding normative work need to be implementd. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance the NEF service to support IMS event subscriptoin/notification and data channel establishment, update and termination. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No normative specification for conclusions of KI#1 and KI#2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.6.1, 5.2.6.X (new), 5.2.6.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | Notify |  | AF, NWDAF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_EASDeployment | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_ECSAddress | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_TrafficInfluenceData | Subscribe | Subscribe/Notify | SMF |
|  | Unsubscribe |  | SMF |
|  | Notify |  | SMF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
|  | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | AppRelocationInfo | Subscribe/Notify | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | AF |
|  | TriggerNotify | Subscribe/Notify | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
|  | GroupDeliveryNotify | Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | DeleteNotify | Subscribe/Notify | SMF |
|  | Delivery | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request/Response | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ECRestriction | Get | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ApplyPolicy | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_Location | LocationUpdateNotify | Notify | AF |
| Nnef_TimeSynchronization | ConfigUpdate | Request/Response | AF |
|  | ConfigCreate | Request/Response | AF |
|  | ConfigDelete | Request/Response | AF |
|  | ConfigUpdateNotify | Subscribe/Notify | AF |
|  | CapsSubscribe | Subscribe/Notify | AF |
|  | CapsUnsubscribe | Subscribe/Notify | AF |
|  | CapsNotify | Subscribe/Notify | AF |
| Nnef_ASTI | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
| Nnef_AMPolicyAuthorization | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | Subscribe |  | AF |
|  | Unsubscribe |  | AF |
| Nnef_AMInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_UEId | Get | Request/Response | AF, V-NEF |
|  | UeIdMappingGet | Request/Response | GMLC |
|  | UeIdMappingCreate | Request/Response | AF |
|  | UeIdMappingUpdate | Request/Response | AF |
|  | UeIdMappingDelete | Request/Response | AF |
| Nnef_SMService | MoForwardSm | Request/Response | SMS-SC |
| Nnef_PDTQPolicyNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_MemberUESelectionAssistance | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
| Nnef_AF_request_for_QoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_DNAIMapping | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | UpdateNotify |  | AF, NWDAF |
| Nnef_UEAddress | Get | Request/Response | AF |
| Nnef_ImsEventExposure (ImsEE) | Subscribe | Subscribe/Notify | NF, AF |
|  | Unsubscribe | Subscribe/Notify | NF, AF |
|  | Notify | Subscribe/Notify | NF, AF |
| Nnef_ImsSessionManagement (ImsSM) | Create | Request/Response | NF, AF |
|  | Update | Request/Response | NF, AF |
|  | Delete | Request/Response | NF, AF |

# * * * * Second change * * * *

### 5.2.6.X	Nnef_ImsEventExposure service

##### 5.2.6.X.1	General

The service provides the capability to subscribe, unsubscribe or be notified of IMS events.

##### 5.2.6.X.2	Nnef_ImsEventExposure_Subscribe operation

Service operation name: Nnef_ImsEventExposure_Subscribe

Service description: This service enables the consumer NF to receive an IMS event(s), or if the IMS event(s) are already subscribed in NEF, then the subscription is updated.

Inputs, Required: List of Event Category IDs, Target of Event Reporting (Public Identity(s) of IMS subscriber(s)), Notification endpoint.

Event Category IDs include the following values:

-	IMS_Registration_Event_Category: the events in this category is reported by the HSS that the specific subscriber is registered or unregistered.

-	IMS_Session_Event_Category: the events in this category is reported by the IMS AS when the session status and media of a specific IMS session change and has been defined in 3GPP Rel-18 TS 29.175 [12].

-	IMS_DC_Application_Event_Category: the events in this category is reported by the DCSF when a specific subscriber downloads some IMS data channel application from the DCSF.

If Target of Event Reporting is null, it indicates any IMS subscriber.

Inputs, Optional: Event Filter, Subscription Correlation ID (in the case of modification of the event subscription), Expiry time.

Event Filter contains the parameters which filter a group IMS subscribers whose events need to be monitored, including:

-	Serving Number: it is the subscriber served by the DC AS.

-	Destination Number: it is the called party number and used to subscribe non-subscriber events e.g. an event like "a user has called number 800-123456" where 800-123456 is a DestionationNumber.

Outputs, Required: Result indication and when the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: First corresponding event report is included, if available.

##### 5.2.6.X.3	Nnef_ImsEventExposure_Unsubscribe service operation

Service operation name: Nnef_ImsEventExposure_Unsubscribe

Description: This service operation enables the consumer NF to delete the subscription of an IMS event(s) if already subscribed in the NEF.

Inputs, Required: Subscription Correlation ID.

Outputs, Required: Result indication.

##### 5.2.6.X.4	Nnef_ImsEventExposure_Notify service operation

Service operation name: Nnef_ImsEventExposure_Notify

Description: This service operation enables NEF to report the IMS event(s) to the consumer NF.

Inputs, Required: Event ID, Notification Correlation Information, time stamp.

Inputs, Optional: Event information (defined on a per Event ID basis).

Outputs, Required: Result indication.

# * * * * Third change * * * *

### 5.2.6.Y	Nnef_ImsSessionManagement service

##### 5.2.6.Y.1	General

The service provides the capability to create and release an IMS session and modify the media in a specific IMS session.

##### 5.2.6.Y.2	Nnef_ImsSessionManagement_Create operation

Service operation name: Nnef_ImsSessionManagement_Create

Description: This service enables the consumer NF to create an IMS session with audio, video and/or data channel media.

Inputs, Required: Serving Number, Destination Number, Media Information set.

Serving Number: it is the subscriber served by the DC AS.

Destination Number: it is the called party number and used to subscribe non-subscriber events e.g. an event like "a user has called number 800-123456" where 800-123456 is a DestionationNumber.

Media Inforamaiton set includes a set of media information indicating how the media used in the created session. Each media information contains:

-	Media Type: it indicates the media type used in the created session, and can be Audio, Video or Data Channel.

-	Media Parameter: it indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Audio or Video, it need indicate the codec type the consumer NF requires to use.

-	When the Media Type is Data Channel, it need indicate the Data Channel Type (ADC or BDC).

-	When ADC is selected, it need indicate the ADC type (P2A, P2P, P2A2P) and the party with wich the P2A ADC is used.

-	When BDC is selected, it need indicate the party (the caller, or the callee) with which the BDC is used.

Inputs, Optional: DC Info.

DC Info indicates the attributes of data channel to be created when initiating the call.

Outputs, Required: Result indication and when the operation is successful: Session ID.

Outputs, Optional: None.

##### 5.2.6.Y.3	Nnef_ImsSessionManagement_Update service operation

Service operation name: Nnef_ImsSessionManagement_Update

Description: This service operation enables the consumer NF to modify the media in a specific IMS session, such as creating or releasing bootstrap data channel(s) or application data channel(s).

Inputs, Required: Session ID, Media operation set.

Media operation set includes a set of media operations commands indicating how to modify the media used in the specific session. Each media operation command contains:

-	Operation Type: it indicates adding, updating or removing the corresponding media.

-	Media Type: it indicates the operated media is Audio, Video or Data Channel.

-	Media Parameter: it indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Audio or Video, it need indicate the codec type the consumer NF requires to use.

-	When the Media Type is Data Channel, it need indicate the Data Channel Type (ADC or BDC).

-	When ADC is selected, it need indicate the ADC type (P2A, P2P, P2A2P) and the party with wich the P2A ADC is used.

-	When BDC is selected, it need indicate the party (the caller, or the callee) with which the BDC is used.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

##### 5.2.6.Y.4	Nnef_ImsSessionManagement_Delete service operation

Service operation name: Nnef_ImsSessionManagement_Delete

Description: This service operation enables the consumer NF to release the specific IMS session.

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

# * * * * End of changes * * * *