SA WG2 Meeting #164	S2-2408397

Maastricht, NL, 19 - 23 August, 2024	revision of S2-240yyyy

Source:	vivo

Title:	Evaluation for DualSteer KI#1.3

Document for:	Approval

Agenda Item: 	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes evaluation on KI#1.3.

1	Discussion

This paper proposes evaluation on KI#1.3.

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V1.0.0.

* * * * Start of Changes (all new text) * * * *

7.1.3	Evaluation for Key Issue #1.3

There are eleven solutions address the Key Issue #1.3, following table shows the way proposed by solutions for the four critical aspects of DualSteer session management:

| Sol. | SMF selection for source PDU Session | Information sent by DualSteer device for both PDU Sessions | Session anchoring method | Switch mechanism |
| --- | --- | --- | --- | --- |
| Sol. #1.1 | 1. Based on DNN/S-NSSAI specific for DualSteer (Alt1) 2. AMF selects SMF based on DualSteer capability when device indicates DualSteer (Alt2) | 1. Use same (DNN, S-NSSAI) as source session 2. Includes DS Correlation ID | 1. Linked SUPI and <DS Correlation ID, SMF ID> pair in subscription data 2. SMF sends DS Correlation ID to UDM for update subscription data. 3. SMF of target session triggers HTTP redirect based on subscription data of linked SUPI (Alt1) 4. Target AMF selects same SMF based on subscription data of linked SUPI (Alt2) 5. SMF anchors both sessions based on DS Correlation ID | ATSSS-like 1. SMF sends rules to device during PDU Session Est. procedure 2. Common N4 for both sessions |
| Sol. #1.2 | AMF selects SMF based on DualSteer capability when device indicates DualSteer | 1. Use same (DNN, S-NSSAI) as source session 2. Use same PDU Session ID as source session | 1. Linked SUPI in subscription data 2. Target AMF selects SMF based on subscription data of linked SUPI and PDU Session ID 3. SMF anchors both sessions based on PDU Session ID 4. Same IP address assigned | ATSSS-like 1. SMF sends rules to device during PDU Session Est. procedure 2. Common N4 for both sessions |
| Sol. #1.3 | Based on DNN/S-NSSAI specific for DualSteer | 1. Use same (DNN, S-NSSAI) as source session 2. Includes linked PDU Session ID | 1. Linked SUPI and <DNN/S-NSSAI, SMF Info> pair of the linked SUPI in subscription data 2. When SMF registers in UDM, UDM also updates subscription data of linked SUPI 3. SMF of target session triggers HTTP redirect or SMF redirect based on subscription data (Alt1) 4. Target AMF selects same SMF based on subscription data of linked SUPI (Alt2) 5. SMF anchors both sessions based on Source PDU Session ID 6. Same IP address assigned | Session HO 1. Separate N4 with linkage information between each other 2. Common rules may be used for both N4 sessions |
| Sol. #1.4 | Based on DNN/S-NSSAI specific for DualSteer | MA PDU Session over non-3GPP access with underlayer 3GPP access | ATSSS | ATSSS |
| Sol. #1.5 | AMF selects SMF based on DualSteer capability when device indicates DualSteer | 1. Use same (DNN, S-NSSAI) as source session 2. Includes linked PDU Session ID | 1. Target AMF selects same SMF based on subscription data of linked SUPI and linked PDU Session ID 2. SMF anchors both sessions based on linked PDU Session ID 3. Same IP address assigned | ATSSS |
| Sol. #1.11 | Based on DNN/S-NSSAI specific for DualSteer | Use same (DNN, S-NSSAI) as source session | 1. When SMF registers in UDM, UDM updates subscription data of linked SUPI with the SMF info 2. Target AMF selects same SMF based on subscription data | ATSSS-like 1. SMF sends rules to device during PDU Session Est. procedure |
| Sol. #1.12 | Based on DNN/S-NSSAI specific for DualSteer | 1. Use same (DNN, S-NSSAI) as source session 2. Both target and source session include same DS Session ID (new) | 1. SMF redirect based on the subscription data of both SUPIs 2. SMF anchors both sessions based on DS Session ID | ATSSS |
| Sol. #1.13 | Based on DNN/S-NSSAI specific for DualSteer | MA PDU Session for target with "external" access path information | No CP anchoring 1. Target SMF sends “external IP address” to device 2. Device uses the “external IP address” for the UL traffic | ATSSS 1. ATSSS rules are extend to specify which traffic should be routed via the external access path |
| Sol. #1.14 | Based on DNN/S-NSSAI specific for DualSteer | Not mentioned | 1. SMF registers in UDM with Correlation ID received from AMF 2. Target AMF selects SMF based on Correlation ID in subscription data | Session HO 1. Source PDU Session is released |
| Sol. #1.15 | AMF selects SMF based on DualSteer capability when device indicates DualSteer | 1. Use same (DNN, S-NSSAI, SSC mode) as source session 2. Includes source PDU Session ID | 1. SMF registers in UDM with optional Session Correlation ID allocated by SMF 2. UDM sends the SMF ID and <PDU Session ID, SUPI>/Session Correlation ID to target AMF 3. Target AMF selects SMF based on information from UDM 4. SMF anchors both sessions based on Source PDU Session ID | Session HO 1. Source PDU Session is released |
| Sol. #1.19 | Based on DNN/S-NSSAI specific for DualSteer | 1. Use same (DNN, S-NSSAI) as source session | 1. How to find same SMF is not mentioned 2. SMF anchors both sessions based on (DNN, S-NSSAI) | Not mentioned 1. Target PDU Session is released if single-access is allowed |

Seven of the proposed solutions (1.1, 1.2, 1.4, 1.5, 1.11, 1.12, 1.13) are based on ATSSS or ATSSS-like mechanism for DualSteer traffic switching. Three of the proposed solutions (1.3, 1.14, 1.15) are based on session handover mechanism for DualSteer traffic switching.

Eight of the proposed solutions (1.1, 1.3, 1.4, 1.11, 1.12, 1.13, 1.14, 1.19) use legacy SMF selection method for SMF selection, i.e., specific DNN/S-NSSAI for DualSteer needs to be deployed. Four of the proposed solutions (1.1, 1.2, 1.5, 1.15) assumes AMF can be enhanced to select SMF based on DualSteer capability.

Five of the proposed solutions (1.1, 1.2, 1.3, 1.5, 1.15) requires target AMF selecting same SMF based on subscription data of linked SUPI. Three of the proposed solutions (1.1, 1.14, 1.15) use correlation ID in subscription data for session anchoring at same SMF. Three of the proposed solutions (1.1, 1.3, 1.12) use HTTP redirect or SMF redirect for session anchoring at same SMF. One of the proposed solutions (1.13) reuses ATSSS mechanism for session anchoring at same SMF. One of the proposed solutions (1.13) does not require CP anchoring, only UP anchoring is needed.

Three of the proposed solutions (1.3, 1.5, 1.15) require DualSteer device sending linked PDU Session ID for SMF correlating source and target PDU sessions. Two of the proposed solutions (1.1, 1.12) require DualSteer device sending same additional Correlation ID for SMF correlating source and target PDU sessions. One of the proposed solutions (1.2) requires same PDU Session ID for source and target PDU Sessions.

Two of the proposed solutions (1.4, 1.13) reuse MA PDU Session.

* * * * End of Changes * * * *