3GPP TSG-SA2 Meeting #164	S2-2408472

# Maastricht, Netherlands, 19 - 23 August 2024

Source:	Nokia

Title:	KI#2: NWDAF with internal structure of MTLF, AnLF and ADRF

Document for:	Approval

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract of the contribution: This contribution analyses how to handle VFL at an NWDAF with internal structure of MTLF, AnLF and ADRF for key issue 2. A related CR is in S2-2408495.

# Discussion

In TR 23.700-84, the following conclusion was agreed for KI#2

P#2.1:  VFL related new functionalities include:

P#2.1.1 VFL Server: An NWDAF or AF that integrates local training results for the local ML model update in VFL training process. It also coordinates the VFL training process by discovering and selecting VFL clients. In VFL inference process, The VFL server aggregates local inference results from VFL clients to generate the final VFL inference result and sends the final VFL inference result to the consumer. Only one VFL server exists for each VFL process.

P#2.1.2 VFL Client: An NWDAF or AF that holds the local dataset and performs local training and inference as asked by VFL Server. There can be multiple VFL Clients in VFL training and inference.

P#2.1.3 VFL process is associated with an analytics ID (i.e. VFL training of models for analytics ID, VFL inference for an analytics ID) when VFL server is the NWDAF.

P#2.1.4 VFL process is associated with internal AF process (i.e. VFL training of models for internal AF process, VFL inference for internal AF process) when VFL server is AF.

P#2.1.5: 5GC shall support vertical federated learning (VFL), i.e. a federated learning technique without exchanging/sharing local data set or ML models, in the following scenarios:

-	VFL among NWDAFs in a single PLMN.

-	VFL between AF and NWDAF(s) in a single PLMN.

The NWDAFs as VFL Server determines based on internal logic and the operator's policy whether or not to use VLF to provide a particular analytics ID.

P#2.1.6: NWDAF and AF are the only NFs that may act with any of the above VFL functionalities, i.e. VFL Server and VFL client.

P#2.2:  For registration and discovery of VFL entities:

P#2.2.1 The NWDAF as VFL client shall register to NRF with NF profile including VFL capability information (VFL capability type (i.e. VFL Clients) ). For an untrusted AF as the VFL client, the NEF registers based on configuration at the NRF within its NF profile information about the AF as specified in clause 6.2.2.3 of TS 23.288 [5] and includes as part of the information about the AF an VFL capability information (VFL capability type (i.e. VFL Clients) ).

P#2.2.2 The NWDAF as VFL server will select NWDAF(s) as candidate VFL client(s) and /or AF(s) (via NEF profile for untrusted AF) from NRF for VFL training process.

P#2.2.3 The AF (via NEF in case of untrusted AF) as VFL server will select candidate NWDAF(s) as VFL client(s) from NRF for VFL training process.

NOTE 1:	Whether and how Vendor specific feature information in P#2.2.1 is included will be determined in the normative phase.

P#2.3:  For sample alignment for VFL:

P#2.3.1: For NWDAF acting as VFL Server, NWDAF triggers sample alignment, may query NWDAFs or AFs acting as VFL clients to query availability of samples, and generates the interclause of samples.

P#2.3.2 In case of VFL between the NWDAF as VFL server and AF as VFL client, the NEF may be involved in sample alignment with mapping of information (e.g. internal versus external information) , then NWDAF as VFL server will determine the final list of participants supporting the samples for VFL training.

P#2.3.3: For AF acting as VFL Server and NWDAFs acting as VFL clients, AF performs sample alignment, selects samples to be used in the training process and generates the interclause of samples.

P#2.3.4: Feature description information may be registered to the NRF or locally configured in the NWDAF or the AF, negotiated between VFL server and VFL client when performing feature alignment. Feature alignment is optional in a VFL process.

NOTE 2:	Whether and how the feature alignment is supported will be determined in normative phase.

NOTE 3:	The procedure to perform sample alignment is not agreed yet, i.e. it can be done in a standalone procedure or part of the preparation phase, which will be determined in normative phase.

NOTE 4:	How NEF assists sample alignment in P#2.3.3 will be determined in normative phase.

NOTE 5:	For P#2.3.2 and P#2.3.3, whether and how NEF supports to be involved in generating the intersection of candidate samples is to be discussed in normative phase.

P#2.4:  For VFL training process:

P#2.4.1: Either the NWDAF or the AF can act as VFL server and initiate VFL training process with the VFL client(s).

P#2.4.2: If an untrusted AF is involved in the VFL training process, their interactions with the NWDAF(s) are via NEF. When the NWDAF acts as VFL Server, the NWDAF can receive labels from an AF.

P#2.4.3: An identifier is allocated by a VFL server, which is used to correlate the participants during the VFL training and subsequent VFL inference processes and it is associated with the distributed ML Models in the VFL joint model training process.

P#2.4.4: VFL Clients compute the intermediate results for their local ML models involved in the VFL training and provide reports with the intermediate results to the AF or NWDAF acting as VFL server.

P#2.4.5: VFL clients may also provide intermediate results (e.g. gradient information, loss information) to other VFL clients as instructed by the VFL server.

P#2.4.6: An AF or NWDAF acting as VFL server aggregates intermediate results from VFL client(s), trains a local model, computes intermediate results based on its local ML model, and sends the intermediate results towards VFL clients involved in the joint VFL training process.

P#2.4.7: VFL server may compute different intermediate training information (e.g. gradient information, loss information) for updating its own local model and the models of VFL clients during the VFL training process after processing the received intermediate results (that may include convergence reports), sends the updates to the VFL client(s), and the VFL server/client(s) update their local ML model based on the received information. The VFL server determines when the VFL training process terminates, then it will inform the VFL Clients that the training ends.

NOTE 6:	Whether further specification on how the AF provides the label to NWDAF is needed in P#2.4.2 will be discussed in normative work.

NOTE 7:	Whether and how P#2.4.5 is supported will be determined in normative phase.

NOTE 8:	How the initial model is provided to VFL clients is to be discussed in normative phase.

P#2.5:  For VFL inference process:

P#2.5.1: NWDAF acting as VFL server scenario, the NF consumer of analytics will obtain the required output based on the VFL inference process coordinated by VFL server, which is generated via the VFL inference process between VFL sever and corresponding AFs or NWDAF acting as VFL client(s):

-	For NWDAF acting as VFL server, NWDAF triggers the VFL inference phase after receiving the subscription or request for analytics and delivers the analytics to the NF consumer.

P#2.5.2: For the AF acting as a VFL server, the AF acting as VFL server can also start an inference process with corresponding NWDAF acting as VFL client(s). The AF may be triggered to start the inference by a 5GC consumer (i.e. NWDAF containing AnLF). Interactions are via NEF if AF is untrusted.

P#2.5.3: Before performing the VFL inference, the NWDAF acting as VFL server determines corresponding AF(s) and/or NWDAF(s) as VFL client(s) for the inference process based on the same identifier used in the VFL training process. Related details will be specified in the normative phase.

P#2.5.4: The VFL inference process may be controlled by a set of requirements, i.e. whether the determination if all VFL participants associated with the same identifier are needed in the VFL inference process may be based e.g. on accuracy requirements, the VFL signalling and load cost, contribution weights of each client, and temporal availability of output from VFL participants.

P#2.5.5: When performing VFL model performance monitoring, inference data can be used for model retraining, which is aligned with R18.

NOTE 9:	Whether and how P#2.5.4 is supported will be determined in normative phase.

NOTE 10:	Whether the NWDAF supports both AnLF and MTLF or not during VFL training and inference is to be discussed in normative phase.

NOTE 11:	How multiple NWDAFs are involved in VFL when AF is acting as VFL server is defined in normative phase.

NOTE 12:	The details of the accuracy monitoring related to VFL process will be defined in the normative phase.

NOTE 13:	The details of (optionally new) services and detailed list of parameters to enable the VFL processes will be defined in the normative phase.

NOTE 14:	Details of the ML model storage will be defined in the normative phase.

This contribution provides additional analysis related to the internal split of the NWDAF into AnLF, MTLF and ADRF.

Observation 1: In the above conclusion, The VFL Server and Client doesn’t consider the existing internal Split of NWDAF Training Instance (Model Training Logical Function (MTLF)) and NWDAF Inference Instance (Analytics Logical Function (AnLF)). This may lead to inconsistencies and inefficiencies in optimizing the VFL training and inference Procedures”.

Proposal 1:	NWDAF internal Split i.e. MTLF and AnLF should be considered while defining Vertical Federated Learning and Inference Procedures The following roles can be defined

- VFL Server:

  - The NWDAF acting as a VFL Server should explicitly leverage its internal split:

    - NWDAF containing MTLF (Model Training Logical Function(MTLF)): This component should manage and coordinate the VFL training process. It will aggregate intermediate results from VFL Clients, update the global model, and distribute updates back to the clients. The role can be designated as “VFL training server”

    - AnLF containing AnLF (Analytics Logical Function(AnLF)): This component should handle the VFL inference process. After the model is trained, the AnLF will coordinate the inference, aggregate results from VFL Clients, and generate the final analytics output to be sent to the requesting entity. The role can be designated as “VFL inference server”

- VFL Client:

  - NWDAF acting as a VFL Client should also utilize its internal Split architecture:

    - NWDAF containing MTLF: Handles local model training, computes intermediate results, and communicates these results to the VFL Server (NWDAF/MTLF). The role can be designated as “VFL training client”

    - NWDAF containing AnLF: Performs local inference and analytics, contributing to the overall inference process managed by the VFL Server (NWDAF/AnLF). The role can be designated as “VFL inference client”

Proposal 2:	The following VFL capabilities should be defined: VFL training server VFL training client, VFL inference server, VFL inference client.

Observation 2: In the above conclusion, it is not clear where the VFL trained Model is stored after training is concluded”.

However according to the existing procedures storage of any models at the ADRF should be supported

Proposal 3: Storage of VFL related models at ADRF should be supported.

Observation 3: VFL models that were trained together need to be used together for inference.

Observation 4: The already agreed correlation ID allocated by the VFL server can be used to correlate models trained together.

Proposal 4: Store the correlation ID together with models trained by VFL at the ADRF.

Observation 5: VFL Client models relate to special features that can be identified by vendor-specific feature information

Proposal 5: For client models, store vendor-specific feature information together with models trained by VFL at the ADRF.

Observation 6: According to existing procedures, an AnLF retrieves a trained model from the MTLF

Proposal 6: The VFL inference server ANLF retrieves the server model from the VFL training server MTLF

Proposal 7: The VFL inference client ANLF retrieves the client model for a feature from the corresponding VFL training client.

Proposal 8: The VFL training server provides the correlation ID and the features used for the model training together with the model to the VFL inference server.

Proposal 9: The VFL inference server provides the correlation ID to the VFL inference clients when requesting them to do inference-

Proposal 10: The VFL inference clients use the Correlation ID to retrieve models from the VFL training clients.

Summary of Proposals

Proposal 1:	NWDAF internal Split i.e. MTLF and AnLF should be considered while defining Vertical Federated Learning and Inference Procedures.

Proposal 2:	The following VFL capabilities should be defined: VFL training server VFL training client, VFL inference server, VFL inference client.

Proposal 3:	Storage of VFL related models at ADRF should be supported.

Proposal 4: Store the correlation ID together with models trained by VFL at the ADRF.

Proposal 5: For client models, store vendor-specific feature information together with models trained by VFL at the ADRF.

Proposal 6: The VFL inference server ANLF retrieves the server model from the VFL training server MTLF

Proposal 7: The VFL inference client ANLF retrieves the client model for a feature from the corresponding VFL training client.

Proposal 8: The VFL training server provides the correlation ID and the features used for the model training together with the model to the VFL inference server.

Proposal 9: The VFL inference server provides the correlation ID to the VFL inference clients when requesting them to do inference-

Proposal 10: The VFL inference clients use the Correlation ID to retrieve models from the VFL training clients.