3GPP TSG-SA2 Meeting #164	S2-2408506

Maastricht, Netherlands, August 19 – 23, 2024

Title:	[draft] LS on FS_XRM_Ph2: L4S and PDU Set support in non-3GPP access

Response to:

Release:	Rel-19

Work Item:	FS_XRM_Ph2

Source:	Nokia, will be SA2

To:	BBF

Cc:

# Contact Person:

Name:	Georgios Gkellas

E-mail Address:	georgios.gkellas@nokia.com

Send any reply LS to:		3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1. Overall Description:

SA2 has evaluated solutions for key issues for the study of XR and Media Services Ph2 (TR 23.700-70). SA2 would like to coordinate with BBF on the following conclusions agreed in SA2 for KI#6, KI#7:

## 8.6	Conclusions for Key Issue #6

The following aspects are concluded as principles for the normative work:

1.	Dedicated 5G QoS Flow(s) and non-3GPP access resources (e.g. IPsec Child SAs) are used for carrying L4S enabled IP traffic.

2.	For wireline access:

a.	ECN marking for L4S is supported in W-AGF. It is controlled via N2 signalling (Indication of ECN marking for L4S for a corresponding QoS Flow(s)) and applies to proper mapping between L4S-enabled QoS profile(s) and L4S-enabled W-UP resource(s).

b.	ECN marking for L4S is supported in 5G-RG in UL. It is controlled via N1 signalling (Indication of ECN marking for L4S for a corresponding QoS Flow(s)) and applies to proper mapping between L4S-enabled QoS rule(s) and L4S-enabled W-UP resource(s).

3.	For untrusted/trusted access:

a.	ECN marking for L4S is supported in N3IWF/TNGF. It is controlled via N2 signalling (Indication of ECN marking for L4S for a corresponding QoS Flow(s)) and applies to proper mapping between L4S-enabled QoS profile(s) and L4S-enabled IPsec Child SAs.

b.	N3IWF/TNGF in UL shall support and UE in DL can support the IP-in-IP tunnel behaviour of copying ECN bits between outer and inner headers as per IETF RFC 6040 [47].

NOTE:	To support this functionality, the UE needs to support UL L4S feedback as described in IETF RFC 9330 [14] which is not in the scope of 3GPP.

## 8.7	Conclusions for Key Issue #7

The following aspects are concluded as principles for the normative work:

1.	For wireline access:

a.	W-AGF:

i.	In addition to PDU QoS parameters, uses the PDU Set QoS parameter(s) sent by SMF over N2 to determine W-UP resources.

ii.	Uses PDU Set Information received in the GTP-U header over N3 to identify PDU Sets.

b.	5G-RG:

i.	5G-RG may receive the UL Protocol Description associated with the QoS rule over N1 from SMF to identify PDU Sets.

ii.	How the 5G-RG may identify PDU Sets and determine W-UP resources to use is left up to 5G-RG implementation.

2.	For untrusted/trusted access:

a.	N3IWF/TNGF:

i.	In addition to PDU QoS parameters, uses the PDU Set QoS parameter(s) sent by SMF over N2 to determine IPsec Child SAs.

ii.	Uses PDU Set Information received in the GTP-U header over N3 to identify PDU Sets.

iii.	Based on operator local configuration, optionally, uses the PDU Set Importance received in the GTP-U header to determine the drop precedence of DSCP value for the DL packets.

b.	UE:

i.	As specified in clause 5.37.5.1 of TS 23.501 [2], UE may receive the UL Protocol Description associated with the QoS rule over N1 from SMF to identify PDU Sets. Whether and how the UE may identify UL PDU Sets is left up to UE implementation.

# 2. Actions:

# To BBF:

ACTION: SA2 kindly asks BBF to consider the conclusions above and coordinate with SA2 when needed.

# 3. Date of Next SA2 Meetings:

SA2#165	14th Oct. – 18th Oct. 2024					Hyderabad, India

SA2#166	18th Nov. – 22nd Nov. 2024				Orlando, USA