3GPP TSG-WG SA2 Meeting #164	S2-2408535

Maastricht, NL, 19th Aug –23rd Aug, 2024	(revision of S2-240xxxx)

Source:	Qualcomm Incorporated

Title:	Conclusion on KI#1 of FS_5G_Femto

Document for:	Approval

Agenda Item:	19.12

Work Item / Release:	FS_5G_Femto / Rel-19

Abstract: This contribution proposed a conclusion for KI#1 of FS_5G_Femto study.

# 1. Introduction/Discussion

The Key Issue #1 of TR 23.700-45 contains two aspects:

-	The mobility scenarios to be studied for the UE move (e.g. CSG cell to CAG cell of 5G Femto, vice versa).

-	Whether and how control signalling procedures are enhanced to support the mobility scenarios.

NOTE 1:	This Key Issue assumes that the existing CAG concept defined for PNI-NPN is re-used for access control when a UE wants to access a 5G Femto cell without impacts to PNI-NPN.

NOTE 2:	This Key Issue expects that solutions avoid impacts on EPC (e.g. MME), E-UTRAN and NG-RAN.

During the study phase, both the IDLE mode and CONNECTED mode mobility scenarios were reviewed.

For the IDLE mode mobility, solution #7 presented the approach reusing existing mechanisms, without any additional enhancements. Solution#10 proposed to have a fixed mapping between CAG and CSG IDs. However, such mapping operation should be achieved by implementation, e.g. by mapping the Allowed CAG ID list and CSG ID list, and use that as manual selection inputs. It should not affect the AS layer behavior of the UE and should not require NAS layer changes, because such changes would render the solution not usable for exist UEs.

Proposal 1: To conclude KI#1 that no normative work is required for IDLE mode mobility between CSG cells and CAG cells.

For CONNECTED mode mobility, there should be not impacts to EPC, E-UTRAN, NG-RAN, as noted in the SID. Based on the existing CSG design in 4G, during the CONNECTED mode mobility, the CSG verification is always performed at the MME (per TS 36.300 clause 10.5.1.2 for inbound mobility to CSG/Hybrid cells):

Figure 10.5.1.2-1: Mobility to HeNB's CSG and hybrid cells.

In addition, in TS 23.401, the HO procedures also clearly states that the source MME performs the access control check, e.g. in clause 5.5.1.2.2 S1-based handover, normal:

3.	The source MME selects the target MME as described in clause 4.3.8.3 on "MME Selection Function" and if it has determined to relocate the MME, it sends a Forward Relocation Request (MME UE context, Source to Target transparent container, RAN Cause, target eNodeB Identity, CSG ID, CSG Membership Indication, target TAI, MS Info Change Reporting Action (if available), CSG Information Reporting Action (if available), UE Time Zone, Direct Forwarding Flag, Serving Network, Local Home Network ID, LTE-M UE Indication) message to the target MME. The target TAI is sent to the target MME to help it to determine whether S-GW relocation is needed (and, if needed, aid SGW selection). The old Serving Network is sent to target MME to support the target MME to resolve if Serving Network is changed.  In network sharing scenarios Serving Network denotes the serving core network.

The source MME shall perform access control by checking the UE's CSG subscription when CSG ID is provided by the source eNodeB. If there is no subscription data for this CSG ID or the CSG subscription is expired, and the target cell is a CSG cell, the source MME shall reject the handover with an appropriate cause unless the UE has emergency bearer services.

The MME UE context includes IMSI, ME Identity, UE security context, UE Network Capability, AMBR, Selected CN operator ID, APN restriction, Serving GW address and TEID for control signalling, and EPS Bearer context(s).

…

It is clear that the Access Control for the CSG subscription is done at the EPC, i.e the source MME, instead of the source RAN node. Therefore, for the 5GS to EPS CSG cell handover, similar principle could be applied, i.e. to perform subscription based verification at the source AMF, similar to that of the proposal in sol#6.

Although for the 5GS to EPS CSG Handover scenario, there is no CSG ID provided by the Source NG-RAN to AMF, in Release 17, to support Handover to open HeNB cell, the HeNB ID was provided over the NGAP towards the AMF in the signalling. Therefore, the AMF can use a configured mapping between the target HeNB ID and the CAG ID to perform access control, using existing Allowed CAG List from the subscription.

This enhancement at the source AMF addressed the issue identified by RAN3 in Rel-17 LS exchanges for closed or hybrid CSG cell HO support.

In addition, for the CSG ID in the Handover Request to the Target CSG Cell/HeNB, it was meant to verify the UE has reported CSG ID correctly (e.g. it did not fake a CSG ID for the cell). Given that the solutions for 5GS to EPS CSG cell HO uses the target HeNB ID to perform the access control, there is no need to perform such verification at the target HeNB, as UE doesn't report any.

In the current RAN3 specifications (i.e. TS 36.413), the target HeNB behaviour description only mandated the CSG ID verification when it is provided in the HO Request: (e.g. TS 36.413 clause 8.4.2.2 for Handover Resource Allocation):

If the CSG Id IE is received in the HANDOVER REQUEST message, the eNB shall compare the received value with the CSG Id broadcast by the target cell.

If the CSG Membership Status IE is received in the HANDOVER REQUEST message and the CSG Membership Status is set to “member”, the eNB may provide the QoS to the UE as for member provided that the CSG Id received in the HANDOVER REQUEST messages corresponds to the CSG Id broadcast by the target cell.

If the CSG Membership Status IE and the CSG Id IE are received in the HANDOVER REQUEST message and the CSG Id does not correspond to the CSG Id broadcast by the target cell, the eNB may provide the QoS to the UE as for a non member and shall send back in the HANDOVER REQUEST ACKNOWLEDGE message the actual CSG Id broadcast by the target cell.

If the target cell is CSG cell or hybrid cell, the target eNB shall include the CSG ID IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the target eNB receives the CSG Id IE and the CSG Membership Status IE is set to “non member” in the HANDOVER REQUEST message and the target cell is a closed cell and at least one of the E-RABs has a particular ARP value (see TS 23.401 [11]), the eNB shall send back the HANDOVER REQUEST ACKNOWLEDGE message to the MME accepting those E-RABs and failing the other E-RABs.

Therefore, there seems to be no mandatory behaviour requirements for the target HeNB to perform any verification if there is no CSG ID included in the Handover Request message.

In this case, the source AMF in solution#6 may not even need to provide any CSG ID to the target MME in the Relocation Request. This however depends on the RAN3 verification of the target HeNB behaviour, i.e. whether a CSG cell will perform check if no CSG ID is provided in Handover Request.

Proposal 2: To conclude KI#1 that for CONNECTED mode mobility between CSG cells and CAG cells, solution#6 should be used as the baseline.

Proposal 3: To send an LS to RAN3 to verify the HeNB behaviour when no CSG ID is provided in the Handover Request message to the CSG cell. SA2 normative work can be aligned with the answer from RAN3.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-45.

# * * * * First change * * * *

## 8.X	Key Issue #1: Support of UE move between CAG cell of 5G Femto and CSG cell

It is concluded that:

-	For IDLE mode mobility, no normative work is needed.

-	For CONNECTED mode mobility, Solution#6 is used as baseline.

# * * * * End of changes * * * *