3GPP TSG-WG SA2 Meeting #164	S2-2408548

Maastricht, NL, 19th Aug – 23rd Aug, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Proposal on open issues of NTZ provisioning to UAV

Document for:	Approval

Agenda Item:	19.10.1

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract: open issues on NTZ provisioning to UAV is discussed and solution is proposed.

1. Introduction/Discussion

TR 23.700-59, v1.0.0 includes the below interim conclusion:

8.3.2	Configuring/Provisioning NTZ assistance information to UAV UE and NTZ enforcement by UAV UE

… (void unrelated texts)

The above NTZ assistance information is made available to the UAV UE by using one or more following ways:

-	pre-configured;

-	provided/updated by the AMF during the registration procedure;

-	provided/updated by the AMF during the UE Configuration Update procedure specified in clause 4.2.4.2 of TS 23.502 [4];

-	provided/updated by the MME during the attach procedure;

-	provided/updated by the MME during the TAU procedure;

-	provided/updated by the PCF;

-	provided/updated by the AF (e.g. NTZ AF, USS, TPAE).

NOTE 2:	Whether all options are required will be determined in normative phase.

This contribution proposes to address the NOTE 2 with the following justification.

Pre-configuration of NTZ on the UAV is possible if the UAV manufacturer is asked to configure the NTZ to the UAV, for example, requested by local regulator. This option has no 3GPP standard impact.

If the 5G core network support the NTZ assistance information provisioning to UAV, the NTZ information is a type of (new) policy data stored in the PCF.

PCF does not need to understand the meaning of NTZ assistance information.

During UE registration, after verifying the UAV by UUAA mechanism, AMF requests the PCF to provide the NTZ information and further delivers to the UE.

If AMF is notified by PCF of the update of NTZ assistance information, and AMF stores the UAV UE context, AMF may use the UE Configuration Update procedure to overwrite the existing NTZ assistance information with updated one.

In case of MME, the storage of NTZ assistance information is on the PCRF and it is the MME to provision the NTZ information to UE.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-59.

# * * * * First change * * * *

8.3.2	Configuring/Provisioning NTZ assistance information to UAV UE and NTZ enforcement by UAV UE

The following NTZ assistance information (or NTZ policy) is configured/provisioned in the UAV UE.

-	A list of the restricted frequency band(s) with Geographical Area (including coordinates with longitude, latitude and height restrictions).

NOTE 1:	It is assumed UAV UE is configured with NTZ assistance information before accessing to the operator's network, if the local regulation requires NTZ.

The above NTZ assistance information is made available to the UAV UE by using one or more following ways:

-	pre-configured;

-	provided/updated by the AF (e.g. NTZ AF, USS, TPAE).

If NTZ assistance information is stored in the PCF in 5GS or PCRF in EPS, it may be provisioned to UAV UE using the below options:

-	provided/updated by the AMF during the registration procedure;

-	provided/updated by the AMF during the UE Configuration Update procedure specified in clause 4.2.4.2 of TS 23.502 [4];

-	provided/updated by the MME during the attach procedure;

-	provided/updated by the MME during the TAU procedure;

-	provided/updated by the AF (e.g. NTZ AF, USS, TPAE).

The UAV UE is responsible for NTZ enforcement independently of the UAV UE 3GPP Release.

The UAV UE supporting NTZ (i.e. Rel-19 and onward UAV UEs) performs the following:

a)	enforces NTZ based on the configured/provisioned NTZ assistance information.

b)	upon applying NTZ enforcement, the UAV UE does not transmit any signalling or data.

c)	does not select/reselect any cells impacted entirely by an NTZ.

NOTE 3:	Whether indication about UAV UE's NTZ support to the AMF/MME is needed will be decided during normative phase. This work does not have RAN impacts.

NOTE 4:	For the network to know about UAV presence inside NTZ, whether registration updates by the UAV UE before entering the NTZ and when exiting the NTZ are needed or the existing mechanism (e.g. AMF subscribing with NG-RAN by providing NTZ as AoI) can be applied will be decided during normative phase. This work does not have RAN impacts.

NOTE 5:	Whether extension of Service Restrictions for NTZs using Restricted Transmission Area (RTA) mapped from the NTZ information is needed will be discussed during normative phase. This work does not have RAN impacts.

# * * * * End of changes * * * *