3GPP TSG-WG SA2 Meeting #164	S2-2408553

Maastricht, NL, 19th Aug – 23rd Aug, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Conclusion on Ambient IoT Device Identification

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: this contribution proposed conclusion principles for key issue#2 --- Ambient IoT Device identification.

1. Introduction/Discussion

This contribution proposed conclusion principles for key issue#2 --- Ambient IoT Device identification.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * First change * * * *

8	Conclusions

# * * * * Second change * * * *

8.a	General Principles

-	Characteristics of Ambient IoT Device include: low complexity, limited energy storage capability

Editor’s Note: Other characteristics of an Ambient IoT Device are FFS.

-	An Ambient IoT Device does not differentiate the Ambient IoT traffic from/to the BS Reader in topology 1 or UE reader in topology 2

# * * * * Third change * * * *

8.c	Conclusion for key issue #2

8.c.1	Ambient IoT Device Identification

The following principles are agreed for Ambient IoT Device identification:

-	Ambient IoT Device Identifier is either assigned by an operator or by a third-party and is stored in the Ambient IoT Device’s non-volatile memory.

The length of Ambient IoT Device Identifier is variable according to the service requirements.

Editor’s Note: How to distinguish between the two assignment options of the Ambient IoT Device Identifier is FFS.

In the case that the Ambient IoT Device Identifier is assigned by an operator:

-	The Ambient IoT Device Identifier consists of a Network Identifier and an Operator Allocated Identifier:

-	The Network Identifier is an identifier to indicate the network operator (e.g. MMC+MNC) which allocated the Device ID and holds the subscription-like information for the AIoT Device.

-	The Operator Allocated Identifier is an identifier used to identify the specific Ambient IoT Device.

NOTE:	The Operator Allocated Identifier format is determined by the operator and can follow any format. For example, it can be divided into subfields e.g. Enterprise ID or Group ID and an Instance ID, or use any other format, e.g. EPC.

-	The Ambient IoT Device Identifier is included in the subscription-like information of the Ambient IoT Device, and the subscription-like information is either stored in the UDM or in a AAA server.

-	The Ambient IoT Device Identifier can be validated by AIoTMF.

Editor’s Note: Other information stored in the subscription-like information for an Ambient IoT Device is FFS.

In the case that the Ambient IoT Device Identifier is assigned by a third-party:

-	The Ambient IoT Device Identifier consist of a Network Identifier and an Application Allocated Identifier:

-	The Network Identifier is an identifier to indicate the operator (e.g. MCC+MNC) who holds the subscription-like information for the Ambient IoT Device. The Network Identifier may be set to a pre-defined/reserved value, e.g. 999999, if no operator holds subscription-like information for the Ambient IoT Device.

-	The Application Allocated Identifier is an identifier used to identify the specific Ambient IoT Device allocated by the 3rd Party, for example an EPC.

-	The Network Identifier is used by the MNO to derive  subscription-like information related to Ambient IoT Device.

-	The Ambient IoT Device Identifier is included in the subscription-like information of the Ambient IoT Device, and it is either stored in the UDM or in the AAA server.

-	Ambient IoT Device ID validation is supported by AIoTMF, if the AIoTMF is able to access the credential holder (e.g. UDM or AAA server) of the Ambient IoT Device.

# * * * * End of changes * * * *