3GPP TSG-WG SA2 Meeting #164	S2-2408555

Maastricht, NL, 19th Aug – 23rd Aug, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Conclusion on key issue 1 for topology 2 architecture

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: conclusion principles are proposed on key issue 1 for topology 2 architecture.

1. Introduction/Discussion

Conclusion principles are proposed on key issue 1 for topology 2 architecture.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * First change * * * *

8.b	Conclusion on Key Issue #1

8.b.2	Architecture to Support Topology 2

Key issue #1 includes the following aspects:

-	System architecture identified along with the solutions for KI#2 and KI#3.

Key issue#2 aspect on “Ambient IoT Device subscription management” and key issue#3 aspect on “Ambient IoT service exposure” is considered in this section.

The following principles are agreed for the architecture to support topology 2:

-	The AMF is responsible for authorizing a registered UE to act as UE Reader

-	Radio resources required by the UE Reader for Ambient IoT operations are allocated and controlled by RAN

-	A new core network function, AIoTMF, is introduced to support Ambient IoT

Figure 8.b.2-1: illustrative architecture for topology 2

-	A UE reader communicates with an AIoTMF for Ambient IoT service operation, via the AMF

-	AIoT NAS protocol is supported between an Ambient IoT Device and the AIoTMF

# * * * * End of changes * * * *