3GPP TSG SA WG2#164	            S2-2408569

Maastricht, 19-23 August 2024 	(revision of)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4985 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SMF triggering N6 delay monitoring at candidate UPF(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To address the outcome of KI#2 in approved eEDGE_5GC_Ph3 WID SP-240996 on N6 delay measurement, the functionality of SMF to trigger N6 delay measurements at the candidate UPF(s) is introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce SMF to trigger N6 delay monitoring at the candidate UPF(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.1, 4.17.6.1, 5.2.7.2.2, 5.2.26.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR 5606 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 4.15.4.5.1	General

This clause contains the detailed description and the procedures for how the UPF event exposure service (see clause 5.2.26.2) is used for UPF data collection.

The list of NF consumer which may receive UPF event notifications is defined in clause 5.8.2.17 of TS 23.501 [2].

To get exposure data from UPF, NF consumer may subscribe to the UPF directly or indirectly via SMF. This is further defined in clause 5.8.2.17 of TS 23.501 [2].

The UPF event exposure events are described in clause 5.2.26.2. In this Release of the specification, the following events are used for UPF Data collection:

-	QoS Monitoring. This event provides QoS Flow performance information.

-	User DataUsage Measures. This event provides information of user data usage of the User PDU Session.

-	User DataUsage Trends. This event provides statistics related to user data usage of the User PDU Session.

-	N6 Delay Monitoring. This event provides N6 delay measurements between UPF N6 interface and an external measurement endpoint at the data network, as defined in clause 5.8.2.X of TS 23.501 [2].

A consumer of UPF event exposure can subscribe to QoS monitoring event via SMF only and UPF sends the QoS Flow Performance information directly to this consumer. For this event, the interaction between SMF and UPF is over PFCP (TS 29.244 [69]). TS 23.501 [2] describes the QoS monitoring parameters that can be measured by means of QoS monitoring and how to enable the measurements for QoS flows. When the Subscription request for QoS monitoring event indicates that it is for the QoS Flow associated with the default QoS rule, based on local configuration the subscription request may trigger SMF to enable QoS monitoring.

NOTE 1:	Packets from multiple applications may share the QoS Flow associated with the default QoS Rule which may diminish the relevance of some measurements like data rate.

The subscription to QoS monitoring event can target the QoS flows bound to an application by including an Application Identifier. In this case, at subscription request and/or when the PCC rules change, SMF identifies the active PCC Rule that includes a DataCollection_ApplicationIdentifier matching that Application Identifier. SMF enables this consumer (e.g. NWDAF) to receive the QoS Monitoring reports enabled by that PCC Rule. The consumer may indicate that it can receive QoS Flow Performance information for the QoS Flow associated with the default QoS rule if there are no measurements available for the Application Identifier (that is, if no PCC rule is identified). In this case the SMF may instruct the UPF to perform QoS monitoring for the QoS Flow associated with the default QoS rule and include the Indication of QoS Flow associated with the default QoS Rule (see clause 5.8.2.18 of TS 23.501 [2]). The UPF will then include the Indication of QoS Flow associated with the default QoS Rule in the Nupf_EventExposure_Notify service operation when sending reports. Otherwise, the SMF may accept the request and indicate in the response that reporting will be activated when the measurements are enabled by a PCC rule or the SMF may reject the subscription request for that Application Identifier.

NOTE 2:	Extensive usage of QoS Monitoring has significant impact on load and signalling.

A consumer of UPF event exposure such as NWDAF may subscribe to User Data Usage events (i.e. User Data Usage Measures and User Data Usage Trends) directly to UPF (under the conditions defined in clause 5.8.2.17 of TS 23.501 [2]) or via SMF, and UPF sends the event notifications directly to this consumer. For these events, the interaction between SMF and UPF is over SBI. For User Data Usage events, the subscription request targets the traffic matching Event Filter Information (details are described in clause 5.2.26.2.1) for a user's PDU Session or for each PDU Session served by the UPF. The subscription request may indicate the granularity requested, that is whether the measurement reports should be provided per data flow, per application, or per PDU Session.

If the event notification can be delayed, i.e. delay tolerant, Reporting suggestion information is included. The Reporting suggestion information includes Report urgency and Reporting time information. Reporting urgency information indicates whether this event report can be delay tolerant, i.e. the event report can be delayed. If the Reporting urgency information indicates "delay tolerant", the Reporting time is also provided, which defines the last valid reporting time and UPF shall report the detected event before the last valid time.

If a consumer subscribes to an UPF event via the SMF including an AoI ("Area of Interest"), the SMF starts the subscription to the UPF only when the UE is located in the requested AoI. When the UE leaves the AoI, the SMF stops the subscription on the UPF. The SMF may subscribe to AMF about UE moving in or out of an AoI for this purpose. Clause 4.15.4.5.4 describes an alternative procedure for subscription via SMF to UPF event exposure service related with AOI.

Table 4.15.4.5.1-1: Input parameters in subscription to UPF event Exposure events

| Information | To SMF | To UPF (NOTE 4) |
| --- | --- | --- |
| UE identification (NOTE 5) | Y | Y |
| GroupID | Y | N |
| Any UE | Y | Y (NOTE 6) |
| DNN | Y | Y (NOTE 6) |
| S-NSSAI | Y | Y |
| AOI | Y | N |
| BSSID/SSID | Y | N |
| DNAI (NOTE 3) | Y | N |
| UPF Id (NOTE 3) | Y | not applicable |
| Type of Measurement | Y | Y |
| Granularity of Measurement (NOTE 2) | Y | Y |
| Application ID (NOTE 1) | Y | Y |
| Traffic Filtering Information (NOTE 1), (NOTE 2) | Y | Y |
| Reporting suggestion information | Y | Y |
| Measurement endpoint information (NOTE X), (NOTE Y) | Y | Y |
| Measurement protocol(s) (NOTE X) | Y | Y |
| Measurement protocol-specific parameters (NOTE X) | Y | Y |
| NOTE 1: Application ID and Traffic Filtering Information are exclusive. NOTE 2: This parameter does not apply to QoS monitoring event. NOTE 3: This parameter is used to indicate a UP Path as defined in Table 6.4.1-1 of TS 23.288 [50]. NOTE 4: This column is not applicable to events where SMF-UPF interactions are using PFCP (e.g. QoS Monitoring event). That interaction is described in clause 5.8.5 of TS 23.501 [2]. NOTE 5: Input parameter when the target is a UE. SUPI is the input to SMF and UE IP address is the input to UPF. NOTE 6: In a deployment where UPF reporting filtered per DNN and /or S-NSSAI is expected, the SMF should provide UPF with the necessary information over N4. If the SMF is not configured to provide UPF with this information then such UPF reporting filtering is not possible. NOTE X: This parameter is only applicable to N6 delay measurement event. NOTE Y: This parameter can be the IP address(es)/prefix(es) of the external measurement endpoint at a data network which can be independent from any UE PDU session. |  |  |

Second CHANGE

#### 4.17.6.1	General

This clause describes the provisioning of available UPFs in SMF using the NRF as documented in clause 6.3.3 of TS 23.501 [2].

This optional node-level step takes place prior to selecting the UPF for PDU Sessions and may be followed by N4 Node Level procedures defined in clause 4.4.3 where the UPF and the SMF exchange information such as the support of optional functionalities and capabilities.

As an option, UPF(s) may register in the NRF. This registration phase uses the Nnrf_NFManagement_NFRegister operation and hence does not use N4.

For the purpose of SMF provisioning of available UPFs, the SMF uses the Nnrf_NFManagement_NFStatusSubscribe, Nnrf_NFManagement_NFStatusNotify and Nnrf_NFDiscovery services to learn about available UPFs.

NOTE 1:	The protocol used by UPF to interact with NRF is described in TS 29.510 [37]

UPFs may be associated with UPF Provisioning Information in the NRF. The UPF Provisioning Information consists of:

-	a list of (S-NSSAI, DNN);

-	UE IPv4 Address Ranges and/or IPv6 Prefix Range(s) per (S-NSSAI, DNN); and

NOTE 2:	The above information can be used by the SMF for UPF selection when static IP address/prefix allocation is required for a UE.

-	a SMF Area Identity the UPF can serve. The SMF Area Identity allows limiting the SMF provisioning of UPF(s) using NRF to those UPF(s) associated with a certain SMF Area Identity. This can e.g. be used if an SMF is only allowed to control UPF(s) configured in NRF as belonging to a certain SMF Area Identity.

-	the supported ATSSS steering functionality, i.e. whether MPTCP functionality or ATSSS-LL functionality or MPQUIC functionality, or any combination of them is supported.

-	the supported UPF event exposure service and supported Event IDs, e.g. local notification of QoS Monitoring to AF or e.g. events for data collection to NWDAF by Nupf_EventExposure_Notify.

-	the supported functionality associated with high data rate low latency services, eXtended Reality (XR) and interactive media services, specified in clause 5.37 (for example, ECN marking for L4S, specified in clause 5.37.3, PDU Set Marking, specified in clause 5.37.5, UE power saving management, specified in clause 5.37.8).

-	the supported N6 delay measurement protocol. This can be used by SMF to determine candidate UPF(s) to perform N6 delay measurements and EAS selection as defined in clause 6.2 of TS 23.548 [74].

Editor’s Note: The exact measurement protocol(s) to be supported will be decided in collaboration with SA3.

The SMF Area Identity and UE IPv4 Address Ranges and/or IPv6 Prefix Range(s) are optional in the UPF Provisioning Information.

Third CHANGE

##### 5.2.7.2.2	Nnrf_NFManagement_NFRegister service operation

Service Operation name: Nnrf_NFManagement_NFRegister.

Description: Registers the consumer NF in the NRF by providing the NF profile of the consumer NF to NRF and NRF marks the consumer NF available.

Inputs, Required: NF type, NF instance ID, FQDN or IP address of NF, Names of supported NF services (if applicable) and PLMN ID e.g. if NF needs to be discovered by other PLMNs/SNPNs.

NOTE 1:	for the UPF, the addressing information within the NF profile corresponds to the N4 interface.

NOTE 2:	For the purpose of the Nnrf_NFManagement service, the SCP is treated by the NRF in the same way as NFs. Specifically, the SCP is designated with a specific NF type and NF instance ID. However, the SCP does not support services and related NF profile parameters do not apply (e.g. NF Set ID, NF service set ID, Endpoint Address(es) of instance(s) of supported service(s)), see clause 6.2.6.3 of TS 23.501 [2].

Inputs, Optional:

-	If the consumer NF stores Data Set(s) (e.g. UDR): Range(s) of SUPIs, range(s) of GPSIs, range(s) of external group identifiers, Data Set Identifier(s).

-	If the consumer is BSF: Range(s) of SUPIs, range(s) of GPSIs, Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes, IP domain list as described in clause 6.1.6.2.21 of TS 29.510 [37], Range(s) of SUPIs, range(s) of GPSIs.

NOTE 3:	Range of SUPI(s) is limited in this release to a SUPI type of IMSI as defined in TS 23.003 [33].

-	If the consumer is UDM, UDR, PCF, BSF or AUSF, they can include UDM Group ID, UDR Group ID, PCF Group ID, BSF Group ID, AUSF Group ID respectively.

-	For UDM and AUSF, Routing Indicator, or Routing Indicator and Home Network Public Key identifier; Home Network Identifier: PLMN ID in the case of PLMN, PLMN ID + NID in the case of SNPN. Optionally, some NFs may additionally include a Home Network Identifier (including the identification of the CH with AAA Server or DCS with AAA Server) in the form of a realm e.g. in the case of access to an SNPN using credentials owned by CH with AAA Server or in the case of SNPN Onboarding using a DCS with AAA Server.

-	For NSSAAF, Home Network Identifier in the form of a realm e.g. in the case of access to an SNPN using credentials owned by CH with AAA Server or in the case of SNPN Onboarding using credentials from a DCS with AAA Server.

-	If the consumer is AMF, it includes list of GUAMI(s). In addition, AMF may include list of GUAMI(s) for which it can serve as backup for failure/maintenance.

-	If the consumer is CHF, it may include Range(s) of SUPIs, Range(s) of GPSIs, or Range(s) of PLMNs as defined in TS 32.290 [42].

-	If the consumer is CHF, primary CHF instance and the secondary CHF instance pair. If the CHF does not provide NF set ID or NF Service Set ID, it shall provide a primary CHF instance and the secondary CHF instance pair and otherwise it may do so.

-	If the consumer is P-CSCF, the P-CSCF IP address(es) to be provided to the UE by SMF.

-	If the consumer is HSS, IMPI range, IMPU range, HSS Group ID (as defined in TS 23.228 [55]) can be used as optional input parameters.

-	For the UPF Management: UPF Provisioning Information as defined in clause 4.17.6.

-	S-NSSAI(s) and the associated NSI ID(s) (if available).

-	DNN(s) if the consumer is PCF or BSF. DNN(s) per S-NSSAI if the consumer is SMF, UPF or TSCTSF.

-	If the consumer is a trusted AF it may include one or multiple combination(s) of S-NSSAI and DNN corresponding to the AF. In addition, it may include supported Application Id(s), Event ID(s) and Internal-Group Identifier. It may include an indication whether it supports mapping between UE IP address (IPv4 address or IPv6 prefix) and UE ID (i.e. SUPI).

-	Information about the location or serving scope of the NF consumer (operator specific information, e.g. geographical location, data centre).

-	TAI(s).

-	NF Set ID.

-	NF Service Set ID.

-	If the consumer is PCF or SMF, it includes the MA PDU Session capability to indicate if the NF instance supports MA PDU session or not.

-	If the consumer is PCF, it includes the DNN replacement capability to indicate if the NF instance supports DNN replacement or not.

-	If the consumer is PCF or SMF, it includes the slice replacement capability to indicate if the NF instance supports slice replacement or not.

-	If the consumer is PCF, it may include the 5G ProSe Capability as specified in TS 23.304 [77].

-	If the consumer is PCF, it may include the V2X capability as specified in TS 23.287 [73].

-	If the consumer is PCF, it may include the A2X capability as specified in TS 23.256 [80].

-	If the consumer is PCF, it may include the Ranging/SL Positioning Capability as specified in TS 23.586 [88].

-	If the consumer is PCF, it may include the indication of PCF support of URSP delivery in EPS.

-	If the consumer is PCF, it may include the indication of PCF support of VPLMN specific rules.

-	If the consumer is PCF, it may include the indication of PCF support of URSP rule enforcement.

-	If the consumer is NWDAF, it may include:

-	Analytics ID(s) (possibly per service).

-	NWDAF Serving Area information and Supported Analytics Delay per Analytics ID(s) (if available).

-	Analytics aggregation capability and/ or Analytics metadata provisioning capability if such capability is provided by the NWDAF.

-	Roaming exchange capability if such capability is provided by NWDAF.

-	If the consumer NWDAF contains MTLF, it may also include the ML model Filter information parameters S-NSSAI(s) and Area(s) of Interest for the trained ML model(s) per Analytics ID(s) and ML Model Interoperability indicator per Analytics ID(s), if available (see clause 5.2 of TS 23.288 [50]).

-	If the consumer is NWDAF containing MTLF with Federated Learning (FL) capability, it includes FL capability information per analytics ID containing FL capability type (i.e. FL client and/or FL server, if available) and Time interval supporting FL, if available (see clause 5.2 of TS 23.288 [50]).

-	If the consumer is NWDAF containing MTLF with ML Model Accuracy checking capability, it includes ML Model Accuracy checking capability for ML model accuracy monitoring (see clause 5.2 of TS 23.288 [50]).

-	If the consumer is NWDAF containing AnLF with Analytics Accuracy checking capability, it includes Analytics Accuracy checking capability for Analytics Accuracy Monitoring (see clause 5.2 of TS 23.288 [50]).

-	It may also include NF Set ID and NF Type of the NF data sources, if data management service is available.

Details about NWDAF specific information are described in clause 6.3.13 of TS 23.501 [2].

-	If the consumer is ADRF, it may include:

-	Data and analytics storage and retrieval capability if available.

-	ML model storage and retrieval capability if available.

Details about ADRF specific information are described in clause 6.3.20 of TS 23.501 [2].

-	If the consumer is NEF, it may include Event ID(s) supported by AFs, the S-NSSAI and DNN corresponding to the untrusted AF served by the NEF, Application Identifier(s) supported by AFs, range(s) of External Identifiers, or range(s) of External Group Identifiers, or the domain names served by the NEF. It may also include an indication whether the untrusted AF supports mapping between UE IP address (IPv4 address or IPv6 prefix) and external UE ID (i.e. GPSI). If the consumer is local NEF, it may include parameters of list of supported TAI or list of supported DNAI additionally.

-	If the consumer is a NSACF, it includes the S-NSSAI(s) of the PLMN or SNPN where the NSACF is located, the NSAC Service Area Identifier(s) n and NSACF service capabilities. Details about NSAC Service Area Identifier and NSACF service capabilities are described in clause 6.3.22 of TS 23.501 [2].

-	Notification endpoint for default subscription for each type of notification that the NF is interested in receiving.

-	Endpoint Address(es) of instance(s) of supported service(s).

-	NF capacity information.

-	NF priority information.

-	If consumer is NF, SCP domain the NF belongs to.

-	If the consumer is SCP, it may include:

-	SCP domain(s) the SCP belongs to.

-	Remote PLMNs reachable through SCP.

-	Endpoint addresses or Address Domain(s) (e.g. IP Address or FQDN ranges) accessible via the SCP.

-	NF sets of NFs served by the SCP.

-	If the consumer NF is MB-SMF, it may include MB-SMF service area and the MBS Session ID(s), Area Session ID(s), the corresponding MBS service area(s) if available, as specified in TS 23.247 [78].

-	If the consumer is DCCF, the request may include DCCF Serving Area information, NF type of the NF data source, NF Set ID of the NF data sources, support for relocation of data subscription. Details about DCCF discovery and selection are described in clause 6.3.19 of TS 23.501 [2].

-	If the consumer is EASDF, it may include S-NSSAI, DNN, N6 IP address of the PSA UPF, Supported DNS security protocols of EASDF, location as per NF profile and DNAI (if exists).

-	For ON-SNPN, if the consumer is AMF, Capability to support SNPN Onboarding, or, if the consumer is SMF, Capability to support User Plane Remote Provisioning.

-	If the consumer is NEF, it may include the support for UAS NF functionality, the capability to support Multi-member AF session with required QoS and the capability to support member UE selection assistance functionality.

-	If the consumer is UPF and UPF can expose NAT information, it may include the range of IP addresses the NAT uses towards the DN (e.g. public IP addresses). This IP address range may be on a per IP domain, DNN and S-NSSAI.

-	If the consumer is UPF, it may include the supported protocol to be used to measure N6 delay between an external measurement endpoint and UPF N6 interface (as defined in clause 5.8.2.X of TS 23.501 [2]).

-	If the consumer is DCSF, it may include an IMS domain name or a list of IMS domain names it serves, IMPU range of calling identity or called identity it serves, or IMPI range it serves.

-	If the consumer is MF, it includes the data channel media capabilities it supports. It may also include MF location information as specified in TS 23.228 [55].

-	If the consumer is MRF or MRFP, it includes the list of supported IMS media services (as defined in TS 23.228 [55]).

Outputs, Required: Result indication.

Outputs, Optional: None.

See clause 5.21.2.1 of TS 23.501 [2], the AMF registers itself to NRF.

Fourth CHANGE

#### 5.2.26.2	Nupf_EventExposure Service

##### 5.2.26.2.1	General

Service description: This service can expose UPF related information to other NFs. There are several operations for this service:

-	Notifying events to the NFs.

-	Allow consumer NFs to subscribe and unsubscribe for an Event ID on UPF.

The following events can be notified to a NF consumer:

-	QoS Monitoring. This event provides QoS Flow level performance information and can be used for direct event notification of QoS Monitoring reports to a different NF than the SMF as defined in clause 5.8.2.18 of TS 23.501 [2], or it may be for UPF Data collection by NWDAF for analytics (see TS 23.288 [50]) as described in clause 4.15.4.5.

Subscription to this event is always indirect via SMF. The subscription specifies the type of measurement that is being requested. A combination of the information listed below can be requested.

UPF and SMF interact using Session Reporting Rules as defined in clause 5.8.5.11 of TS 23.501 [2].

The event notification may contain following information:

-	QoS monitoring result for the QoS monitoring parameter(s) defined in clause 5.45 of TS 23.501 [2], e.g. UL packet delay, DL packet delay, or round trip packet delay.

-	Indication of QoS Flow associated with the default QoS Rule (if requested by SMF, see clause 4.15.4.5.1).

-	User Data Usage Measures or User Data Usage Trends. These events provide information about traffic matching Event Filter Information (see below) for a user's PDU Session or for each PDU Session served by the UPF. It can be used for UPF Data Collection by NWDAF for analytics (see TS 23.288 [50]) as described in clause 4.15.4.5.

SMF shall use Service Based Interface subscription service operation to subscribe this UPF event. Other direct consumers can subscribe directly to UPF under the conditions defined in clause 5.8.2.17 of TS 23.501 [2]. The subscription request contains the Type of Measurement that is being requested, the Target of Event Reporting and the Event Filter Information (which describes the target traffic) and the required Granularity of Measurement. The Target of Event Reporting can be either a specific PDU Session or "any UE". The Event Filter Information may contain one or more of the following parameters: DNN, S-NSSAI, either Application ID(s) or Traffic Filtering Information. The Granularity of Measurement can indicate per data flow, per application, or per PDU Session. A combination of the information listed below for User Data Usage Measures can be requested.

For User Data Usage Measures, the event notification may contain following event related information:

-	Volume Measurement: measurements of data volume exchanged (UL, DL and/or overall) and/or number of packets exchanged (UL, DL and/or overall) determined for the requested Granularity of Measurement.

-	Throughput Measurement: measurements of data throughput (UL and DL) determined for the requested Granularity of Measurement.

-	Application related Information: URL(s) and/or Domain information (domain name and protocol) detected for the target traffic. This Type of Measurement requires that Application Id(s) or Traffic Filtering Information is provided (i.e. this measurement is not possible to be applied for all traffic handled by the UPF).

For User Data Usage Trends, the event notification may contain following event related information:

-	Throughput Statistic Measurement (average and/or peak throughput) over the measurement period determined for the requested Granularity of Measurement.

For User Data Usage Measures and User Data Usage Trends, if the subscription is targeting any UE, the UPF shall perform the requested measurements for every PDU Session that matches the Event Filter Information and send the information per PDU Session, including the DNN and S-NSSAI and optionally, the sampling ratio achieved by the UPF. When the subscription requests that information is provided per data flow, the notification includes the Packet Filter Set and the Applications Identifier, if available.

NOTE 1:	If the Target of Event Reporting is "any UE", care needs to be taken with regards to load and major signalling impacts and it is expected that the subscription request contains Event Filter Information and appropriate Event Reporting Information (like Sampling ratio and Partitioning criteria, see Table 4.15.1-1) in order to minimize the UPF performance impact of the per PDU Session measurement.

NOTE 2:	The UPF can combine information for different PDU Sessions into one notification message (see TS 29.564 [93]).

-	TSC management information (UMIC, PMIC, NW-TT port number) as defined in clause 5.8.5.14 of TS 23.501 [2].

TSC management information event can be the result of an implicit subscription of the PCF on behalf of the TSNAF/TSCTSF via the Npcf_SMPolicyControl service as described in clause 5.28 of TS 23.501 [2].

- N6 delay measurement. This event provides N6 delay measurements between the UPF N6 interface and an external measurement endpoint in a DN. The measurement endpoint in the DN can be used for delay measurements representing just a single EAS IP address or representing multiple EAS IP addresses within a certain IP address prefix/subnet, for instance when the measurement endpoint is deployed in a node such as a router or load balancer on path or close to the EAS, as defined in clause 5.8.2.X of TS 23.501 [2].

SMF shall use Service Based Interface subscription service operation to subscribe this UPF event. The subscription request contains the Target of Event Reporting, the Event Filter Information, the measurement protocol(s) that can be used, the measurement protocol-specific parameters, the reporting frequency and/or reporting threshold. The Target of Event Reporting can be the IP address(es)/prefix(es) of the external measurement endpoint at an application side which can be independent from any UE PDU session. The Event Filter Information may contain Network Instance.

The event notification may contain the following information:

- N6 delay measurement result per provided IP address(es)/prefix(es). The measurement result may contain minimum, maximum and average values of the performed measurements based on the provided protocol-specific parameters.

If the consumer of UPF service is NWDAF and the target of UE is any UE, according to the Analytic ID from consumer, the NWDAF can decide which kind of information should be collect from UPF and event ID to use. Subscription may be performed directly towards UPF or via SMF as described in clause 5.8.2.17 of TS 23.501 [2]. The UPF collects the data according to the event ID and exposes the related information directly regardless of whether the subscription has been relayed by the SMF.

##### 5.2.26.2.2	Nupf_EventExposure_Notify service operation

Service operation name: Nupf_EventExposure_Notify

Description: This service operation reports the event and information to the consumer that has subscribed (implicitly or explicitly).

Input Required: Event ID, UE address (i.e. IP address or MAC address) or IP address/prefix of the external measurement endpoint in the DN, Notification Correlation Information.

NOTE:	In case Event ID is N6 delay measurement, the IP address/prefix is used as the required input parameter instead of a UE address.

Input, Optional: UE ID, Event specific parameters as described in clause 5.2.26.2.1, time stamps for the measurements, Application Id(s), Packet Filter Set(s), Achieved sampling ratio.

Output Required: Result Indication.

Output, Optional: None.

##### 5.2.26.2.3	Nupf_EventExposure_Subscribe service operation

Service operation name: Nupf_EventExposure_Subscribe

Description: This service operation is used by an NF to subscribe or modify a subscription to UPF event exposure notifications e.g. for the purpose of UPF data collection on a specified PDU Session or for all PDU Sessions of one UE or any UE.

Input, Required: NF ID, Target of Event Reporting (but only a specific UE address (i.e. IP address) or "any UE" or an IP address/prefix of an external measurement endpoint at an application side is allowed), (set of) Event ID(s) defined in clause 5.2.26.2.1, Notification Target Address (+ Notification Correlation ID), Event Reporting Information defined in Table 4.15.1-1 (TS 29.564 [93] defines the possible parameters).

Input, Optional: Subscription Correlation ID (in the case of modification of the event subscription), Expiry time, DNN, S-NSSAI, Network Instance, either Application ID(s) or Traffic Filtering Information, Type of Measurement, Granularity of Measurement, Reporting suggestion information, (for N6 delay measurement event) the measurement protocol(s) and the measurement protocol-specific parameters.

Output, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Output, Optional: First corresponding event report is included, if available (see clause 4.15.1). Notification Target Address (+ Notification Correlation ID) is used to correlate Notifications sent by UPF with this subscription.

End of CHANGES