3GPP SA WG2 Meeting #S2-164	S2-2408572

19 - 23 August, 2024, Maastricht, Netherlands                                	(revision of)

# Title:	[Draft] LS on Measurement protocols to be used to measure N6 delay

# Response to:

# Release:	Rel-19

# Work Item:	FS_eEDGE_5GC_Ph3

Source:	SA2

To:	SA3

Cc:

Contact Person:

Name:	Tezcan Cogalan

E-mail Address:	tezcan.cogalan@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TR 23.700-49

1. Overall Description:

As part of the approved eEDGE_5GC_Ph3 WID (SP-240996), the following objectives are being studied:

- Key Issue #2 (Enhancement of EAS and local UPF (re)selection):

  - SMF can select local PSA UPF considering N6 delay, when available.

  - SMF can collect N6 delay measurement from candidate UPF per pair of 5GC N6 termination point (i.e. UPF) and the measurement endpoint at application side (e.g. EAS/Designated IP (range)) as following:

    - SMF determines the candidate UPF(s) according to the candidate DNAI(s) which is based on the list of DNAI(s) received in AF request or EDI.

    - SMF determines the measurement protocol based on the protocol supported by UPF and the measurement protocol(s) supported by application side (e.g. Pre-configuration in SMF or via AF request).

    - SMF configures the candidate UPF based on the N6 delay monitoring information such as measurement endpoint (e.g. target EAS IP address(es)/Designated IP (range)) per DNAI in the list, the determined measurement protocol and optionally the protocol-specific configuration parameters.

    - UPF(s) performs N6 delay monitoring and reports the N6 delay monitoring result to SMF.

  - N6 delay between 5GC N6 termination point (i.e. UPF) and the measurement endpoint at application side (i.e. EAS IP address(es)/Designated IP (range)) is measured by leveraging existing mechanisms (e.g., ICMP, TWAMP, OWAMP or other protocols defined by IETF).

  - 5GC may enable the N6 delay measurement via AF influence procedure per clause 4.3.6 in TS 23.502[3] or via EAS Deployment Information Management defined in clause 6.2.3.4 in TS 23.548 [5]. When AF influence procedure is used, in Nnef_TrafficInfluence_Create/Update Request, AF can provide an indication of considering N6 delay, a list of DNAI(s), measurement endpoint (e.g. target EAS IP address(es)/Designated IP (range)) per DNAI in the list, the measurement protocol(s) and optionally the protocol-specific configuration parameters.

According to the approved WID, the enhancement of EAS and local UPF (re)selection is based on N6 delay between a measurement endpoint and UPF N6 interface. Performing the N6 delay measurements at UPF aims to leverage the existing protocols defined by IETF. A possible shortlist of common protocols to be used to perform N6 delay measurements can be: ICMP, OWAMP, TWAMP, STAMP.

As the N6 delay measurements are between a 5GS entity and an outside 5GS entity, SA2 seeks an advice from SA3 on the shortlist of the common IETF protocols listed above, based on their security level and impact of enabling the security features/options in these protocols.

2. Actions: SA2 kindly asks SA3 to study the security aspects and provide feedback on the listed protocols that can be used to perform N6 delay measurements.

3. Date of Next SA2 Meetings:

SA2#165	Oct 14 – 18, 2024	Hyderabad, IN

SA2#166	Nov 18 – 22, 2024	Orlando, US