3GPP SA WG2 Meeting #164	S2-2408580

Maastricht, Netherlands, 19 – 23 August, 2024

Source:	Nokia, NEC, Samsung

Title:	KI#1 Evaluations

Document for:	Approval

Agenda Item:	19.12

Work Item / Release:	FS_5G_Femto/ Rel-19

Abstract of the contribution: This paper proposes evaluations of KI#1.

# Discussion

During NWM discussions, the evaluations and conclusions discussion on KI#1 are postponed until we received LS an reply back from RAN2 and RAN3. Although, SA2 hasn’t received RAN3 LS reply yet before SA2#164 meeting, the initial discussion happened at RAN3#124 meeting as well as the received LS reply from RAN2 (S2-2407427) clearly indicate that for a technically feasible solution to enable interworking between a closed 5G Femto CAG and 4G Femto CSG cell would have a RAN (NG-RAN for 5G to 4G, E-UTRAN for 4G to 5G mobility) impact. This finding is aligned with a previously sent RAN3 LS in R3-226035.

Although the NOTE 2 in KI#1 and Editor’s Notes as part of solutions #8, #9 and #11 in TR 23.700-45 (v1.0.0) indicate that the expectation is to have a solution without RAN impact, this seems not possible based on the RAN2 and RAN3 assessments. It is also worth noting that a management (O&M) or AMF-based solution would either (i) have repeated handover requests that would increase a handover failure probability due to blindly triggering the request without a proper access control or (ii) rely on a static configuration of the mapping between 4G CSG and 5G CAG that means it will not support per UE-mobility decision based on allowed access to 4G CSG and 5G CAG cells. Increase of handover failure probability or not making per UE access control for mobility is something that SA2 should avoid as this would go against all the work that RAN WGs have done for many releases to improve and can be considered as a technically not feasible solution. In order to clarify this issue further, an example is given below.

Taking the example of a campus drawn below where each UE has different access rights:

- UE1 is any student which has only access to certain open areas reserved for classes, some equipped with 4G Femtos CSG 11 and some equipped with Femtos CAG 10.

- UE2 is any teacher which has access to student classes + access to some teacher room areas equipped with 4G Femtos of CSG 20.

- UE3 is any member of executive administrative staff leading the campus which has also access to student classes + some administrative areas equipped with 4G Femtos of CSG 30.

- UE4 corresponds to some specific UE3 which has additionally access to the office of the director of the campus which is equipped with a 5G Femto of CAG 40.

- UE5 is the director of campus and can access all areas.

It can be seen that an O&M/AMF-based solution with the static configuration cannot work in the above typical example. This is because:

- CSG 20 can be correlated to CAG 10 only for UE2s and UE5 but UE1s, UE3s, UE4s should be having a restricted access.

- CSG 30 can be correlated to CAG 10 only for UE3, UE4, UE5 but UE1s and UE2s should be having a restricted access.

- CAG 40 can be correlated to CSG30 and CSG 11 only for UE4s and UE5 but UE1s, UE2s, UE3s should be having a restricted access.

Even in the above simple example with limited number of CSGs and CAGs, it is obvious that a mobility decision between a CAG cell and CSG cell cannot be statically configured by O&M or at AMF but depends on each and every UE; and therefore, it requires either:

- Source RAN node to be made aware of the CSG/CAG allowed list of any UE; and/or

- Any UE to report some membership information.

In light of the given discussion above, the evaluations proposed in this document considers all the solutions captured in TR 23.700-45 (v1.0.0) and provides a comparison of different aspects for each solution.

# Proposal

In light of the given discussion above, it is proposed to capture KI#1 evaluations in the FS_5G_Femto TR 23.700-45.

*** First change ***

## 7.2	Key Issue #1: Support of UE move between CAG cell of 5G Femto and CSG cell

Solution #5, #6, #7, #8, #9, #10 and #11 are proposed to address Key Issue#1.

During the study, it is expected that the proposed solutions avoid impacts on EPC (e.g. MME), E-UTRAN and NG-RAN. However, based on liaisons received from RAN2 (S2-2407427) and RAN3 (SP-240xxxx), and previously received LS R3-226035), it is seen that a working and feasible solution would have either RAN (E-UTRAN and/or NG-RAN depending on the mobility direction) and/or MME impact. Therefore, the tables below provide a comparison of different aspects for each solution addressing KI#1 in this TR. The solutions proposed on connected mobility (Table 7.2-1 and Table 7.2-2) and idle mode mobility (Table 7.2-3) are compared in separate tables.

Table 7.2-1: Comparison of access control, RAN and UE impact of solutions on Connected Mobility

| Sol. | Access Control |  | RAN Impact |  |
| --- | --- | --- | --- | --- |
|  | 4G-to-5G Mobility | 5G-to-4G Mobility | 4G-to-5G Mobility | 5G-to-4G Mobility |
| #5 | Source MME | Source gNB | eNB to receive the CSG ID and Membership Status in the NR part of the 4G measurement report. | gNB to receive the CAG ID in the 4G measurement report. |
| #6 | Target AMF | Source AMF | There can be PCI confusion as UE reports PCI only. As target closed CAG cell is reported as an open cell, it will translate to repeated handover failures which results unnecessary handover processing and signalling overhead and degraded handover KPI. | As target closed CSG cell is reported as an open cell, it will translate to repeated handover failures which results unnecessary handover processing and signalling overhead and degraded handover KPI. |
| #8 | Target AMF | Source gNB | There can be PCI confusion as UE reports PCI only. As target closed CAG cell is reported as an open cell, it will translate to repeated handover failures which results unnecessary handover processing and signalling overhead and degraded handover KPI. | NG-RAN node is preconfigured with the CSG information including CSG ID and Cell Access Mode of neighbouring 4G CSG Cells. Performs access control based on UE allowed CSG list. Indicates CSG ID and Access mode to AMF. |
| #9 | Target AMF | Not Applicable | eNB to receive CAG ID in the NR part of the 4G measurement report. eNB to forward CAG ID in a transparent container to target gNB via MME and AMF. It will translate to repeated handover failures which results unnecessary handover processing and signalling overhead and degraded handover KPI if target AMF performs access control. | Not Applicable. |
| #11 | Source eNB | Source gNB | Configuration of PCI to Global RAN Node ID and to CSG/CAG ID encoding information is available at eNB. eNB receives HO restriction list with UE Allowed mapped CSG IDs. eNB performs access control. | Configuration of PCI to HeNB ID and to CSG/CAG ID encoding information |

Table 7.2-2: Comparison of CSG-CAG provisioning, Core Network and PNI-NPN impact of solutions on CONNECTED MODE Mobility

| Sol. | CSG-CAG Provisioning | Core Network Impact | UE Impact |
| --- | --- | --- | --- |
| #5 | A partition of CAG IDs are reserved to be mapped to CSG IDs. When a new CSG/CAG cell is added, CSG/CAG ID can be dynamically assigned. | Yes. Source AMF includes CSG ID to Forward Relocation Request message to target MME. | Yes. Constructs CSG ID (5G-to-4G mobility) or CAG ID (4G-to-5G mobility) based on CSG-CAG ID partitioning. Adds CAG ID to 4G measurement report. Adds CSG ID and membership status to NR part of 4G measurement report. |
| #6 | AMF is configured with the mapping of (target HeNB ID, target CSG ID, mapped CAG ID list). When a new CSG/CAG cell is added, the mapping configuration in AMF needs to be updated by O&M. | Yes. Source AMF includes CSG ID and Cell Access Mode information and CSG membership to Forward Relocation Request message to target MME. Performs access control which duplicates the functionality at NG-RAN node based on the existing PNI-NPN mechanism. Derives, based on correlation information, CSG ID (5G-to-4G mobility), cell access mode and CSG membership indication. | Yes. If CGI reporting is configured at the UE, UE needs to read CGI from the NPN-list instead of the PLMN list. |
| #8 | Not Applicable. UE allowed CSG list and allowed CAG lists are provided to NG-RAN as part of the Mobility Restriction List. | Yes. Source AMF includes CSG ID to Forward Relocation Request message to target MME. AMF needs to retrieve 4G CSG list from the UDM. Includes the allowed 4G CSG list to the mobility restriction list. | No. |
| #9 | Not Applicable. UE allowed CSG list and allowed CAG lists are provided to NG-RAN as part of the Mobility Restriction List. | Yes. Target AMF receives CAG ID in Forward Relocation Request message from the source MME in a transparent container. | Yes. Adds CAG ID to the NR part of 4G measurement report. |
| #11 | A partition of CAG IDs are reserved to be mapped to CSG IDs. When a new CSG/CAG cell is added, CSG/CAG ID can be dynamically assigned. | Yes. Source AMF includes CSG ID to Forward Relocation Request message to target MME. MME provides handover restriction list with UE allowed mapped CSG IDs to E-UTRAN node. | No. |

Table 7.2-2: Comparison of and of solutions on IDLE MODE Mobility

| Sol. | Procedure |  | UE Impact | RAN Impact |
| --- | --- | --- | --- | --- |
|  | 4G-to-5G Mobility | 5G-to-4G Mobility |  |  |
| #7 | Assumes that UE is pre-configured with allowed CAG list before camping on 4G cell. If a CAG cell is not part of UE allowed list, UE can perform manual selection via existing mechanisms. On selection of a CAG by NAS, select any acceptable or suitable cell belonging to the selected CAG and give an indication to NAS that access is possible (for the registration procedure). Manual CAG selection is only supported for a CAG if allowed indicator is broadcasted. | Assumes that UE is pre-configured with allowed CSG list before camping on 5G cell. If a CSG cell is not part of UE allowed list, UE can perform manual selection and initiate tracking area update procedure to 4G via existing mechanisms. If the CSG ID is not present in the CSG subscription, MME rejects tracking area update request. | No. Re-uses the existing mechanism. | Re-uses the existing mechanism. However, if there is an update to UE allowed CAG list while the UE camps on 4G or an update to UE allowed CSG list while UE camps on 5G, the updated list can only be provided to the UE after the UE manually selected CAG/CSG cell and attempted registration/tracking area update procedures. |
| #10 | Assumes that UE is pre-configured with CSG/CAG ID encoding information. UE can be updated with allowed mapped CAG list via existing procedures to update allowed CSG list. If a mapped CAG cell is not part of UE allowed CSG list, UE will not consider this cell as a suitable cell. | Assumes that UE is pre-configured with CSG/CAG ID encoding information. UE can be updated with allowed mapped CSG list via existing procedures to update allowed CAG list. If a mapped CSG cell is not part of UE allowed CAG list, UE will not consider this cell as a suitable cell. | Yes. Constructs CSG ID (5G-to-4G mobility) or CAG ID (4G-to-5G mobility) based on CSG-CAG ID partitioning. Checks mapped CAG ID against the allowed CAG list if the CSG ID is not part of the allowed CSG list. | Re-uses the existing mechanisms. |

*** END of changes ***