3GPP TSG-WG SA2 Meeting #164	S2-2408586

# Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2135 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDU Set handling in wireline access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CableLabs, Charter Communications, Tencent, Tencent Cloud, InterDigital Inc., Lenovo, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-10-zz |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following R19 TR 23.700-70 conclusions for Key Issue #7 (i.e., clause 8.7) were agreed as principles for normative work: - For wireline access: - W-AGF: - In addition to PDU QoS parameters, uses the PDU Set QoS parameter(s) sent by SMF over N2 to determine W-UP resources. - Uses PDU Set Information received in the GTP-U header over N3 to identify PDU Sets. - 5G-RG: - 5G-RG may receive the UL Protocol Description associated with the QoS rule over N1 from SMF to identify PDU Sets. - How the 5G-RG may identify PDU Sets and determine W-UP resources to use is left up to 5G-RG implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifying conclusions on PDU Set handling for wireline access |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature not implemented in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.x, 5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new text) * * * *

## 4.x	Support for high data rate low latency services, eXtended Reality (XR) and interactive media services

### 4.x.1	General

This clause provides an overview of W-5GAN functionalities for support of XR services (AR/VR applications) and interactive media services that require high data rate and low latency communication, as defined in clause 5.37 of TS 23.501 [2] for 5GS:

-	W-5GAN may support ECN markings for L4S, see clause 4.x.2.

-	W-5GAN may support PDU Set based QoS handling, see clause 4.x.3.

### 4.x.3	PDU Set based Handling

PDU Set based QoS handling, as defined in clause 5.37.5 of TS 23.501 [2], by the W-5GAN is determined by PDU Set QoS Parameters in the QoS profile of the QoS Flow and PDU Set Information provided by the PSA UPF.

For QoS Flows with PDU Set based QoS handling enabled, PDU Set QoS Parameters are determined by the PCF (based on information provided by AF and/or local configuration) and provided by SMF to the W-AGF as part of the QoS profile. Alternatively, the SMF may be configured to support PDU Set based QoS handling without receiving PCC rules from PCF. When the W-AGF receives N2 requests related to PDU Session resources, the W-AGF maps the QoS profile(s) received from the 5GC to W-UP level QoS.

For the downlink direction, the PSA UPF identifies PDUs that belong to PDU Sets and provides the PDU Set Information to the W-AGF in the GTP-U header, as described in clause 5.37.5.2 of TS 23.501 [2].

For the uplink direction, 5G-RG may idenitify PDU Sets, and how this is done is left to 5G-RG implementation. The SMF may send UL Protocol Description associated with the QoS rule to the 5G-RG, as described in clause 5.37.5.1 of TS 23.501 [2].

# * * * * Second change * * * *

### 5.1.1	W-AGF

The functionality of W-AGF in the case of Wireline 5G Access network includes the following:

-	Termination of N2 and N3 interfaces to 5G Core Network for control - plane and user-plane respectively.

-	Handling of N2 signalling from SMF (relayed by AMF) related to PDU Sessions and QoS.

-	Relaying uplink and downlink user-plane packets between the 5G-RG and UPF and between FN-RG and UPF. This involves:

-	Enforcing QoS corresponding to N3 packet marking, taking into account QoS requirements associated to such marking received over N2.

-	N3 user-plane packet marking in the uplink.

-	Supporting AMF discovery and selection defined in TS 23.501 [2] clause 6.3.5 where the 5G-S-TMSI is not used for AMF selection since the wireline AS layer can only carry the GUAMI.

-	Termination of wireline access protocol on Y4 and Y5.

-	In the case of FN-RG the W-AGF acts as end point of N1 on behalf of the FN-RG.

-	Support PDU Set Handling as defined in clause 4.x.3 and TS 23.501 [2] clause 5.37.5.

In the case of Wireline 5G Broadband Access network the definition of W-AGF functionalities is specified in BBF TR-456 [9] and WT-457 [10].

NOTE:	The W-AGF is specified as AGF (Access Gateway Function) in BBF TR-456 [9] for supporting 5G-RG and FN-RG and as FMIF (Fixed Mobile Interworking Function) for supporting FN-RG only in the case of presence of BNG in WT-457 [10]. Both cases for FN-RG support, i.e. AGF and FMIF, have identical interfaces towards 5GC, i.e. it is transparent to 5GC whether AGF or FMIF is used and no difference between AGF or FMIF cases is defined in this specification.

In the case of Wireline 5G Cable Access network the definition of W-AGF functionalities is specified by Cablelabs WR-TR-5WWC-ARCH [27].

# * * *  End of changes * * * *