SA WG2 Meeting #164  	S2-2408597

# Maastricht,  NL, August 19-23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1183 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add missing MFAF Services and Service operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-5-5 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT3 in the LS C3-243483 asks that Nmfaf_3daDataManagement_TransferInitiation service operation and the Nmfaf_ContextManagement service be clarified. In TS23.288 clause 6.2.6.3.8 a procedure that includes MFAF relocation uses Nmfaf_3daDataManagement_TransferInitiation request/response service operation and an Nmfaf_ContexManagement service. However, these services / service operations are not defined in clause 9 In TS23.288 clause 6.2.6.3.8 a procedure that includes MFAF relocation uses Nmfaf_3daDataManagement_TransferInitiation request/response service operation and an Nmfaf_ContexManagement service. However, these services / service operations are not defined in clause 9 From TS 23.288 Figure 6.2.6.3.8-1: Procedure for DCCF relocation initiated by DCCF …. Conditional on MFAF transfer: 4. Source DCCF uses the Nmfaf_3daDataManagement service to request transfer of UE MFAF data subscription context to the target MFAF. 5. Target MFAF retrieves MFAF subscription context from Source MFAF. 6. Target MFAF accepts the data subscription(s) context transfer. 7. Target MFAF responds to the DCCF indicating the transfer is complete. The response may contain a Transaction Reference ID from the Target MFAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing Nmfaf_3daDataManagement_TransferInitiation request/response service operation and an Nmfaf_ContexManagement service in clause 9 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is incomplete and self-contradicting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1, new 9.2.4, new 9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

## 9.1	General

Table 9.1-1 shows the MFAF services and MFAF service operations.

Table 9.1-1: NF services provided by MFAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nmfaf_3daDataManagement | Configure | Request / Response | DCCF, NWDAF |
|  | Deconfigure | Request / Response | DCCF, NWDAF |
|  | TransferInitiation | Request/Response | DCCF |
| Nmfaf_3caDataManagement | Notify | Subscribe / Notify | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF |
|  | Fetch | Request / Response | NWDAF, PCF, NSSF, AMF, SMF, NEF, AF, ADRF |
| Nmfaf_ContextManagement | Transfer | Request/Response | MFAF |

2nd change

### 9.2.4	Nmfaf_3daDataManagement_TransferInitiation service operation

Service operation name: Nmfaf_3daDataManagement_TransferInitiation

Description: The consumer may send this to a target MFAF to request that it transfer the MFAF UE context from the indicated MFAF.

Inputs, Required: ID of the MFAF currently holding the UE context, (Set of) UE MFAF Context Identifier(s) (i.e. the Transaction Reference ID that was returned to the consumer when using the Nmfaf_3daDataManagement_Configure service operation to configure or update the MFAF).

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Transaction Reference Id.

3rd change

## 9.4	Nmfaf_ContextManagement service

### 9.4.1	General

Service Description: This service is used to transfer MFAF UE context information to a consumer (e.g. a target MFAF). It may be triggered by a Nmfaf_3daDataManagement_TransferInitiaton request.

### 9.4.2	Nmfaf_ContextManagement_Transfer service operation

Service operation name: Nmfaf_ContextManagement_Transfer

Description: Provides MFAF UE context to a consumer.

Inputs, Required: (Set of) UE MFAF Context Identifier(s) (i.e. the Transaction Reference ID that was returned to the consumer when using the Nmfaf_3daDataManagement_Configure service operation to configure or update the MFAF).

NOTE:	These identifiers can have been received from the DCCF via the Nmfaf_3daDataManagement_TransferInitiation request (see clause 6.2.6.3.8).

Inputs, Optional: None

Outputs, Required: Operation execution result indication.

Outputs, Optional: (Set) of MFAF Context(s) (i.e. the configuration(s) that correspond with the Transaction Reference ID(s) provided as inputs).

End of changes