3GPP TSG SA WG2 Meeting #164	S2-2408600

# Maastricht, Netherlands, August 19 – August 23, 2024                  (revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1184 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New analytics for signaling storm mitigation and prevention |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per the conclusion for KI#4 in TR 23.700-84, the CR proposes required changes to define a new analytics for mitigation and prevention of signaling storm in the 5G network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a clause for the Signaling Storm analytics |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Analytics for signaling storm mitigation and prevention cannot be supported by the NWDAF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all text is new) * * * *

## 6.X	Signalling Storm Analytics

### 6.X.1	General

The Signalling Storm analytics provides statistics and prediction information related to the signalling storm occurred in the network. The signalling storm can be caused by abnormal behaviours of NFs or signalling from massive UEs.

The consumer of these analytics may indicate in the request or subscription:

-	Analytics ID = "Signalling Storm";

-	Target of Analytics Reporting: a list of NFs;

-	Analytics Filter Information:

-	NF IDs involved in the procedure of interest, such as Connection, Registration, Mobility and Session Managements procedures

-	NF IDs with UE or session context of interest

-	Area of Interest (AOI(s))

-	An Analytics target period indicates the time period over which the statistics or predictions are requested;

-	Optionally, preferred level of accuracy of the analytics;

-	Optionally, Reporting Thresholds, which apply only for subscriptions and indicate conditions on the levels to be reached for the respective analytics subsets

-	Optionally, maximum number of reported occurrences.

### 6.X.2	Input Data

The NWDAF collects input data listed in Table 6.X.2-1, 6.X.2-2, and 6.X.2-3, to produce Signaling Storm analytics.

Table 6.X.2-1: Input data from AMF

| Information | Source | Description |
| --- | --- | --- |
| NF ID | AMF | NF instance ID of the service producer or consumer. |
| Signalling exchange information | AMF | NF procedures containing signalling exchange information related to a particular UE or session from the Connection, Registration, Mobility and Session Managements procedures. |
| > Type of requests from UE/RAN | AMF | Request type for N1 or N2 interface, such as received Initial Registration Request, Mobility and Periodic Registration Request, Service Request, etc. |
| > Number of requests from UE/RAN | AMF | Number of requests for N1 or N2 interface, such received Initial Registration Request, Mobility and Periodic Registration Request, Service Request, etc. |
| > Time duration from receiving request from UE/RAN to response to UE/RAN | AMF | Time duration between the request from UE/RAN and response to UE/RAN. |
| > Number of successful responses of UE/RAN | AMF | Number of successful responses associated to their initial requests. |
| > Number of failed responses of UE/RAN | AMF | Number of failed responses associated to their initial requests. |
| > Reason of failed responses of UE/RAN | AMF | Reasons of failed responses associated to their initial requests. |
| > A posterior Type of requests of UE/RAN (0..max) | AMF | A posterior Request types triggered from UE/RAN, for NF Service request, or request to UE/RAN. |
| > > A posterior Type of responses of UE/RAN (0..max) | AMF | A posterior Response types triggered from UE/RAN, for NF Service response, or response to UE/RAN. |
| > Type of requests from NF | AMF | Request type received from NF, e.g. Namf_N1N2Trans, Namf_comm, etc. |
| > Number of requests from NF | AMF | Number of requests received from NF, e.g. Namf_N1N2Trans, Namf_comm, etc. |
| > Time duration from receiving request from NF to response to NF | AMF | Time duration between the request from NF and response to NF. |
| > Number of successful responses of NF | AMF | Number of successful responses associated to their initial requests. |
| > Number of failed responses of NF | AMF | Number of failed responses associated to their initial requests. |
| > Reason of failed responses of NF | AMF | Reasons of failed responses associated to their initial requests. |
| > A posterior Type of requests of NF (0..max) | AMF | A posterior Request types triggered from NF, for NF Service request. |
| > > A posterior Type of responses of NF (0..max) | AMF | A posterior Response types triggered from NF, for NF Service response. |
| > Public Warning information | AMF | Public Warning information such as WRITE-REPLACE WARNING REQUEST, etc. |
| > Number of UE AM Policy Updates | AMF | Number of policy requests from PCF such as URSP, Restriction Area, etc. |
| NF Context information | AMF | NF context information related to a particular UE/Session context. |
| > State transition information | AMF | UE related state transition information such as transition type, frequency of CM state changes etc. State transition identifier: - "Access Type change to 3GPP access"; - "Access Type change to non-3GPP access"; - "RM state change to RM-DEREGISTERED"; - "RM state change to RM-REGISTERED "; - "CM state change to CM-IDLE"; - "CM state change to CM-CONNECTED"; - "Handover"; or - "Mobility Registration Update". |
| > timer information | AMF | UE related timer information such as timer type, duration, etc. |

Table 6.X-2: Input data from SMF

| Information | Source | Description |
| --- | --- | --- |
| NF ID | SMF | NF instance ID of the service producer or consumer. |
| Signalling exchange information | SMF | NF procedures containing signalling exchange information related to a particular UE or session from the Connection, Registration, Mobility and Session Managements procedures. |
| > Type of requests from NF | SMF | Request type received such as PDU Session Establishment, Modification, Release Request, etc. |
| > Number of requests from NF | SMF | Number of requests received such as PDU Session Establishment, Modification, Release Request, etc. |
| > Time duration from receiving request from NF to response to NF | SMF | Time duration between the request from NF and response to NF. |
| > Number of successful responses of NF | SMF | Number of successful responses associated to their initial requests. |
| > Number of failed responses of NF | SMF | Number of failed responses associated to their initial requests. |
| > Reason of failed responses of NF | SMF | Reasons of failed responses associated to their initial requests. |
| > A posterior Type of requests of NF (0..max) | SMF | A posterior Request types triggered from NF, for NF Service request. |
| > A posterior Type of responses of NF (0..max) | SMF | A posterior Response types triggered from NF, for NF Service response. |
| > Number of requests for UE SM Policy Update | SMF | Number of requests from PCF to update UE SM Policy per PDU Session, such as QoS, PCC Rules, etc. |
| > Load information of connected UPFs | SMF | Load information of connected UPFs such as using PFCP Load Control Information. |
| > Number of receiving Session Report from UPFs | SMF | Number of receive Session Report from UPF triggered by DL packet in case of PDU Session is in 5GCM-idle state. |
| NF Context information |  | NF context information related to UE/Session context. |
| > Usage information of UE IP address resources | SMF | Usage information of UE IP address resources (dynamic and static, V4, V6, etc.) for CP or UP allocation, such as number, usage, number of UE IPs, which prohibit allocation during certain time interval, etc. |
| > State transition information | SMF | PDU Session related state transition information such as transition type, frequency SM state changes, etc. State transition identifier: - "PDU Session Establishment"; - "PDU Session Release"; - "Communication failure"; or - "PLMN change". |
| > timer information | SMF | PDU Session related timer information such as timer type, duration, etc. an individual and/or groups of NF’s information contains. |

Table 6.X-3: Input data from NRF/SCP

| Information | Source | Description |
| --- | --- | --- |
| NF ID | NRF | NF instance ID of the service producer or consumer. |
| Signalling exchange information | NRF | NF procedures containing signalling exchange information related between NF and NRF procedures. NRF/SCP can be used within deployment Model-C/D scope of indirect communication with/without delegated discovery. |
| > Type of requests from NF | NRF | Request type received such as NF registration and discovery, etc. |
| > Number of requests from NF | NRF | Number of NF requests received such as NF registration and discovery, etc. |
| > Time duration from receiving request from NRF to response to NF | NRF | Time duration between the request from NRF and response to NF. |

Editor's note:	It is FFS whether all parameters in the tables above are required or what additional parameters are necessary for producing the output analytics.

### 6.X.3	Output Analytics

The output analytics for signalling storm mitigation and prevention can include statistics information defined in Table 6.X.3-1 and/or prediction information defined in Table 6.X.3-2.

Table 6.X.3-1: Signalling Storm statistics

| Information | Description |
| --- | --- |
| Report (1..max) | List of observed signalling storm. |
| > Target NF ID | NF instance that suffered the signaling storm |
| > Abnormality ID | The potential cause of NF Abnormality (e.g. massive UE/frequent access, NF abnormal signalling, etc.). |
| > Source SUPI/NF list | SUPI(s) of the UE(s) or NF(s) which caused the signalling storm. |

Table 6.X.3-2: Signalling Storm predictions

| Information | Description |
| --- | --- |
| Report (1..max) | List of observed signalling storm. |
| > Target NF ID | NF instance that is predicted to suffer the signalling storm |
| > Abnormality ID | The potential cause of NF Abnormality (e.g. massive UE/frequent access, NF abnormal signalling, etc.). |
| > Source SUPI/NF list | SUPI(s) of the UE(s) or NF(s) which cause the signalling storm. |
| > Confidence | Confidence of this prediction. |

### 6.X.4	Procedures

Figure 6.X.4 shows the procedure for NWDAF-assisted signaling storm mitigation and prevention.

Figure 6.X.4-1: NWDAF-assisted signaling storm mitigation and prevention

1.	The consumer NF subscribes to or request the signalling storm analytics from the NWDAF using Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation respectively. The consumer can additionally include thresholds for confidence level and accuracy of detection in the subscripton or the request message. It may also include as analytic filters an indication of the use case for signalling storm (e.g. signalling storm due to UE mobility).

2.	The NWDAF retrieves required input data from NFs, using Nnf_EventExposure_Subscribe, such as signalling exchange information and context information as defined in the clause 6.X.2. The NWDAF can also collect additional Input data from MDAF/MDAS and/or AFs.

3.	The NWDAF derives the analytics.

4.	The NWDAF invokes Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response to provide the consumer NF with the analytics as defined in the clause 6.X.3.

5.	The consumer executes signaling storm mitigation and/or prevention operations based on the analytics received from the NWDAF.

# * * * * End of changes * * * *