SA WG2 Meeting #164	S2-2408672

# Maastricht, Netherlands, August 19 – 23, 2024	(was S2-240xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3815 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Store and Forward Satellite Operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There was a study in SA2 on integration of satellite components in the 5G architecture Phase 3 which included a KI (KI#2) on support of store and forward satellite operation. The conclusions for this KI in clause 8.2 of TR 23.700-29 support use of a split MME architecture with HSS on the ground and a full CN (EPC) onboard a satellite. A WI on integration of satellite components in the 5G architecture Phase III was agreed at SA#104 in SP-240628 “to produce normative specifications fulfilling the results of the feasibility study as documented in the conclusions section of TR 23.700-29 v 1.0.0 for each of the three key issues investigated”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add normative support for the normative conclusions of TR 23.700-29 and objectives of the WI in SP-240628 related to store and forward satellite operation. Add an informative description in an Annex of the solution using a full CN (EPC) onboard a satellite. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Store and forward satellite operation would not be supported according to the conclusions of TR 23.700-29 and objectives of the WI in SP-240628. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.x (new), Annex Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.13.x	Store and Forward Satellite operation

#### 4.13.x.1	General

Store and Forward (S&F) Satellite operation is illustrated in Figure 4.13.x.1-1 and enables UE access to PLMN services when a satellite has a service link to the UE but no feeder link to a serving ground PLMN. To enable S&F Satellite operation, MO data and control information received from a UE by a satellite with no feeder link (e.g. a satellite at Location L1 in Figure 4.13.x.1-1) is stored in the satellite and forwarded to the serving ground PLMN when the satellite later has a feeder link (e.g. at Location L2 in Figure 4.13.x.1-1). Likewise, MT data and control information for a UE received from the serving ground PLMN by a satellite with no service link to the UE (e.g. a satellite at Location L2 in Figure 4.13.x.1-1) is stored in the satellite and forwarded to the UE and/or used to support UE access, when the satellite later has a service link to the UE (e.g. at Location L1 in Figure 4.13.x.1-1).

Figure 4.13.x.1-1: Store and Forward (S&F) Satellite operation

Store and Forward Satellite operation is supported by two alternative architectures:

-	Model A: Split MME with HSS on the ground

-	Model B: Full EPC in each satellite

NOTE:	Support of Model A versus Model B is up to a network operator and can be transparent to a UE.

#### 4.13.x.2	Model A: Split MME with HSS on the ground

In Model A, each satellite operating in S&F mode supports one or more ground based PLMNs and includes an onboard eNB function and an onboard partial EPC function. The onboard partial EPC function includes an MME function but no HSS function. Each ground based PLMN includes an MME function, an HSS function and additional NFs needed for communication with external entities (e.g. AFs, other PLMNs, data networks). The following aspects are out of scope in this Release:

-	NF content of an onboard partial EPC and ground based serving PLMN additional to that stated in this clause; and

-	Signalling and procedures used inside a ground based serving PLMN and satellite and used between a ground based serving PLMN and satellite that are additional to that stated in this clause.

Annex Y.1 provides an informative description of the principles of Model A and the architecture, procedures and signalling for one example of Model A.

Interaction for S&F operation of a UE between a serving MME in the ground based serving PLMN and the HSS is as defined for normal NTN access with the following differences.

-	When the MME is notified by a satellite of a successful or unsuccessful Attach attempt by a UE that occurred for S&F operation at some previous “Request Time”, the MME may indicate to the HSS in an Update Location Request the "Request Time", to allow the HSS to check that no other (e.g. terrestrial) MME has sent an Update Location Request for the UE after the "Request Time".

-	When the MME is notified by a satellite of an unsuccessful initial Attach attempt by a UE for S&F operation, where the UE was requested by the satellite to reattempt the Attach at a later time, the MME fetches an authentication vector and other security details from the HSS. The MME may also trigger an Update location with the Update location ACK providing all the subscription details of the UE. The Update Location Request includes an indication that this location update is provisional i.e. the HSS must not consider the UE as registered until it receives a later final Update Location Request. The MME then provides the authentication vector and UE subscription information to one or more satellites to enable a second successful Attach attempt by the UE.

#### 4.13.x.3	Model B: Full EPC in each satellite

In Model B, each satellite operating in S&F mode supports one or more ground based PLMNs and includes an onboard eNB function and an onboard full EPC function. The onboard full EPC function includes an MME function, an HSS function and other NFs necessary to support access by a UE without use of any NFs on the ground. There may also be ground based serving PLMN elements.

The following aspects are out of scope in this Release:

-	NF content of an onboard full EPC function additional to that stated in this clause;

-	Functions and architecture of any ground based serving PLMN elements; and

-	Signalling and procedures inside a satellite and between a satellite and ground based serving PLMN elements.

Annex Y.2 provides an informative description of the principles of Model B and the architecture, procedures and signalling for one example of Model B.

#### 4.13.x.4	Common Procedures for Models A and B

The procedures and signalling used between a UE and satellite for Models A and B are the same as those used for normal satellite access with the differences defined in this clause. These differences are the same for Models A and B .

The onboard eNB in a satellite without a feeder link (e.g. at Location L1 in Figure 4.13.x.1-1) that is operating in S&F mode broadcasts an indication of S&F operation along with the IDs of supported PLMNs that are allowed to provide S&F service to UEs in the coverage area of the onboard eNB.

NOTE 1:	The onboard eNB should only broadcast the indication of S&F operation when the satellite has no feeder link and when at least one supported PLMN is allowed by regulation to provide S&F service to UEs in the coverage area of the onboard eNB. When the first condition occurs but not the second, operation of the onboard eNB should be deactivated.

A UE that is not able to access a PLMN for normal service and that supports S&F operation may access a satellite that broadcasts an indication of S&F operation.

When a UE first accesses a satellite for S&F operation and if the UE is not, and was not previously, attached for S&F operation, the UE may select a PLMN indicated as supported by the satellite, establish an AS signalling connection to the onboard eNB and send an Attach Request to an MME function onboard the satellite. If the UE was previously attached for S&F operation to a serving PLMN but was detached or performed a local detach, the UE should select the same PLMN and send an Attach Request to the MME function onboard the satellite in scenarios where the UE is not required (e.g. by regulation, subscription, service or operation requirements or by a user) to use a different PLMN. If the UE is already attached to a serving PLMN for S&F operation, but needs to perform a TAU, the UE sends a TAU Request to the onboard MME function in the satellite.

The UE includes in the Attach Request or TAU Request the IMSI and an indication that the UE supports S&F operation. The onboard MME function rejects any Attach Request or TAU Request that does not include the indication that the UE supports S&F operation. If the satellite (e.g. an onboard MME function for Model A or onboard HSS function for Model B) already contains subscription and security data for the UE, the onboard MME function may proceed with the Attach or TAU, authenticate the UE and establish a secure NAS connection and possibly a secure AS connection the same as for normal satellite access. The Attach Accept or TAU Accept returned to the UE may optionally include an S&F monitoring list of satellites IDs, a Detach indication and/or an estimated delivery time for MO data. When an S&F monitoring list is included, the UE accesses only the satellites in the S&F monitoring list for later MO/MT data/signalling. If the Detach indication is included, the UE locally detaches from the serving PLMN after the service link is no longer available if the UE is still attached. The UE should then continue to access the same serving PLMN for any new satellite access for S&F operation that may be needed for later MO/MT data/signalling and should first Attach to the new satellite.

If a satellite does not contain subscription and security data for a UE that has sent an Attach Request or TAU Request, the onboard MME function returns an Attach Reject or TAU Reject to the UE and includes an indication that the UE can retry the Attach or TAU at a later time, a Wait timer value and optionally an S&F monitoring list of satellites IDs. When the Wait timer has expired and if the UE has not successfully attached to another PLMN, the UE may again access a satellite for the selected PLMN and in the S&F monitoring list if this was provided and resend the previous Attach Request or TAU Request. If the satellite is now configured with subscription and security data for the UE, the satellite and UE can complete the Attach or TAU procedure and establish secure NAS and possibly AS connections. The Attach Accept or TAU Accept returned to the UE may optionally include a new Wait timer, a new S&F monitoring list of satellites IDs, a Detach indication and/or an estimated delivery time for MO data.

NOTE 2:	During the Wait timer received in an Attach Reject or TAU Reject, the UE can search for another PLMN to receive normal service.

NOTE 3;	After the first Attach Request or TAU Request is rejected and when the satellite later has a feeder link to a ground portion of the serving PLMN selected by the UE (e.g. at Location L2 in Figure 4.13.x.1-1), the satellite may indicate the Attach or TAU attempt and provide the IMSI to the ground portion of the serving PLMN. The ground portion of the serving PLMN may then configure the satellite and/or other satellites with subscription and security data for the UE. Further details of this are described for Models A and B in Annex Y.1 and Y.2.

NOTE 4:	How each S&F monitoring list of satellites IDs is determined is outside the scope of 3GPP in this release.

NOTE 5:	Usage by a UE of the estimated delivery time for MO data, if provided, and informing a user and applications of S&F operation are implementation dependent.

When a UE is attached, or was previously attached, to a serving PLMN for S&F operation, the UE may access any satellite supporting the serving PLMN that indicates S&F operation after the latest received Wait timer has expired and provided the satellite is in the latest S&F monitoring list provided to the UE for this serving PLMN. During a UE access to any new satellite, the MME function in the satellite may provide a new Wait timer, an indication of whether the Wait Timer is mandatory or advisory and/or a new S&F monitoring list of satellites IDs to the UE. When the Wait Timer indication is mandatory, a UE shall not access another satellite until the Wait Timer has expired. When the Wait Timer indication is advisory, a UE may access another satellite before the Wait Timer has expired.

NOTE 6:	The Wait timer and/or S&F monitoring list of satellites IDs can be used to help ensure that any new satellite accessed by the UE contains necessary security and subscription data for the UE to support UE access and that there is enough time for a new satellite to include any MT data responses to the UE sent by remote endpoints.

# * * * * Next change * * * *

Annex Y (informative):
Example Models of Store and Forward Satellite operation

# Y.1	Model A

# Y.2	Model B

## This clause describes an example of Model B for support of Store and Forward Satellite operation as defined in clause 4.13.x.

## Y.2.1	Architecture and Principles of Operation

An example architecture of Model B is shown in Figure Y.2.1-1. Each satellite contains the functionally of an eNB plus a full EPC that can include an MME, SGW, PGW, HSS, E-SMLC, SMSC etc. Each satellite further includes an endpoint proxy function that emulates the behaviour of a real endpoint (e.g. an AF) from the perspective of a UE. There is also a store and forward function (SFF) on the ground that may be part of, or connected to, an NTN Gateway and that contains a UE proxy function that emulates the behaviour of a real UE from the perspective of remote endpoints and any ground based serving PLMN.

Figure Y.2.1-1: Example Architecture of Model B

For Model B, the signalling and procedures used between a UE and satellite to support UE access and UE services in S&F operation are the same as used between a UE and serving PLMN for normal satellite access except for the differences described in clause 4.13.x.4. A UE thus sees the onboard eNB and EPC as being equivalent to a serving PLMN for normal operation. While some PLMN services may not be available for S&F operation due to subscription restriction for a UE or lack of support by the onboard eNB and/or EPC, the restriction of services does not lead to new impacts at a 3GPP level, though may have impacts outside the scope of 3GPP at an application or user level.

A UE accesses and attaches to a satellite for Model B as described in clause 4.13.x.4. The onboard EPC may obtain the UE location and verify that the UE is allowed to access the PLMN that was selected by the UE for the Attach. The UE may then, if supported by the onboard EPC, establish PDN connections to the onboard EPC and perform IMS Registration to an onboard satellite IMS if supported.

Depending on what is supported by the onboard eNB and EPC, the UE can perform mobile originated (MO) transactions such as sending SMS, sending data (e.g. using IP or non-IP protocols), establishing SIP sessions and sending SIP media (e.g. voice, video, text) or sending signalling (e.g. for HTTP queries). Each MO transaction is transferred by the onboard EPC to the onboard endpoint proxy which stores transaction data and signalling (e.g. SMS, data, voice, HTTP queries) and associated protocol and remote endpoint data. The onboard endpoint proxy also returns responses to the UE at a transport and application level that are necessary to allow correct transport and application protocol operation, avoid timeouts and enable a user (if participating) to be aware of the one way communication status. For example, in case of a user, a preconfigured SMS, voice or HTTP reply might be returned indicating when an SMS message, voice message or HTTP query sent by the user may reach the remote endpoint. For real time (e.g. IMS) services, service is restricted to one way transfer of messages (e.g. a voice message or a video clip). The recipient can then return a similar one way message in a reply.

Shortly before satellite coverage will be lost, the onboard EPC detaches the UE. If there is a radio link failure before this, the satellite and UE both perform a local detach. If the UE can access a new satellite before coverage is lost, a satellite could transfer UE status including data for ongoing transactions to the new satellite using ISL and the UE may then perform a handover.

After the UE loses coverage and when the satellite obtains a feeder link to a ground based portion of the serving PLMN selected by the UE for the Attach, the satellite (or the onboard endpoint proxy) transfers data for the UE (e.g. the IMSI and last known UE location) and all of the stored data and signalling for the UE MO transactions to an S&F function for the serving PLMN. The S&F function contains a UE proxy that stores the data and signalling for the UE MO transactions and then forwards the data and signalling for each MO transaction to the associated remote endpoint. The UE proxy can also receive and store voice, data and signalling for mobile terminated (MT) transactions that may be returned by the remote endpoints to the UE.

The S&F function is part of the serving PLMN selected by the UE and supports, or has access to, functions to support a UE Attach when the UE is roaming from its HPLMN and UE interaction with remote endpoints. Support of, or access to, other functions is possible according to three alternative variants, B1, B2 and B3, of Model B.

-	Variant B1: the S&F function connects to a second ground based PLMN at a Uu level by emulating the behaviour of an NTN Gateway with normal (non S&F) access to the UE via a satellite. Variant B1 is only applicable when the UE is roaming (i.e. the serving PLMN selected by the UE is not the HPLMN).

-	Variant B2: the S&F function connects to the EPC of a second ground based PLMN at an S1 level by emulating the behaviour of an eNB with normal (non S&F) access to the UE via a satellite. Variant B2 is only applicable when the UE is roaming (i.e. the serving PLMN selected by the UE is not the HPLMN).

-	Variant B3: the S&F function contains EPC functions or is otherwise a part of (and integrated with) with a ground based EPC for the serving PLMN. Variant B3 can support both roaming and non-roaming.

In all three variants, the UE is represented by the UE proxy in the S&F function which initiates any Attach needed to an HPLMN when the UE is roaming and sends MO transactions to, and receives MT transactions from, remote endpoints on behalf of the real UE.

For an initial Attach when the UE proxy in the S&F function first receives data for the UE, the UE proxy performs a NAS Attach on behalf of the UE with the HPLMN when the UE is roaming. The Attach occurs via the second PLMN for variants B1 and B2 or directly for variant B3. Subsequent to any Attach and in the case of roaming, the UE proxy may establish PDN connections and perform an IMS Registration if needed. For both roaming and non-roaming, the UE proxy initiates MO transactions corresponding to those initiated earlier by the UE which forward the MO data and signalling to the remote endpoints. The UE proxy in the S&F function also interacts with remote endpoints to receive and store data and signalling for MT transactions and, similar to the endpoint proxy in the satellite, can return responses to the remote endpoints at a transport and application level that are necessary to allow correct transport and application protocol operation, avoid timeous and enable a user at a remote endpoint to be aware of the one way communication status. For example, in the case of a user, a preconfigured SMS or voice reply might be returned indicating when an SMS message or voice message may reach the UE.

The S&F function and UE proxy simulate continuous reachability of the real UE and may remain permanently in a CM Connected state. The MT transaction data and signalling received and stored by the UE proxy are transferred to one or more satellites that are expected to, and allowed to (e.g. according to an S&F monitoring list sent earlier to the real UE), later provide coverage to the UE. When the UE accesses such a satellite, the endpoint proxy in the satellite can retrieve address information for the corresponding UE (e.g. PDN address/UE IP address, TLTRI) if the S&F function or second PLMN connects to remote endpoints via N6/SGi or N33/T8 interface and transfer the MT transaction data to the UE along with supporting any new MO transactions from the UE based on the received address information.

When the UE is detached after each satellite access, satellites carrying MT transaction data for the UE do not page the UE but instead rely on the UE instigating access. However, after initiating MO transactions, a UE or user can be advised when a satellite with MT transaction data in reply will become available and/or can be provided with coverage data for satellites to facilitate access.

For MT transactions, the UE could simply attach to every allowed visible satellite and then wait for possible MO transactions from the user or applications on the UE or for MT transactions from the satellite, but this could be highly inefficient. A smarter UE could first wait for an indication from the user or from an application on the UE of a pending MO transaction or could wait based on knowledge of when MT transactions may arrive e.g. according to an S&F monitoring list and Wait timer, before performing an attach. This is implementation dependent.

## Y.2.2	Support of MO and MT Transactions

An MO or MT transaction for Model B can correspond to:

-	Transfer of an SMS message (e.g. using NAS or IMS).

-	Transfer of data to or from a remote endpoint using CP CIoT or UP.

-	IMS (e.g. SIP) session establishment to or from a remote endpoint, plus one way transfer of media (e.g. voice, video and/or text) followed by IMS session release.

-	An HTTP request and HTTP response where the response may be preconfigured.

Figure Y.2.2-1 shows how MO and MT transactions can be supported between a UE and remote endpoints in S&F operation.

Figure Y.2.2-1: Support of MO and MT Transactions for S&F Operation

1.	A satellite in S&F operation broadcasts an S&F indication and one or more supported PLMN IDs in a SIB (e.g. SIB1).

2.	At a time T1, a UE that is not attached for S&F operation and that supports S&F operation, discovers the satellite, selects a supported PLMN, obtains an RRC Connection from the onboard eNB and sends a NAS Attach Request. The UE includes in the Attach Request its IMSI and an indication of S&F capability. The onboard MME will reject an Attach if the S&F indication is not included. The rest of the Attach procedure occurs as for normal satellite access to a ground based PLMN, with the differences described in clause 4.13.x.4, with the onboard eNB and EPC replacing the eNB and EPC in the ground based PLMN. The Attach Accept returned to the UE by the onboard EPC may include an S&F monitoring list of satellites IDs, a Detach indication and/or an estimated delivery time for MO data. If the UE needs to transfer data over UP or transfer SIP media data, the UE establishes PDN connection(s) and, for SIP media, performs an IMS Registration. All UE interactions are with the eNB, EPC and Endpoint proxy onboard the satellite and do no involve any ground based entities. Support of security for the Attach and any IMS Registration is described in clause Y.2.3.

3.	The UE instigates one or more MO transactions to the satellite on board EPC. Existing 3GPP procedures applicable to normal satellite access are used without change. The on board EPC forwards MO transaction data and protocol and signalling information applicable to the remote endpoint(s) to the endpoint proxy which stores this and returns any responses to the UE necessary to ensure correct operation of transport and application protocols. The endpoint proxy may also return preconfigured application level messages to a user of the UE to clarify S&F operation. For example, a preconfigured voice message may indicate an expected delay in forwarding MO transactions to the remote endpoint(s) and may further indicate a later time when MT transaction responses may be received back at the UE from the remote endpoint(s). The endpoint proxy may also request and obtain the location of the UE (e.g. location at time T1) and may verify that the UE is allowed to access the selected PLMN at this location. The endpoint proxy stores the MO transaction data from the UE.

4.	Shortly before the UE will lose satellite coverage, the onboard EPC performs a NAS Detach of the UE. If satellite coverage is lost before this can occur, the UE (if the Detach indication was received at step 3) and the onboard EPC each perform a local Detach or local De-registration. If ISL is supported and another satellite becomes available to the UE before coverage is lost, the satellite can transfer UE state and ongoing transaction information to the new satellite using ISL and instigate UE handover. Preventing a UE attempting handover or cell change to a new satellite when ISL is not supported can be supported without new impact by broadcasting a different TA from each satellite and prohibiting UE access to other TAs.

5.	At a later time T2 when the satellite has feeder link access to a ground based S&F function for the PLMN selected at step 2, the endpoint proxy transfers UE status (e.g. IMSI, location of the UE at time T1 if known and GPSI of the UE if the S&F function connects to remote endpoints via N6/SGi or N33/T8 interface) and MO transaction data to a UE proxy in the S&F function. The transfer procedure is not defined by 3GPP. If there were no MO transactions at step 3, an indication of UE presence only is transferred.

6.	Step 6 is omitted for Variant B3 when the selected PLMN in step 2 is, or will act as, the HPLMN. Otherwise, the S&F function and UE proxy simulate normal (non S&F) satellite access by the UE at its location at time T1. For variant B1 or B2, the UE proxy sends a NAS Attach Request for the UE to the MME in the second PLMN at a Uu level for variant B1 or S1 level for variant B2. The rest of the Attach procedure occurs as for normal satellite access by a UE to the second PLMN. For variant B1, the UE proxy can send the UE location at time T1 to the second PLMN when requested. For variant B2, if the PLMN has defined a TA and fixed cell that covers the location of the UE at time T1, the S&F function and UE proxy can indicate UE presence in this TA and fixed cell over the S1 interface to the EPC in the second PLMN. For variant B3, the S&F function, or a ground based EPC for the selected PLMN to which the S&F function is connected, performs Attach related procedures with the UE HPLMN the same as for a normal UE Attach.

For variants B1 and B2, if the UE proxy needs to transfer data over UP or transfer SIP media data, the UE proxy establishes PDN connection(s) and, for SIP media, performs an IMS Registration. For variant B3, PDN connections may not be needed (depending on serving ground PLMN architecture) but IMS Registration with an HPLMN would be needed to transfer SIP media for a roaming UE.

Support of security for an Attach or IMS Registration is described in clause Y.2.3..

7.	The UE proxy instigates one or more MO transactions to transfer any data and SIP media for the MO transactions at step 3 to the remote endpoint(s). Existing 3GPP procedures are used without change. The UE proxy may also send preconfigured application level messages to a user of a remote endpoint to clarify S&F operation. For example, in the case of a SIP voice media message, the UE proxy could indicate to the user that it is about to play a voice message from the UE, may indicate the time and possibly location at which the message was originally sent and may further prompt the user at the remote endpoint to replay a voice message or return a voice message reply.

NOTE:	There can be differences between the procedures used at steps 2 and 3 by the real UE and the procedures used at steps 6 and 7 by the UE proxy as long as the same services are performed at an application level.  For example, the real UE could use CIoT to transfer data at step 3 whereas the UE proxy could use UP – or vice versa.

8.	Remote endpoint(s) (e.g. a user at a remote endpoint) may instigate one or more MT transactions to return responses to the UE proxy for the MO transaction(s) received at step 7. The UE proxy stores the received data from the remote endpoint and associated protocol and signalling information and returns any responses to the remote endpoint necessary to ensure correct operation of transport and application protocols. The UE proxy may also send preconfigured application level messages to a user at the remote endpoint to clarify S&F operation. For example, application level messages may indicate an expected delay in forwarding MT transactions to the real UE and may further indicate a later time when MO transaction responses from the real UE may be received back at the remote endpoint. The UE proxy may also assemble an internal MT transaction (intended for the real UE) that might be an SMS message or voice message that confirms delivery of the MO transactions at step 7 (e.g. indicates the times when MO transactions were delivered to remote endpoints at step 7).

9.	The S&F function periodically transfers MT transaction data obtained at step 8 to one or more satellites that are allowed to access the UE and will later provide coverage to the UE location for time T1. Each satellite passes the MT transaction data, for a UE identified by the GPSI if the S&F function connects to remote endpoints via SGi or T8 interface, to an Endpoint Proxy for the UE where it is stored. The MT transaction data may be retained at the S&F function until a satellite later indicates delivery of the MT transaction data to the UE. The S&F function can maintain the UE proxy in CM Connected state or, for variant B1 or B2, can allow the second PLMN to place the UE proxy in IDLE state. In the latter case, the S&F function simulates continuous reachability of the UE.

10.	If the S&F function connects to remote endpoints via the SGi or T8 interface, after received MT transaction data from the S&F function in step 9, the endpoint proxy sends the GPSI of the UE to the onboard EPC to subscribe to address information (e.g. PDN address/UE IP address for IP packet, TLTRI for unstructured data) or the reachability of the UE (e.g. UE has attached to the onboard EPC for SMS).

11. At a later time T3, a satellite that received MT transaction data for the UE from the SSFC and UE Proxy at step 9 is able to provide coverage at the UE location. The satellite broadcasts an S&F indication and supported PLMN IDs in a SIB as at step 1.

12.	The UE obtains an RRC Connection to the onboard eNB and sends a NAS Attach Request to the onboard EPC as at step 2. Other actions in step 2 are also performed.

13. The onboard EPC notifies the address information or the reachability of the UE to the endpoint proxy, e.g. if step 10 was performed.

14.	The endpoint proxy instigates one or more MT transactions to the UE to forward the MT transactions received at step 9 based on the received address information or the reachability of the UE in step 13 (e.g. sends MT data to the TLTRI of the onboard SCEF, replaces the target IP address of the MT transaction data to the PDN address/UE IP address and sends to the onboard PGW if the S&F function connects to remote endpoints via the SGi or T8 interface). Existing 3GPP procedures applicable to UE satellite access are used without change. The endpoint proxy may also provide preconfigured application level messages to a user of the UE to clarify S&F operation. For example, transfer of voice messages to the user may be preceded by a preconfigured voice message indicating the sender of each message and the time when each message was sent by the remote endpoint.

15.	The UE may instigate one or more MO transactions to the endpoint proxy as in step 3 to reply to MT transactions received at step 14 or to initiate new transactions.

16.	Shortly before the UE will lose satellite coverage, and unless handover to a new satellite is possible using ISL, the satellite performs a NAS Detach of the UE as in step 4, or the UE and onboard EPC each perform a local Detach.

## Y.2.3	Support of Security

For UE access to a satellite with S&F operation, the IOPS solution defined in Informative Annex F of TS 33.401 [19] is used, where satellites replace the HSSs used for IOPS. This requires an IOPS capable USIM in a UE which can either be the normal USIM enhanced for IOPS or a dual USIM that is used by the UE only for S&F operation. With an IOPS solution, and for each supported UE, each satellite would be provisioned with the IMSI for the UE and a security key K* derived from the long term (master) key K in the USIM. The derivation of K* from K can be based on key derivation values n and m as described in Annex F of TS 33.401 [19]. The satellite does not perform the derivation of K* and is just provisioned with K* by the HPLMN and satellite operators along with the n and m values. The USIM performs the derivation after receiving (or inferring) the n and m values as part of authentication. The derived key K* is used for authentication and ciphering. Each satellite can have different n and m values and a different K* for the same UE or a group of satellites can share a common n value, common m value and common K* value for a UE. There are no new impacts to an ME (except for selecting a dual USIM for S&F access). The IOPS solution avoids exposure of the long term (master) key K to satellites (and the S&F function) and is thus more secure.

When the initial satellite accessed by a UE for S&F operation first accesses the ground based S&F function, the satellite (or endpoint proxy) transfers the authenticated IMSI for the UE to the S&F function. The S&F function then maps the IMSI to a second IMSI for the UE that is provisioned in the S&F function by the HPLMN and satellite operators. The second IMSI has a second long term security key K2 (different to the key K) that is also provisioned in the S&F function. The S&F function uses the second IMSI and the second key K2 to authenticate the second IMSI to the HPLMN, where the second IMSI and second key K2 are also configured in the HPLMN HSS. The second IMSI is also associated with existing public identifiers (e.g. MSISDN or GPSI) for the UE to allow MT service data from remote endpoints to reach the UE without any new impact to the remote endpoints. The capability to share the same public user identities among different private identities for a UE is already supported by some PLMNs in the form of a second IMSI for an additional user device such as a smart watch that is associated with a primary user device (e.g. a smart phone). The capability can include support for simultaneous coexistence of an attach by both devices. HPLMN specific logic can then be used to direct MT services to one or both of the devices.

The K2 values for different UEs (who have the same HPLMN) may be different or may be the same because authentication of the UE proxy is actually an authentication of the S&F function for either case and not the individual UE which is only authenticated by the satellite. The usage of a second IMSI allows a different security key K2 in the S&F function to the security key K in the USIM.

When a dual USIM in a UE (not the normal USIM) is used, and is used only for S&F operation with a different IMSI than that in the primary USIM, the mapping by the S&F function of the IMSI to a second IMSI is not needed and the IMSI from the dual USIM can be used to authenticate the UE proxy with the HPLMN. Even though the same IMSI would then be authenticated by both a satellite and HPLMN, the two keys (K and K2) that are used for the two authentications can still be different.

## Y.2.4	Transition between S&F and Normal Operation

Orbiting LEO and MEO satellites can support S&F operation along orbital segments where no feeder link is available and normal operation along segments where a feeder link is available. UEs which only see satellites on the S&F segments or only on the normal segments would not need to deal with transitions of operation. However other UEs may as described here.

When a satellite supporting normal operation loses access to a feeder link, the satellite would cease support for normal operation and may switch to S&F operation. UEs which accessed the satellite previously may be able to handover to another satellite with a feeder link if one is available, but will otherwise experience a coverage gap. Some of these UEs may then commence access for S&F operation as described in clause Y.2.2.

When a satellite supporting S&F operation obtains a feeder link, the satellite may cease support for S&F operation and switch back to normal operation. UEs in S&F operation can be detached prior to cessation of S&F operation as described in clause Y.2.2. Satellite coverage data provided to a UE (if supported) can indicate the durations of satellite coverage for S&F operation and, separately, the durations of coverage for normal operation. UEs can then know ahead of time when coverage for S&F operation will cease. When a coverage gap for S&F operation is about to occur, a UE can detach from the satellite.

A UE that switches between S&F and normal operation may experience problems in interacting with remote endpoints as data and voice sent in normal operation could arrive (at the UE or at a remote endpoint) before data and voice sent earlier for S&F operation. MT data sent to a UE that was previously attached for normal operation will also be buffered in a serving PLMN, while the UE is in a coverage gap for normal operation, which could lead to an opposite effect where MT data sent later to the UE when in S&F operation arrives first. In addition, a new Attach by a UE in normal operation would normally pre-empt and release a pre-existing Attach by the UE for S&F operation or the reverse. This is not a problem when a UE transitions from normal operation to S&F operation as the Attach for normal operation is then no longer needed. But for a transition from S&F operation back to normal operation, the Attach for normal operation could be released if the S&F function instigates an Attach for S&F operation after the Attach for normal operation has occurred. Alternatively, the Attach for S&F operation could be released if the Attach for S&F operation occurs before the Attach for normal operation has occurred, which might prevent ongoing MO and MT transactions for S&F operation from being completed.

The security solution described in clause Y.2.3 can avoid conflicts between S&F and normal operation for any UE due to use of a different IMSI for the UE for each of these, which allows an Attach for S&F operation to coexist with an Attach for normal operation without either of these affecting the other. If the UE (e.g. a user or application) also consistently uses only S&F operation or only normal operation for communication with each remote endpoint, mis-ordering of MO and MT data and voice arrival can also be avoided. However, mis-ordering might be tolerated by users if aware of the dual usage of S&F operation and normal operation.

## Y.2.5	Support of Discontinuous Satellite Coverage

With discontinuous satellite coverage, a UE may not be able to send and receive much MO and MT transaction data in S&F operation before coverage is lost and may be limited to only certain times of sending and receiving. For example with just one minute of satellite coverage, MO transactions initiated by a user and MT transactions sent to a user would be very limited (e.g. to voice messages of less than 1 minute in duration).

To overcome such limitations on not being able to send and receive much MO and MT data, a UE (e.g. an Application in the UE) can package and assemble a generic data message containing data supplied by a user or other applications for multiple MO transactions. The generic data message can also contain information for the intended remote endpoints. This is illustrated in Figure Y.2.5-1.

Figure Y.2.5-1: Packaging of MO or MT Transactions into a Single Packaged Transaction

The generic data message includes information about the constituent MO transactions that it contains (e.g. might include information on data and voice encoding) and enables the original MO transactions to be re-obtained.

The generic data message is sent to a satellite in a single MO transaction and later transferred to the UE proxy in the S&F function where it is unpackaged into the original data for the different MO transactions. The original data is then forwarded as normal to the remote endpoints. Similar packaging and unpackaging can occur for MT transactions where data for multiple MT transactions and information for the associated remote endpoints is packaged by the S&F function (or UE proxy) into a single MT transaction which is transferred to a satellite, later transferred to the UE as a single MT transaction and finally unpackaged by the UE (e.g. by an Application in the UE) into the original MT transactions which are made available (e.g. at a later time) to the user or applications in the UE.

The transfer of a single packaged MO transaction from the UE to a satellite or a single packaged MT transaction from the satellite to the UE can occur using maximum available bandwidth and can thus be less limited by limited coverage time at the UE and availability of the user.

To overcome limitation to only certain times of sending and receiving and assist with UE power saving, the UE may also include a preferred satellite monitoring mode (e.g. single satellite, a subset of satellites, all satellites) in a NAS Attach Request for S&F operation. If the onboard MME accepts the Attach, the onboard MME may expose the preferred satellite monitoring mode and the UE location to the endpoint proxy, then the endpoint proxy may determine an S&F monitoring list based on the information provided by the onboard MME, the communication pattern of the UE, the mobility trajectory of the UE, location of the S&F function and satellite ephemeris, and provides the S&F monitoring list to the onboard MME, which then provides the S&F monitoring list to the UE in an Attach Accept.

After Detach, the UE keeps the S&F monitoring list. The S&F monitoring list is also transferred to the S&F function as part of the UE status and the S&F function may select target satellite(s) for future UE access based on the S&F monitoring list.

# * * * * End of changes * * * *