3GPP SA WG2 Meeting #164	 S2-2408683

# Maastricht, NL, 19-23 August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.032 | CR | 0029 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Relative Velocity Clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per TS 23.032, the relative velocity with uncertainty of a device B relative to a device A is characterised by a radial velocity component and a perpendicular transverse velocity component. The radial velocity component has described only the rate of the range since it has a fixed direction from device B towards device A; the perpendicular transverse velocity component has only the rate of direction change and should have speed information included. And the velocity component should have direction instead of change rate of direction. Furthermore, the change of angle description is not consistent with two perpendicular velocity components. Since there is no initial direction, the original literal text describing the change of direction/angle can not deduce the direction of the perpendicular velocity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Merge the text of clause 4.4.2 Rang and Direction in TS 23.586 into clause 5.10 and keep a reference in the CR of 23.586 - Updates clause 8.4a with both direction and speed of radial velocity component and a perpendicular transverse velocity component. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The speed of the perpendicular transverse velocity component is missing and the concept is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10, 8.4a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>First Change<<<<

## 5.10	Range and Direction

The "range and direction" from a point A to a point B is characterised by three components comprising a range from point A to point B which refers to a straight line distance between point A and Point B, an azimuth direction from point A to point B and an elevation direction from point A to point B as shown in Figure 3d. The range provides a straight-line distance from point A to point B. The azimuth provides a direction to point B from point A in a horizontal plane through point A and as measured clockwise from North. The elevation provides a direction to point B from point A in a vertical plane through the points A and B and as measured upwards or downwards from a horizontal plane through point A. The range, azimuth and elevation can be each independently included or excluded in a "range and direction" and each has an uncertainty and a confidence.

Figure 3d: Description of a Range and Direction

Editor's note:	Support of this shape will be confirmed by RAN WGs.

# >>>>Second Change<<<<

## 8.4a	Relative Velocity with Uncertainty

The relative velocity with uncertainty of a device B relative to a device A is characterised by a radial velocity component and a perpendicular transverse velocity component. The radial velocity component is characterized by the direction from device B towards device A and a rate of change of a range between the device A and device B. The transverse velocity component is characterized by a speed along the perpendicular direction to radial velocity component, where the perpendicular transverse direction includes an angle of azimuth measured clockwise ( as positive value) or counter-clockwise (as negative value) from North in a horizontal plane through the device A and an angle of elevation measured upwards (as positive value) or downwards (as negative value) in a vertical plane through the devices A and B from a horizontal plane through the device A. Radial velocity component and a perpendicular transverse velocity component can be each independently included or excluded in the relative velocity. The speed and directions (e.g., angles of azimuth and elevation) can be each independently included or excluded in the velocity component and each has an uncertainty and a confidence,

Figure 12a: Description of a Relative Velocity with Uncertainty

# >>>>END OF CHANGES<<<<