SA WG2 Meeting #164 	S2-2408715

# 19 - 23 August 2024, Maastricht, Netherlands

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1165 | rev | -1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Public UE IP address exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Nokia, Ericsson, ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As concluded in TR 23.700-63, the UPF needs to be enhanced to support exposure of NATed UE public IP address and Port for a particular PDU Session |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new UPF event exposure to exposure the NATed UE public IP address and Port for a particular PDU Session |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The feature is not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.8.2.4, 6.2.8.2.4.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * *

6.2.8.2.4	Correlation between UE data collection and the NWDAF data request

6.2.8.2.4.1	General

The UE IP address is used to identify the user plane connection established between the UE application and the AF for data collection, while the AF receives the Naf_EventExposure_Subscribe to request for the specific UE data collection by using SUPI (for AF in trusted domain) or external UE ID (i.e. GPSI) (for AF in untrusted domain). AF is required to correlate the UE IP address to the SUPI or to GPSI.

If the AF supports requests addressed to External Group ID (for AF in untrusted domain) or Internal Group ID (for AF in internal trust domain), the AF must correlate the list of external UE ID (i.e. GPSI) or SUPI, respectively, with the group(s) the UE belongs to, so that the AF can further correlate the UE ID (external or internal) to the UE IP address.

AF may indicate in NF profile and register to NRF in clause 6.2.8.2.2 if it supports to do the mapping itself or ask NWDAF to do it. If the AF is in a trusted domain, it may also indicate the supported list of S-NSSAI, DNN combinations to NRF in NF profile.

NOTE 1:	If any method in Annex A is used, the AF always indicates in the NF profile that it supports mapping by itself.

Accordingly, if AF supports the mapping, for AF in trusted domain, it is required to correlate the UE IP address and SUPI as described in clause 6.2.8.2.4.2 or by other means, e.g. as described in Annex A. This, after receiving the data collection request from NWDAF and there is no mapping information storage in the AF. For AF in untrusted domain, the procedure to correlate the UE IP address and GPSI is described in clause 6.2.8.2.4.4 or by some other means, e.g. as described in Annex A. If there is NAT between the UE and the AF, one of the methods in Annex A or method in clause 6.2.8.2.4.x can be used.

If there is no NAT between the UE and the AF, NWDAF may collect the mapping information as described in clause 6.2.8.2.4.4 before sending request to AF in step 3a or step 3b in Figure 6.2.8.2.3-1.

If the user plane session between the UE and the AF is released, the AF / NWDAF removes the stored correlation information between UE IP address and UE SUPI / GPSI.

For all procedures defined in this clause 6.2.8.2.4.1, a specific combination of S-NSSAI/DNN shall be corresponding to a single PDU session for a UE to access the AF (either in trusted domain or untrusted domain).

NOTE 2:	Based on implementation, for the UE to access the Data Collection AF, only a single PDU Session is allowed to be established to the Data Collection AF, by configuring a specific S-NSSAI/DNN for the Data Collection AF only.

* * * Next Change * * *

6.2.8.2.4.x	AF correlates UE data collection and NWDAF request when there is NAT between UE and AF

This is only valid if there is NAT between the UE and the AF, and the AF has retrieved the Private UE address (UE IP address assigned by 5GC for the PDU session) either from the SMF, or provided from the NWDAF in order to request data collection from the UE Application.

.

Figure 6.2.8.2.4.x-1: AF correlates UE data collection and NWDAF request when there is NAT between UE and AF

- The AF retrieves the Private UE IP address from 5GC (i.e., SMF or NWDAF) as described in step 0-4 of Figured 6.2.8.2.4.2-1 or steps 0-6 of Figure 6.2.8.2.4.3-1, or steps 1-5 of Figure 6.2.8.2.4.4-1. The AF also stores the IP address of the remote end IP domain, DNN and S-NSSAI associated with the AF ID

For untrusted AF, the following steps 1-7 apply. For trusted AF, the following steps 3-6 apply and NEF is replaced by the AF

1.	The AF requests NEF to provide the public UE IP address towards the AF using Nnef_UEAddress_Get_Request.

2.	The NEF authorizes the AF request. If the authorisation is not granted, the NEF replies to the AF with a Result value indicating authorisation failure; otherwise the NEF proceeds with the following steps.

3.	The NEF sends the Nnrf_NFDiscovery service request to obtain the address of the UPF implementing NAT functionality for the Private UE address. The request includes the Private UE address. The NEF may also include the DNN and S-NSSAI associated with the AF ID, as well as the IP domain.

4.	The NRF responds with a Nnrf_NFDiscovery service response including the UPF address of the UPF implementing NAT functionality for the Private UE address.

5.	The NEF uses the Nupf_EventExposure_Subscribe service operation to request Public UE address and port information from the UPF. The request includes the Private UE address and an IP address of the remote end IP domain.

6.	The UPF sends a Public UE address and source TCP/UDP port by invoking the Nupf_EventExposure_Notify to the NEF.

7.	The NEF provides the Public UE address and source TCP/UDP port provided by UPF in step 6 to the AF

8.	The AF correlates the UE data by using the retrieved UE public IP address and port information.

* * * End of Change * * *