3GPP TSG-WG SA2 Meeting #164	S2-2408946

# Maastricht, NL, Aug 19 – 23, 2024		(revision of S2-2408544)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0131 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UAV flight planning and monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As concluded in KI#1 of TR 23.700-59 (FS_UAS_Ph3), the USS/UTM can request QoS Sustainability Analytics by containing the UAV flight path information. The information can be used for Pre-mission Flight Planning and In-mission Flight Monitoring. The CR adds this new feaure in TS 23.256. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new feature, named “Flight planning and monitoring for UAVs” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR conclusion for Pre-mission Flight Planning and In-mission Flight Monitoring for UAVs is not specified in TS 23.256 yet. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x(new),5.x.1(new), 5.x.2(new), 5.x.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.x	Pre-mission Flight Planning and In-mission Flight Monitoring for UAVs

5.x.1	General

5G network may support pre-mission flight planning and in-mission flight monitoring for UAVs via the NEF service exposure. The USS/UTM may request assiatance information to NEF of pre-mission flight planning and in-mission flight monitoring for UAV, after the UAV establishes user plane connection with the USS/UTM.

In case of pre-mission flight planning, USS/UTM may request assiatance information to NEF for the following cases:

- Determine the most suitable flight path among several candidate flight paths

USS/UTM provides multiple planned flight paths to the network, and network leverages the network analytics e.g. Movement Behaviour Analytics and/or QoS Sustainability Analytics, to determine the most suitable planned flight path among the candidates.

- Plan a flight path based on starting and ending point of the flight

USS/UTM provides location information of the starting and ending points for the flight to the network, and network leverages the network analytics e.g. Movement Behaviour Analytics and/or QoS Sustainability Analytics, to determine a detailed flight path. NEF responds the USS/UTM about the detailed flight path to assist the pre-mission flight planning.

(3) Output QoS information along the flight path

USS/UTM provides multiple planned flight paths to the network, and network leverages QoS Sustainability Analyticsto output QoS information for each flight path provided.

Editor’s NOTE: what parameter e.g. polygon, is provided by the USS/UTM, to identify the UAV flight path , and how the network maps the parameter to the 3GPP defined area are FFS

In case of in-mission flight monitoring, in addition to QoS Sustainability Analytics request, USS/UTM can provide the UAV flight path and correspongding (arriving) time to the network, in order for the network to verify whether the UAV’s exact location matches the scheduled location represented by the waypoint.

5.x.2	Procedure for NEF Assist Pre-mission Flight Planning

Figure 5.x.2-1: Procedure for NEF Assist Pre-mission Flight Planning

1.	The UAV establishes a PDU Session for communication with the USS/UTM as described in clause 5.2.3.

2.	(via applicaton layer) The UAV requests Pre-mission flight planning service from USS/UTM.

3.	The USS/UTM derives information for the Pre-mission flight planning request and decides to request assistance information from NEF.

NOTE 1:	The content of Pre-mission flight planning service information derived at USS is out of scope.

4.	The USS/UTM sends Pre-mission flight planning assist request to NEF. The request message includes identifier of the UAV (e.g. GPSI), information of the starting and ending points for the flight, requirements on the flight route (e.g. on time), candidate flight route(s) (received from the UAV or local derived at the USS/UTM), and accuracy level of predictions relevant to the flight planning.

5.	The NEF maps parameters included in the request from the USS/UTM to information used by the 3GPP system (e.g. map the geographical area into an area of interest that is represented by a list of Cell IDs, gNB IDs or TAIs). The NEF determines services needed for the request and relevant NFs, e.g. NEF service on UAV tracking and mode (UAV location reporting mode, UAV presence monitoring mode, List of Aerial UEs in a geographic area), NWDAF analytics service (Movement Behaviour analytics, and/or QoS Sustainability Analytics), GMLC service (e.g. Ranging/Sidelink Positioning location).

6.	If the NEF cannot satisfy the requirements on flight planning (e.g. cannot get predictions relevant to the flight planning from NWDAF with the required accuracy level), the NEF responses to the USS/UTM to reject the Pre-mission flight planning assist request, and may include the detail reason.

7.	If being rejected in step 6, the USS/UTM may response to the UAV to reject the Pre-mission flight planning request, may include the detail reason if available.

8.	The NEF performs steps 2-5 in clause 5.3.4, and filter list of UAVs in the areas of interest.

9.	The NEF subscribes/requests for notification on Movement Behaviour analytics provided by NWDAF as defined in clause 6.21.4 of TS 23.288 [X]. The subscribe request message include identifier of the UAV (e.g. GPSI) obtained in step 4 and height information of UAV. The other parameters included in the request are described in clause 6.21.4 of TS 23.288 [X]. The list of UAVs in the areas of interest output from step 8 can be used as inputs for the NEF to request the NWDAF on Movement Behaviour analytics.

10.	The NEF subscribes/requests for notification on QoS Sustainability analytics provided by NWDAF as defined in clause 6.9.1 of TS 23.288 [X]. The subscribe/request message include identifier of the UAV (e.g. GPSI) obtained in step 4 and the UAV flight path information defined as 3D location waypoints. The other parameters included in the request are described in clause 6.9.1 of TS 23.288 [X]. The list of UAVs in the areas of interest output from step 8 can be used as inputs for the NEF to request the NWDAF on QoS Sustainability analytics.

11.	The NEF may request GMLC service(s), e.g. Ranging/Sidelink positioning location as defined in clause 6.20.3 of TS 23.273 [8]. The list of UAVs in the areas of interest output from step 8 can be used as inputs for the NEF to request the GLMC service on Ranging/Sidelink Positioning location.

12.	The NEF generates assistance information for pre-mission flight planning based on the information and analytics from steps 8-10. The assistance information may be the best matched route among the ones provided from the USS/UTM in step 4, or potential flight route(s) if candidate flight route(s) is not provided in step 4.

13.	The NEF responses to the USS/UTM with the assistance information.

14.	The USS/UTM decides flight planning by using the assistance information and flight planning mechanism which is out of scope.

15.	The USS/UTM sends response to the UAV with the planned flight route.

5.x.3	Procedure for NEF Assist In-mission Flight Monitoring

Figure 5.x.3-1: Procedure for NEF Assist In-mission Flight Monitoring

NOTE 1:	The NFs in Figure 5.x.3-1 represents all the other NFs beside GMLC and NEF/UAS NF involved in the procedure.

1.	The USS/UTM sends In-mission flight monitoring assist subscribe message to NEF. The message includes identifier of the UAV (e.g. GPSI) and information of the flight route, monitoring mode (e.g. UAV location monitoring, fight route monitoring, flight environment monitoring, Ranging/Sidelink positioning location monitoring, QoS Sustainability analytics), report requirements (e.g. report format (event triggered or periodically), assistance information (e.g. the monitoring results, information generated from the monitoring results)), and may include planned fight route if fight route monitoring, list of UAVs in the areas of interest if flight environment monitoring, preset value for distance if Ranging/Sidelink positioning location monitoring.

2.	NEF maps the parameters in the message from the USS/UTM to information used by the 3GPP system. The NEF determines services needed for the message and relevant NFs, e.g. NEF service on UAV tracking (UAV location reporting mode, UAV presence monitoring mode, List of Aerial UEs in a geographic area), GMLC service (Ranging/Sidelink Positioning location), NWDAF analytics service (QoS Sustainability Analytics).

3.	If the NEF cannot provide the requested assistance information for In-mission flight monitoring, the NEF responses to the USS/UTM to reject the In-mission flight monitoring assistance subscribe message, may include the detail reason if available.

4.	For UAV location monitoring, the NEF executes steps 2-5 in clause 5.3.2 to obtain the location of the target UAV.

5.	For fight route monitoring, the NEF executes steps 2-4 in clause 5.3.3. Upon to the subscribe message in step 1 for reporting, the NEF may compare the monitored flight route with the planned fight route (if provided in step 1) and generate assistance information on whether the UAV is flight in right route.

6.	For flight environment monitoring, the NEF executes steps 2-5 in clause 5.3.4 to obtain the information of aerial UEs in the same geographic area of the target UAV.

7.	For QoS Sustainability analytics, the NEF requests NWDAF service for QoS results as described in clause 6.9.1 of TS 23.288 [X]. The subscribe/request message include identifier of the UAV (e.g. GPSI) obtained in step 4 and the UAV flight path information defined as 3D location waypoints. The other parameters included in the request are described in clause 6.9.1 of TS 23.288 [X]. The list of UAVs in the areas of interest output from step 8 can be used as inputs for the NEF to request the NWDAF on QoS Sustainability analytics.

8.	For Ranging/Sidelink positioning location monitoring, the NEF requests GLMC service for Ranging/Sidelink Positioning location results as described in clause 6.20.4 of TS 23.273 [8], the procedure of SL-MT-LR for periodic, triggered Location Events (steps 1-20 for initiation the monitoring, steps 21-31 for monitoring periodic). The list of UAVs in the areas of interest output from step 6 can be used as inputs to step 7 for NEF to request GLMC service for Ranging/Sidelink Positioning location, if the UAVs is unknown in the areas of interest.

9.	The NEF notifies the USS/UTM the monitoring results (e.g. the location of the target UAV, indication on whether the UAV is flight in right route, the target UAV presence in areas of interest, the information of aerial UEs in the same geographic area of the target UAV, the distance of other UAVs and the target UAV and whether the distance is smaller than a preset value).

Steps 4-9 may be repeated for report monitoring results periodically.

# * * * * End of changes * * * *