3GPP TSG-SA2 Meeting #164	S2-2409128

# Maastricht, Netherlands, 19 - 23 August 2024	       	 (revision of S2-2408806)

Title:	[draft] LS on XRM metadata exchange between 3GPP Core and an Application server

Response to:

Release:	Rel-19

Work Item:	FS_XRM_Ph2

Source:	Nokia, Will be SA2

To:	IETF Multiplexed Application Substrate over QUIC Encryption WG chairs: Dennis Jackson, Eric Kinnear.

Cc:	liaison-coordination@iab.org, 3gpp-liaison@ietf.org.

Contact Person:

#### Name:	 Laurent Thiebaut

E-mail Address:	 Laurent.thiebaut@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall Description:

Background information

3GPP SA2 is working to provide dedicated Quality of Service (QoS) for XRM (eXtended Reality) traffic flows over 3GPP networks (5G); for this purpose, a 3GPP network needs inband (along with the packets) certain metadata about the structure and characteristics of the application media content in the traffic flows.

3GPP SA2 is considering solutions where an Application Server (AS) collaborates with 5G network by providing XRM metadata to help improving the QoS handling in the Radio Access Network (RAN).

In one of those solutions the 3GPP User Plane Function (UPF) is assumed to detect an e2e QUIC connection and to establish a UDP tunnel (per per RFC 9298 ) to a proxy that may be co-located with the target AS. The tunnel is used to send XRM related packets as well as related XRM metadata. Note that in this solution, the connect-UDP client (UPF) is different from the client of the e2e connection (the UE) .

In the case of media packets being transported over QUIC, the forwarded mode of the “QUIC-Aware Proxying Using HTTP” is proposed, to optimize the use of node and network resources. In this case, XRM metadata is encoded into forwarded packets using a packet transform to be defined by 3GPP.

The draft-ietf-masque-quic-proxy Client would be in the UPF and thus not in the same entity than the end QUIC client. UE and UPF would be connected by IP networking as specified by 3GPP with UE acting as an IP host and UPF as an IP Router. XRM Metadata about the end-to-end (UE to Target) QUIC connection would be carried between UPF and Proxy.

UE

UPF

For this kind of deployment following questions have been asked that require detailed expertise on draft-ietf-masque-quic-proxy:

The 3GPP SA2 working group would like the IETF to provide feedback on the aspects of the solution relating to CID detection and registration, given the somewhat different deployment model than what is assumed in “QUIC-Aware Proxying Using HTTP”. 3GPP SA2 working group would especially like IETF to confirm or further clarify the following points:

- Whether a UDP tunnel client at the UPF will be able to initiate forward mode based on the e2e QUIC connection IDs from the long header packets.

- QUIC allows new end-to-end CID values to be negotiated between the UE (QUIC client) and the Target (QUIC Server) anytime during the lifetime of the end-to-end QUIC connection. When this happens, the UPF would not be able to learn the actual new end-to-end CID values. But the UPF can detect new end-to-end CID values are used in QUIC packets it receives with short headers, even though UPF can only do this detection based on the length of the initial end-to-end CID having been used (as QUIC packets with short headers do not contain CID length).Once it has detected a new end-to-end CID is being used, the UPF acting as a QUIC-Aware proxying client, can negotiate the usage of a new virtual CID to be associated with the subset of the new end-to-end CID it has detected.

- In case that the length of the e2e QUIC connection IDs change, whether the UDP tunnel client at the UPF and the proxy at least will be able to communicate in tunneled mode.

- Actions:

To IETF Multiplexed Application Substrate over QUIC Encryption WG:

ACTION: 	SA2 kindly asks IETF Multiplexed Application Substrate over QUIC Encryption WG to answer to the questions above.

- Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#165 | October 14th – 18th, 2024 | Hyderabad | India |
| SA2#166 | November 18th –22nd, 2024 | Orlando | USA |

Please see https://portal.3gpp.org/Meetings?tbid=375&SubTB=385#/