3GPP TSG-SA2 Meeting #164	S2-2409417

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1126 | rev | 3 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General inference procedure for vertical federated learning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, ETRI, China Telecom, Tencent, ICS, ZTE, Samsung, Huawei, Nokia, KDDI, Ericsson, Lenovo, Interdigital, vivo, Futurewei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-07-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the conclusion captured in the TR 23.700-84 clause 8.2 P#2.4, this CR aims for specify the general inference procedure for vertical federated learning. The NWDAF contain AnLF is the consumer to request the VFL inference result to the VFL server. And the NWDAF contain AnLF based on the VFL inference result generate the analytics output and send the analytics output to the analytics consumer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 6.2X is added. The sub-clause 6.2X.1 and 6.2X.2 are added to specify the general inference procedure for vertical federated learning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can not perform VFL inference if do not specify the VFL inference procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X (new), 6.2X.1 (new). 6.2X.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change (all new)* * * *

6.2X	Vertical Federated Learning

6.2X.1	General

This clause specifies how NWDAF(s) and AF(s) can leverage Vertical Federated Learning technique to perform VFL inference.

6.2X.2	Procedures

6.2X.2.X	General inference procedure for vertical federated learning

Figure 6.2X.2.X-1: General inference procedure for vertical federated learning

Editor’s Note: Whether one general procedure or two separate procedures for NWDAF/AF as server will be discussed and decided in future meeting when the procedures are stable.

Editor's note:	In case any of the Consumer, Server and clients are untrusted AF(s), how the NEF assists the VFL inference process, and whether the existing or new NEF service should be invoked are FFS.

0.	For an NWDAF as a VFL server, the analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g. UE IDs, Analytics Reporting Information=Analytics target period)) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF can not generate the analytics output, the NWDAF containing AnLF sends a subscription request to VFL server NWDAF using Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe including Analytics ID, Target of Analytics Reporting = e.g. UE IDs.

Editor's note:	Which service used between the consumer and the VFL server is FFS.

2.	Based on the information received in the step 0 or 1, VFL server decides to iniate the VFL inference procedure with the VFL clients.

Editor's note:	How the VFL server determines the VFL clients and whether the VFL server may not select all VFL clients is FFS.

Editor's note:	The details on how the VFL server monitoring the accuracy of the VFL is FFS.

If the VFL server is the NWDAF and the VFL client is the NWDAF, VFL server sends a VFL Inference request/subscription to the VFL clients which includes an Analytics ID, the VFL model correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used.

If the VFL server is the NWDAF and the VFL client is the AF, VFL server NWDAF sends a VFL Inference request/subscription to the VFL clients which includes an Analytics ID, the VFL model correlation ID to indicate the VFL client which previously well-trained VFL local model associated with this ID will be used. If the AF is untrusted, the VFL server NWDAF sends the request to the NEF, and NEF forwards the request to the untrusted AF.

Editor's note:	Whether the VFL model correlation ID is needed or using the model ID is FFS.

Editor's note:	Which service is used between the VFL server and VFL client is FFS.

Editor's note:	Additional parameters in the request are FFS, e.g. UE ID(s) which will need to be sent to VFL clients.

3.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

Editor's note:	Whether the VFL client needs to retrieval of stored local models is FFS.

5.	Each VFL Client sends the intermediate local inference results to the VFL server.

Editor's note:	Whether the VFL model correlation is required in step 5 is FFS.

Editor's note:	Whether VFL client may also provide local intermediate inference results to other VFL client is FFS.

The intermediate local inference results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to aggregates and generates the VFL inference results.

6.	The VFL server may collects its local data and generate the intermediate local inference results. The VFL Server aggregates all the intermediate local inference results to generate the VFL inference results based on the VFL model correlation ID.

Editor's note:	Further details of the VFL server computiong the VFL inference is FFS.

7. 	If the NWDAF is the VFL server, the VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify to the consumer (i.e NWDAF containing AnLF) including the VFL inference results.

8. 	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

# * * * * End of changes * * * *