SA WG2 Meeting #S2#164	S2-2407423

19 - 23 August, Maastricht, Netherlands

3GPP TSG-CT WG1 Meeting #149	C1-243934

# Hyderabad, India, 27 – 31 May 2024

# Title:	Reply LS on LS reply on LCS user plane connection binding to the UE

# Response to:	LS reply on LCS user plane connection binding to the UE (C1-243022/S2-2405797)

# Release:	Rel-18

# Work Item:	5G_eLCS_Ph3

Source:	CT WG1

To:	SA WG2

Cc:	CT WG4, SA WG3

Contact Person:

Name:	Xiaoxue Zhao

Tel. Number:	-

E-mail Address:	zhaoxiaoxue@cictmobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	C1-243956, C1-243587

1. Overall Description:

CT1 thanks SA2 for the reply LS (C1-243022/S2-2405797) on LCS user plane connection binding to the UE. Based on SA2 agreed resolutions, CT1 has agreed attached CRs to TS 24.572 including the following adaptations to the order of messages specified in clause 6.18 of 3GPP TS 23.273:

- The UE can initiate a UE requested user plane connection establishment procedure to the LMF which triggers the LMF-initiated user plane connection establishment procedure;

- In both the UE-initiated and the LMF-initiated cases, the LMF performs the user plane connection establishment procedure to the UE with providing LCS-UP binding ID (called LCS UP connection ID in SA2) and the LMF LCS-UP address to the UE;

- The user plane connection establishment command is acknowledged over CP after the binding procedure is performed over UP (call flow see Annex of C1-243956 for information); and

In addition to the above, the LCS-UP binding ID is used to associate the UE with the user plane connection during the user plane connection establishment procedure. The LCS-UP binding ID will be released once used, in order to avoid security threats.

2. Actions:

To SA WG2 group.

ACTION: 	CT WG1 kindly asks SA WG2 to take the above information into account and update their specification if necessary.

3. Date of Next CT1 Meetings:

CT1#150                          19 - 23 August 2024		Maastricht, The Netherlands

CT1#151                           14 - 18 October 2024		China