SA WG2 Meeting #164	S2-2407464

Maastricht, Netherlands, August 19 – August 23, 2024	(Revision of S2-240xxxx)

Source: 	Samsung, SK Telecom

Title: 	KI#1: Conclusion update

Document for: 	Approval

Agenda Item: 	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution provides update of conclusion for key issue #1.

1	Discussion

This contribution provides the conclusions for KI#1.

2 	Proposal

It is proposed to capture the following contents into TR 23.700-66.

FIRST CHANGE

8.1	Conclusion for KI#1: Network energy related information exposure

The following bullets are concluded for normative work for KI#1:

1)	A new network functionality is defined to collect and calculate the energy related information and exposes to the authorized consumer subject to operator's policy:

a)		If the authorized consumer is AF, the granularities include: per application corresponding to the AF, per UE, per UE per AF, and per PDU Session.

b)		If the authorized consumer is 5GC NFs, the granularities include: per application, per UE, per-UE-per-QoS flow, and per PDU Session.

c)		The energy related information for the granularities above include:

i)		Energy consumption information;

ii)		Renewable energy information.

2)	The consumer NFs including 5GC NFs and /or AF, may request the energy consumption information exposure with reporting request e.g. Periodic reporting or Threshold based reporting.

3)	The information for the calculation of the Energy Consumption information is obtained from the following sources:

a)	OAM: provides the energy consumption information at the gNB(s);

b)	OAM: provides the overall data volume for the gNB;

c)	UPF: provides the overall data volume of the UPF, and UE data volume at the UPF;

d)	UPF: provides the data volume for the QoS flow or the SDF.

NOTE 1:	The source of the energy consumption information at the UPF(s) serving the UE, or Network Slice serving the UE will be decided during the normative phase.

4)	The new functionality determines the E2E energy consumption based on energy consumption per the granularities above at serving NF (i.e. NG-RAN and UPF).

5)	In the current release of the specification, only the energy-related information of user plane communication (not control plane signalling) is supported.

NOTE 2:	In this Release, the gNB neither supports per-UE-per-PDU session nor per-UE-per-QoS flow level energy consumption reporting.

NOTE 3:	Depending on TSG RAN decision to work on energy efficiency topic, it can be envisaged to work on procedures where the NG-RAN can detect whether traffic is transmitted for a UE in certain energy state for the network (the definition of these energy state rely on RAN WGs to define these) and reporting the information to the 5GC.

End of CHANGES