SA WG2 Meeting #164	S2-2407466

Maastricht, Netherlands, August 19 – August 23, 2024	(Revision of S2-240xxxx)

Source: 	Samsung, SK Telecom

Title: 	KI#3: Conclusion update

Document for: 	Approval

Agenda Item: 	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract of the contribution: This contribution provides update of conclusion for key issue #3.

1	Discussion

This contribution provides the conclusions for KI#3.

2 	Proposal

It is proposed to capture the following contents into TR 23.700-66.

FIRST CHANGE

8.3	Conclusion for Key Issue #3: 5GS enhancements for network energy saving and efficiency

It is proposed to proceed to normative work based on the following principles:

1)	For Enhancements on NF discovery and (re-)selection based on energy related information:

-	NF profile may be extended (e.g. include the new energy related information or reuse existing NF profile parameters) to allow an operator to influence NF discovery and selection based on their energy strategy.

NOTE 1:	Which energy related information is included in the NF profile is to be further discussed in the normative phase. Only parameters captured in existing solutions will be considered. Coordinated with SA WG5 may be needed.

NOTE 2:	The information within NF profile is expected not to change frequently, e.g. constantly reflecting changes in energy consumption. The procedure for updating the information will be addressed in normative phase.

-	NF discovery and (re-)selection is enhanced to consider the energy related information from the NF profiles and/or discovery request from the NF consumer.

2)	For enhancements on existing operations and procedures for energy saving and energy efficiency:

-	UP path of PDU session may be adjusted.

-	The energy saving control (e.g. the amount of energy consumption, the level of energy efficiency) of the network slices are supported.

-	The AM/SM/UE policy control related procedures (e.g. NF re-selection, Network Slice Replacement, RFSP index re-allocation, URSP rules re-generation) are triggered for the UE which is allowed and requested to activate network energy saving functionality.

NOTE 3:	The energy related decision will also consider operator’s policy.

End of CHANGES