SA WG2 Meeting #S2-164	S2-2407494

Maastricht, Netherlands, 19 - 23 August, 2024	(was S2-24xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1098 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to roaming procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-07-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The rejection related steps procedures only mentioned "the following steps are skipped" while missing the corresponding failure response message description are not workable for the NF consumer to handle without the related rejection response. The roaming analytics and data collection procedures in each procedure figures are not fully aligned with the following steps descriptions. "Nnwdaf" was wrongly described as "Nwdaf" in some roaming procedure figures and step description need to be corrected. Also several editorial corrections are needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In roaming procedures: - add the missing rejection response descriptions to be workable. - correct the roaming analytics and data collection procedures in each procedure figures and steps descriptions to be fully aligned. - correcte "Nwdaf" as "Nnwdaf" in related figures and step description. - several editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not workable and not aligned roaming procedures, also wrong service naming for roaming services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.5.2, 6.1.5.3, 6.2.10, 6.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

#### 6.1.5.2	Analytics Exposure from HPLMN to VPLMN

Figure 6.1.5.2-1 shows the procedure where a V-RE-NWDAF requests network analytics (i.e. slice load level analytics, NF load analytics, service experience analytics, etc.) in HPLMN from a H-RE-NWDAF, upon receiving an analytics information request/subscribe message from a service consumer NF (e.g. AMF) in the VPLMN.

Figure 6.1.5.2-1: Procedure for analytics exposure from HPLMN to VPLMN

1a.	The Consumer NF in VPLMN (e.g. AMF) discovers a V-RE-NWDAF as described in clause 5.2 and sends an Analytics request/subscribe message (Analytics ID, Analytics Filter Information, Target of Analytics Reporting) to the V-RE-NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request service operation or a Nnwdaf_AnalyticsSubscription_Subscribe service operation.

1b.	For the inbound roaming UE(s) indicated in the Target of Analytics Reporting,

- The V-RE-NWDAF determines based on operator configuration and the requested analytics whether analytics or input data from the HPLMN are required, or the analytics can be derived locally. The subsequent steps only apply if analytics from the HPLMN are required. If input data from the HPLMN are required, the procedures in clause 6.2.11 apply.

NOTE 1:	It is possible that the Target of Analytics Reporting sent by the Consumer NF to the V-RE-NWDAF includes both inbound roaming UE(s) and non-roaming UE(s).

-	The V-RE-NWDAF checks the roaming agreements related to analytics from the HPLMN to determine if the roaming analytics request/subscribe message can be accepted or must be rejected. If the V-RE-NWDAF determined the roaming analytics request/subscribe message must be rejected, the following steps 2-5 are skipped.

2.	The V-RE-NWDAF discovers a H-RE-NWDAF as described in clause 5.2 if the V-RE-NWDAF determined the roaming analytics request/subscribe message can be accepted. The V-RE-NWDAF sends a roaming analytics request/subscribe message (Analytics ID, Analytics Filter Information, Target of Analytics Reporting) to H-RE-NWDAF by invoking a Nnwdaf_RoamingAnalytics_Request service operation or a Nnwdaf_RoamingAnalytics_Subscribe service operation, based on the Analytics request/subscribe message received from the Consumer NF in the VPLMN. The Target of Analytics Reporting sent by the V-RE-NWDAF to the H-RE-NWDAF only contains the inbound roaming UE(s).

NOTE 2:	The inbound roaming UE(s) are distinguished by the V-RE-NWDAF according to the UE ID(s) (i.e. SUPI(s)).

3a.	The H-RE-NWDAF checks the roaming agreements between the HPLMN and the VPLMN, and user consent for analytics as defined in clause 6.2.9 if needed, to determine if the roaming analytics request/subscribe message can be accepted or must be rejected. If the roaming analytics request/subscribe message is rejected, the following steps 3b-3c are skipped.

If the H-RE-NWDAF supports to generate the requested analytics, it collects data from the NF(s) and/or OAM in the HPLMN and derives the requested analytics; otherwise steps 3b and 3c are executed.

NOTE 3:	See clause X.7 and Annex V of TS 33.501 [49] for details of the user consent check procedures. See clause X.8 of TS 33.501 [49] for protection of analytics exchange in roaming case.

3b-3c.	[Optional] If the H-RE-NWDAF does not support to generate the requested analytics, it may request/subscribe to other NWDAF(s) in the HPLMN (if available) for the analytics and get corresponding response/notification.

4.	On success, the H-RE-NWDAF sends the HPLMN analytics information to the V-RE-NWDAF using either Nnwdaf_RoamingAnalytics_Request response or Nnwdaf_RoamingAnalytics_Notify service operation, depending on the service used in step 2. The H-RE-NWDAF may restrict the exposed analytics information based on HPLMN operator polices. Otherwise, the rejection cause is sent to the V-RE-NWDAF if the roaming analytics request/subscribe message was rejected in step 3a.

5.	[Optional] If the Consumer NF also indicates request or subscription of analytics information available in the VPLMN (e.g. via Target of Analytics Reporting) in step 1, the V-RE-NWDAF collects data from the NF(s) and/or OAM in VPLMN and derives the requested analytics. These steps can be executed in parallel with steps 3-4. The V-RE-NWDAF may perform analytics aggregation with the analytics information received from the H-RE-NWDAF and analytics information generated by itself, based on the analytics request or subscription.

6.	On success, the V-RE-NWDAF sends the HPLMN analytics information received in step 4, or the aggregated analytics information if step 5 are performed, to the Consumer NF in the VPLMN using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify service operation, depending on the service used in step 1. Otherwise, the rejection cause is sent to the Consumer NF if the analytics request/subscribe message is rejected in step 1b or if the rejection cause was received in step 4 or failure happened to the analytics.

NOTE 4:	The present document describes that the RE-NWDAF can perform analytics aggregation for roaming scenario, but whether and how the RE-NWDAF performs analytics aggregation for roaming scenario are up to implementation.

# *** 2nd Change ***

#### 6.1.5.3	Analytics Exposure from VPLMN to HPLMN

Figure 6.1.5.3-1 shows the procedure that the H-RE-NWDAF requests network analytics (i.e. service experience analytics, slice load level analytics, etc.) in the VPLMN from the V-RE-NWDAF, upon receiving an analytics information request/subscription from the service consumer NF (e.g. PCF) in the HPLMN.

Figure 6.1.5.3-1: Procedure for analytics exposure from VPLMN to HPLMN

1a.	If the Consumer NF in the HPLMN (e.g. H-PCF) is aware that the UE(s) indicated in Target of Analytics Reporting is/are outbound roaming UE(s), the Consumer NF discovers a H-RE-NWDAF as described in clause 5.2 and sends an Analytics request/subscribe message (Analytics ID, Target of Analytics Reporting, Analytics Filter Information) to the H-RE-NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request service operation or a Nnwdaf_AnalyticsSubscription_Subscribe service operation.

1b.	For the outbound roaming UE(s) indicated in the Target of Analytics Reporting,

-	The H-RE-NWDAF determines based on operator configuration and the requested analytics whether analytics or input data from the VPLMN are required, or the analytics can be derived locally. The subsequent steps only apply if analytics from the VPLMN are required. If input data from the VPLMN are required, the procedures in clause 6.2.10 apply.

-	If the Consumer NF is unaware that the Target of Analytics Reporting includes roaming UE(s) and sends the Analytics request/subscribe message to a H-NWDAF which does not support roaming exchange capability, then the H-NWDAF may perform either of the following after determining that the Target of Analytics Reporting includes roaming UE(s) (e.g. by inquiring the AMF(s) serving the UE(s) at the UDM with the Nudm_UECM service):

-	forward the Analytics request/subscribe message to a H-RE-NWDAF after discovering a H-RE-NWDAF as described in clause 5.2. The H-NWDAF may include the VPLMN ID in the Analytics request/subscribe message; or

-	reject the Analytics request/subscribe message with a proper cause value. And the following steps will not be performed.

NOTE 1:	It is possible that the Target of Analytics Reporting sent by the Consumer NF to the H-RE-NWDAF includes both outbound roaming UE(s) and non-roaming UE(s).

NOTE 2:	The H-NWDAF is not depicted in the flow.

-	If PLMN ID of the VPLMN is not included in the Analytics request/subscribe message, the H-NWDAF inquires it at the UDM for the UE(s) indicated as Target of Analytics Reporting. The H-RE-NWDAF checks user consent for analytics as defined in clause 6.2.9. The H-RE-NWDAF checks the roaming agreements between the HPLMN and the VPLMN to determine if the roaming analytics request/subscription can be accepted or must be rejected. If the H-RE-NWDAF determined the roaming analytics request/subscribe message must be rejected, the following steps 2-5 are skipped.

NOTE 3:	See clause X.7 and Annex V of TS 33.501 [49] for details of the user consent check procedures. See clause X.8 of TS 33.501 [49] for protection of analytics exchange in roaming case.

2.	The H-RE-NWDAF discovers the V-RE-NWDAF as described in clause 5.2 if the H-RE-NWDAF determined the roaming analytics request/subscribe message can be accepted.

The 	H-RE-NWDAF sends a roaming analytics request/subscribe message (Analytics ID, Analytics Filter Information, Target of Analytics Reporting, [NF ID(s)]) to the V-RE-NWDAF by invoking a Nnwdaf_RoamingAnalytics_Request service operation or a Nnwdaf_RoamingAnalytics_Subscribe service operation, based on the Analytics request/subscribe message received from the Consumer NF in HPLMN. The Target of Analytics Reporting sent by the H-RE-NWDAF to the V-RE-NWDAF only contains the outbound roaming user(s). The H-RE-NWDAF may obtain NF ID(s) of the NF(s) serving the roaming UE(s) in the VPLMN, e.g. AMF ID(s), SMF ID(s), from the UDM and include the NF ID(s) in the analytics request/subscribe message.

3a.	The V-RE-NWDAF checks the roaming agreements between the HPLMN and the VPLMN, to determine if the roaming analytics request/subscribe message can be accepted or must be rejected. If the roaming analytics request/subscribe message is rejected, the following steps 3b-3c are skipped.

If the V-RE-NWDAF supports to generate the requested analytics, it collects data from the NF(s) serving the roaming UE(s) and/or OAM in VPLMN and derives the analytics; otherwise steps 3b and step 3c are executed. The NF(s) serving the roaming UE(s), e.g. AMF(s) or SMF(s), if indicated in step 2, can be used as data source.

3b-3c.	[Optional] If the V-RE-NWDAF does not support to generate the requested analytics, it may request/subscribe to other NWDAF(s) in the VPLMN (if available) for the analytics and get corresponding response/notification(s).

4.	On success, the V-RE-NWDAF sends the VPLMN analytics information to the H-RE-NWDAF using either Nnwdaf_RoamingAnalytics_Request response or Nnwdaf_RoamingAnalytics_Notify service operation, depending on the service used in step 2. The V-RE-NWDAF may restrict the exposed analytics information based on VPLMN operator polices. Otherwise, the rejection cause is sent to the H-RE-NWDAF if the roaming analytics request/subscribe message was rejected in step 3a.

5.	[Optional] If the Consumer NF also indicates request or subscription of analytics information available in the HPLMN (e.g. via Target of Analytics Reporting) in step 1, The H-RE-NWDAF collects data from the NF(s) and/or OAM in HPLMN and derives the analytics. These steps can be executed in parallel with steps 3-4. The H-RE-NWDAF may perform analytics aggregation with the analytics information received from the V-RE-NWDAF and analytics information generated by itself, based on the analytics request or subscription.

6.	On Success, the H-RE-NWDAF sends the VPLMN analytics information received in step 4, or the aggregated analytics information if step 5 are performed, to the Consumer NF in HPLMN using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1. Otherwise, the rejection cause is sent to the Consumer NF if the analytics request/subscribe message is rejected in step 1b or if the rejection cause was received in step 4 or failure happened to the analytics.

If the Analytics request/subscribe message is forwarded to the H-RE-NWDAF by a H-NWDAF as described in step 1, the H-NWDAF forwards the analytics information received from the H-RE-NWDAF to the Consumer NF using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

NOTE 4:	The present document describes that the RE-NWDAF may perform analytics aggregation for roaming scenario, but whether and how the RE-NWDAF performs analytics aggregation for roaming scenario are up to implementation.

# *** 3rd Change ***

### 6.2.10	Data collection by H-RE-NWDAF from V-RE-NWDAF for outbound roaming users

This procedure may be used by the RE-NWDAF in the HPLMN as service consumer to subscribe/unsubscribe to notifications about input data from the VPLMN for outbound roaming users (from the HPLMN perspective). The H-RE-NWDAF and V-RE-NWDAF in the procedure are NWDAFs with roaming exchange capability.

Figure 6.2.10-1: data collection by H-RE-NWDAF from V-RE-NWDAF for outbound roaming users

1.	For subscription to collected data related to the UE(s), the H-RE-NWDAF checks the user consent of related users depending on local policy or regulations.

NOTE 1:	See clause X.7 and Annex V TS 33.501 [49] for details of the user consent check procedures. See clause X.8 of TS 33.501 [49] for protection of data exchange in roaming case.

2.	The H-RE-NWDAF of HPLMN discovers the V-RE-NWDAF of VPLMN that supports the Nnwdaf_RoamingData service using the NRF as specified in Clause 5.2.

NOTE 2:	The access to the Nnf_EventExposure services is expected to be restricted to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy.

3.	The H-RE-NWDAF subscribes/unsubscribes to notifications about input data by invoking the Nnwdaf_RoamingData_Subscribe / Nnwdaf_RoamingData_Unsubscribe service operation. It optionally may indicate the IDs of AMFs and for local breakout also SMFs in the VPLMN handling related UEs, as obtained from the UDM.

4.	The V-RE-NWDAF checks if the HPLMN is authorised to subscribe to the input data based on VPLMN operator polices (that may depend on the HPLMN and may indicate permissible or restricted input data and related parameters). If the HPLMN is not authorized to subscribe to the input data, the following steps 5-6 are skipped.

5.	The V-RE-NWDAF may trigger new data collection from NF(s) (as indicated via the AMF ID(s) or SMF ID(s)) if needed and monitors the requested input data using procedures as described in clauses 6.2.1 to 6.2.8.

6.	The V-RE-NWDAF may restrict the exposed input data based on VPLMN operator polices (that may depend on the HPLMN) and may store them for auditing.

7.	On success, the V-RE-NWDAF respond or notify the subscribed available input data to the H-RE-NWDAF. Otherwise, the rejection cause is sent to the H-RE-NWDAF if the HPLMN is not authorized to subscribe to the input data in step 4 or failure happen to the input data collection.

# *** 4th Change ***

### 6.2.11	Data collection by V-RE-NWDAF from H-RE-NWDAF for inbound roaming users

This procedure may be used by an RE-NWDAF in the VPLMN to subscribe/unsubscribe to notifications about input data from the HPLMN for inbound roaming users (from the VPLMN perspective). H-RE-NWDAF and V-RE-NWDAF in the procedure are NWDAFs with roaming exchange capability.

Figure 6.2.11-1: Data Collection by V-NWDAF from H-NWDAF for inbound roaming users

1.	The V-RE-NWDAF of VPLMN discovers an H-RE-NWDAF of the HPLMN that supports the Nnwdaf_RoamingData service using the NRF as specified in clause 5.2.

NOTE 1:	The access to the Nnf_EventExposure services is expected to be restricted to NF service consumers within the same PLMN to prevent bypassing checks based on user consent and operator policy

2.	The V-RE-NWDAF subscribes/unsubscribes to input data information by invoking Nnwdaf_RoamingData_Subscribe / Nnwdaf_RoamingData_Unsubscribe service operation.

3.	The H-RE-NWDAF checks if the VPLMN is authorised to subscribe to the indicated input data based on the HPLMN operator polices (that may depend on the VPLMN and may indicate permissible or restricted input data and related parameters) and user consent of related users. If the VPLMN is not authorized to subscribe to the input data, the following steps 5-6 are skipped.

NOTE 2:	See clause X.7 and Annex V of TS 33.501 [49] for details of the user consent check procedures. See clause X.8 of TS 33.501 [49] for protection of data exchange in roaming case.

NOTE 3:	User consent check needs to be aligned with SA WG3.

4.	The H-RE-NWDAF may trigger new data collection if needed and monitors the requested input data, using procedures as described in clauses 6.2.1 to 6.2.8.

5.	The H-RE-NWDAF may restrict the exposed input data based on HPLMN operator polices (that may depend on the VPLMN) and may store them for auditing.

6.	On success, the H-RE-NWDAF respond or notify the subscribed available input data to the V-RE-NWDAF. Otherwise, the rejection cause is sent to the V-RE-NWDAF if the VPLMN is not authorized to subscribe to the input data in step 3 or failure happen to the input data collection.

*** End of Changes ***