3GPP TSG-SA WG2#164	S2-2407507

Maastricht, NL, 19-23 August 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated

Title:	Analysis of RAN2 LS on Multi-Modality at RAN and Way Forward

Document for:	Discussion and Agreement

Agenda Item:	19.3.1

Work Item / Release:	FS_XRM_Ph2

Abstract of the contribution: This paper discusses the contents of the RAN2 LS on multi-modality awareness at RAN, points out the relationship to earlier conclusions that SA2 has reached in the Rel-18 timeframe, lists open issues, and based on this, proposes a war forward.

# 1.	Discussion

## 1.1	Overview

In R2-2405782 [1], RAN2 informs SA2 as follows:

RAN2 has agreed that multi-modality awareness at RAN is supported for both UL and DL. To continue the discussion during the study phase, RAN2 needs to understand whether/what information can be provided from SA2 (e.g., MMSID and/or synchronization thresholds in order to allow e.g., coordinated handling of the flows at RAN).

The remainer of this paper discusses the contents of the LS, points out the relationship to earlier conclusions that SA2 has reached in the Rel-18 timeframe, lists open issues, and based on this, proposes a war forward.

## 1.2	Unclear RAN2 agreements

The RAN2 LS [1] does not explain what "multi-modality awareness at RAN" refers to. Instead, the LS merely gives a single high-level example ("coordinated handling of the flows at RAN") while not elaborating on the details of the foreseen RAN behavior.

Coordinated handling of the flows at RAN can have multiple meanings, for example (non-exhaustive list):

-	Coordinated admission control for multiple flows at RAN;

-	Coordinated scheduling across flows with the goal of inter-flow latency reduction by artificially delaying (buffering) packets of a first flow until the related packets of a second flow are available at RAN so that they can be jointly delivered to the UE in DL direction;

-	Coordinated scheduling across flows with the goal to reduce UE energy consumption.

Without further details on the meaning of multi-modality awareness at RAN, specifically without further details on the foreseen specific NG-RAN actions that "coordinated handling of the flows at RAN" entails, SA2 cannot analyze system impacts that such multi-modality awareness may have. Consequently, SA2 also cannot determine whether and which information to provide to NG-RAN to enable the same.

Observation 1: The RAN2 LS [1] does not explain what "multi-modality awareness at RAN" refers to. The example "coordinated handling of the flows at RAN" given in the LS can be interpreted in many ways. Without further details, SA2 cannot analyze system impacts that such multi-modality awareness may have and hence SA2 cannot determine whether and which information to provide to NG-RAN to enable the same.

## 1.3	 SA2 conclusions on multi-modality support in Rel-18

During the Rel-18 FS_ XRM study, SA2 discussed and analyzed coordinated admission control for multiple flows at RAN as well as coordinated scheduling.

S2-2208390 [2] and S2-2210353 [3] (co-sourced by Ericsson, Deutsche Telekom, Apple, Qualcomm Incorporated, Intel and Meta USA) evaluated all Rel-18 FS_XRM solutions related to multi-modality and dismissed both coordinated admission control and coordinated scheduling for multiple related flows. Based on this, SA2 decided against providing further flow coordination information to NG-RAN.

Observation 2: Based on various evaluation papers (e.g., S2-2208390 [2] and S2-2210353 [3] (co-sourced by Ericsson, Deutsche Telekom, Apple, Qualcomm Incorporated, Intel and Meta USA), SA2 dismissed coordinated admission control and coordinated scheduling for multiple related flows. Consequently, SA2 also decided against providing further flow coordination information to NG-RAN.

## 1.4	Open issues

As illustrated in section 1.1, RAN2's intention for coordinated handling of flows at RAN is unfortunately unclear.

However, if the intention was to enable coordinated scheduling across flows, i.e., to artificially delay (buffer) packets of a first flow until the related packets of a second flow are available at RAN so that they can be jointly delivered to the UE in DL direction, then further SA2 and SA4 studies would be needed first. For instance the following aspects would need to be analysed (non-exhaustive list):

-	Do XR applications benefit from coordinated scheduling (SA4)?

-	Will the buffer management of XR applications need to consider if coordinated scheduling is performed (SA4)?

-	More generally:

-	Is there a need to inform XR applications if NG-RAN artificially delays packets to enable coordinated scheduling (SA4), and if so, how (SA2)?

-	Is there a need to enable XR applications to control whether NG-RAN performs coordinated scheduling (e.g., because an XR application may prefer its packets to not be delayed artificially)?

Observation 3: While RAN2's intention for coordinated handling of flows at RAN is unclear, further aspects would need to be studied in SA2 (and possibly SA4) if the intention was to enable coordinated scheduling across flows. Issues to be studied include benefits of coordinated scheduling for XR applications and the need for XR application involvement.

# 2	Way forward

This paper made the following observations:

-	Observation 1: The RAN2 LS [1] does not explain what "multi-modality awareness at RAN" refers to. The example "coordinated handling of the flows at RAN" given in the LS can be interpreted in many ways. Without further details, SA2 cannot analyze system impacts that such multi-modality awareness may have and hence SA2 cannot determine whether and which information to provide to NG-RAN to enable the same.

-	Observation 2: Based on various evaluation papers (e.g., S2-2208390 [2] and S2-2210353 [3] (co-sourced by Ericsson, Deutsche Telekom, Apple, Qualcomm Incorporated, Intel and Meta USA), SA2 dismissed coordinated admission control and coordinated scheduling for multiple related flows. Consequently, SA2 also decided against providing further flow coordination information to NG-RAN.

-	Observation 3: While RAN2's intention for coordinated handling of flows at RAN is unfortunately unclear, further aspects would need to be studied in SA2 (and SA4) if the intention was to enable coordinated scheduling across flows. Issues to be studied include benefits of coordinated scheduling for XR applications and the need for XR application involvement.

Given these observations, this paper proposes to reply to RAN2 as summarized in S2-2407506 [4].

# 3	References

[1]	RAN2: LS on multi-modality awareness at RAN. R2-2405782

[2]	Ericsson, Deutsche Telekom, Meta, Apple, Qualcomm Incorporated: KI#1 and KI#2, Solutions evaluation and conclusion. S2-2406609.

[3]	Ericsson, Deutsche Telekom, Apple, Qualcomm Incorporated, Intel, Meta USA: KI#1: Updates to conclusions. S2-2210353.

[4]	Qualcomm Incorporated: [Draft] Reply LS on multi-modality awareness at RAN. S2-2407506.