3GPP TSG-SA WG2#164	S2-2407508

Maastricht, Netherlands, 19 – 23 August, 2024	(was S2-240xxxx)

Source:	Qualcomm Incorporated

Title:	KI 1.3: Conclusion for IMS MMTel sessions and SMS delivery

Document for:	Discussion/Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This contribution proposes some principles for the conclusion of KI#1.3.

# 1.	Discussion

## 1.1	Introduction

During the informal email discussion on DualSteer before SA#164, the issue of potential impacts to Mobile Originated and Mobile Terminated voice calls was raised and, it was asked whether one Mobile Station ISDN Number (MSISDN) or two MSISDNs should be used per DualSteer Device. Finally, it was also asked if DualSteer operation has any impact on SMS over NAS delivery.

This paper elaborates on the scenarios of IMS MMTel sessions and SMS over IP or SMS over NAS and addresses the above-mentioned issues.

## 1.2	Handling of IMS MMTel sessions

### 1.2.1	Problem statement

The problem to address with respect to IMS Multi-Media Telephony (MMTel) sessions for DualSteer device is twofold:

-	whether one or two MSISDNs are used by the (UEs of the) DualSteer Device;

-	how to handle IMS MMTel sessions when (one of) the IMS registered UE(s) moves out of coverage.

The authors of this paper identified at least two options which do not require any standard impact (in addition to the Rel-18 standards or to the proposed solution for DualSteer).

### Solution A: Only one MSISDN active for IMS service with the DualSteer Device

This approach assumes the following:

-	Only one of the UEs/stacks of the DualSteer Device is allocated an MSISDN that is registered successfully in IMS. The other UE/stack of the DualSteer device may, but does not have to, have an MSISDN but, in any case, it does not have an IMS subscription and is not registered to IMS.

To address the scenario above, the simplest solution is to apply Solution 1.11, with the following caveats:

-	The list of allowed PLMN/RAT combinations for the UE/stack registered to IMS should be as extensive as possible, e.g., similar to those used for UEs without DualSteer subscription, so to maximize the chances of that UE/stack to be in coverage;

NOTE: The aspects related to PLMN/RAT selection are pending SA1/CT1 progress.

-	The DualSteer UE Rules sent to that UE/stack for the IMS traffic should not allow switching the traffic to the other (non-IMS registered) UE.

No impact on standard is foreseen (in addition to the impacts of Solution 1.11).

### 1.2.3	Solution B: Two MSISDNs allocated to DualSteer Device, both are active for IMS service, but only one is visible to users

This approach assumes the following:

-	Two MSISDNs are allocated to the DualSteer Device (i.e., one MSISDN for each UE) and are active for the IMS MMTel service, but only one of the two is visible/known to the users for MO/MT IMS calls.

-	The UE/stack currently registered for IMS is NOT handed over to the PLMN/RAT combination currently used by the other UE/stack

With the assumptions above, the main step of the solution is:

-	If the UE/stack using IMS loses access to the current PLMN/RAT, then the inter-UE transfer procedure (see text and figure below, extracted from TS 23.237) can be triggered to move IMS voice session over to the other UE/SIM.

TS 23.237, Figure 6a.6.1-1: Inter-UE transfer without establishing Collaborative Session - initiated by the source UE

1.	UE-1 sends IUT media and service control transfer request to the SCC AS to transfer the session that includes Media-A from UE-1 to UE-2. The IUT media and service control transfer request shall include enough information for the network to:

-	identify the transferred session;

-	identify that the target of the transferred session is UE-2;

-	identify the access leg of UE-1 to be transferred to UE-2.

2.	If the SCC AS does not have the list of Public User Identities that share the service profile with the Public User Identity used by UE-1 for the Collaborative Session, the SCC AS retrieves this information. If the Collaborative Session Control transfer request indicates that UE-2 has the same Public User Identity (but different public GRUU) as UE-1, this step is skipped.

3.	The SCC AS ensures that the Public User Identity used by UE-2 shares the service profile with the Public User Identity used by UE-1 for the Collaborative Session. The SCC AS performs authorization as specified in clause 6a.12, then initiates the media session setup procedure with UE-2 for the Media-A, and updates the Remote Party.

4.	The SCC AS releases the session with UE-1.

When the above transfer is completed the SCC AS retains the service state (e.g. media flows status) of UE-2, and the UE-2 has the control of the IMS session.

NOTE 2:	If, besides the Media-A, there are other media flows in an unrelated session between UE-1 and Remote Party, the other media flows are not impacted by the above session transfer operation.

No impact on standards is foreseen, but this solution requires that UE and the IMS deployment support the standard inter-UE transfer procedure described above from TS 23.237.

## 1.3	Handling of SMS over NAS

Another aspect to address for DualSteer is how to handle the delivery Mobile Originated and Mobile Terminated SMS over NAS in case the targeted MSISDN is not reachable.

Even for this aspect, and irrespective of which solution is adopted to support IMS MMTel session, the behavior of (UEs of) a DualSteer Device is the same as for Rel-18 UEs:

-	if the UE/stack is in coverage and the associated MSISDN is reachable, the SMS is delivered immediately;

-	if (the UE/stack is not in coverage and) the associated MSISDN is not reachable, the SMS will be delivered as soon as the MSISDN becomes available again.

Similarly to the handling of IMS MMTel sessions, for simplicity it is recommended that only one MSISDN is visible/known to users for a DualSteer Device.

# 2.	Text proposal

It is proposed to agree with Solution 2 of clause 1.2.2 and to include the following changes in TS 23.700-54:

# >>>>BEGINNING OF CHANGES<<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[6]	IETF RFC 9298: "Proxying UDP in HTTP".

[7]	IETF RFC 9484: "Proxying IP in HTTP".

[8]	IETF draft-ietf-masque-connect-ethernet: "Proxying Ethernet in HTTP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[9]	IETF RFC 9114: "Hypertext Transfer Protocol Version 3 (HTTP/3)".

[10]	IETF draft-ietf-httpbis-connect-tcp: "Template-Driven HTTP CONNECT Proxying for TCP".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[11]	IETF RFC 9297: "HTTP Datagrams and the Capsule Protocol".

[12]	IETF RFC 9000: "QUIC – A UDP based Multiplexed and Secured Protocol".

[13]	IETF RFC 9001: "Using TLS to Secure QUIC".

[14]	IETF RFC 9002: "QUIC Loss Detection and Congestion Control".

[15]	IETF RFC 9221: "An Unreliable Datagram Extension to QUIC".

[16]	IETF draft-ietf-quic-multipath: "Multipath Extension for QUIC".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[17]	3GPP TR 23.700-53: "Study on access traffic steering, switching and splitting support in the 5G system architecture; Phase 3".

[18]	IETF RFC 9369: "QUIC Version 2".

[19]	IETF RFC 9220: "Bootstrapping WebSockets with HTTP/3".

[20]	3GPP TS 33.402: "3GPP System Architecture Evolution (SAE); Security aspects of non-3GPP accesses".

[21]	3GPP TS 29.500: "Technical Realization of Service Based Architecture; Stage 3".

[22]	IETF RFC 4555: "IKEv2 Mobility and Multihoming Protocol (MOBIKE)".

[23]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[24]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[25]	3GPP TS 32.421: "Telecommunication management; Subscriber and equipment trace; Trace concepts and requirements".

[x]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 2".

# >>>>NEXT CHANGE<<<<

## 8.1	Conclusions for DualSteer

### 8.1.xy	KI#1.3 Session management aspects for DualSteer

#### 8.1.xy.1	Generic aspects

# >>>>SKIP TEXT FOR GENERIC ASPECTS<<<<

#### 8.1.xy.2	Handling of IMS MMTel sessions

To handle Mobile Originated and Mobile Terminated IMS MMTel sessions, the following is concluded.

Solution A: Only one MSISDN active for IMS service with the DualSteer Device.

This approach assumes the following:

-	Only one of the UEs/stacks of the DualSteer Device is allocated an MSISDN that is successfully registered to the IMS. The other UE/stack of the DualSteer device may, but does not have to, have an MSISDN but, in any case, it does not have an IMS subscription and is not registered to IMS.

To address the scenario above, the simplest solution is to apply Solution 1.11, with the following caveats:

-	The list of allowed PLMN/RAT combinations for the UE/stack registered to IMS should be as extensive as possible, e.g., similar to those used for UEs without DualSteer subscription, so to maximize the chances of that UE/stack to be in coverage;

NOTE: The aspects related to PLMN/RAT selection are pending SA1/CT1 progress.

-	The DualSteer UE Rules sent to that UE/stack for the IMS traffic should not allow switching the traffic to the other (non-IMS registered) UE.

No impact on standard is foreseen in addition to the impacts already identifier for Solution 1.11.

Solution B: Two MSISDNs allocated to DualSteer Device, both are active for IMS service, but only one is visible to users.

This approach assumes the following:

-	Two MSISDNs are allocated to the DualSteer Device (i.e., one MSISDN for each UE) and are active for the IMS MMTel service, but only one of the two is visible/known to the users for MO/MT IMS calls.

-	The UE/stack currently registered for IMS is NOT handed over to the PLMN/RAT combination currently used by the other UE/stack

-	Both UE and IMS system support the Inter-UE Transfer procedure.

With the assumptions above, the solution is based on executing the Inter-UE Transfer procedure described in TS 23.237 [x] clause 6a.6.1.

No impact on standards is foreseen.

#### 8.1.xy.3	Handling of SMS over NAS

SMS over NAS sent to/from UEs of a DualSteer Device are handled as per Rel-18. For simplicity, only one MSIDN should be visible/known to the users for a DualSteer Device. No impact on standards is foreseen.

# >>>>END OF CHANGES<<<<