3GPP TSG-WG SA2 Meeting #164	S2-2407527

# Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1100 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Model training and provisioning for AI/ML based positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, China Unicom, China Mobile, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following conclusions have been reached for KI#1 "Enhancements to LCS to support Direct AI/ML based Positioning” in clause 8.1 of TR 23.700-84: Principle #1: The LMF is enhanced to perform location calculation based on ML model. The interaction between LMF and MTLF is only for model provisioning and possibly, based on coordination with RAN WGs, for data collection for training. Principle #3: MTLF is enhanced to perform model training for AI/ML based Positioning. - Principle #3.2: NWDAF containing MTLF trains ML model for AI/ML based Positioning based on request from the LMF or internal trigger. - Principle #3.3: MTLF collects training data (e.g. data as defined by RAN for AI/ML based Positioning and historical data stored in ADRF) from data sources. Based on the above conclusions, the procedures that NWDAF containing MTLF trains and provisions ML model for AI/ML based positioning are proposed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedures that NWDAF containing MTLF trains and provisions ML model for AI/ML based positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures of model training and provisioning by the NWDAF containing MTLF for AI/ML based positioning is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x (new), 6.x.1 (new), 6.x.2 (new), 6.x.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

6.x	Procedure for ML model training and provisioning for UE positioning

6.x.1	General

The NWDAF containing MTLF may support training ML models for AI/ML based positioning by the LMF, based on the request from the LMF or internal trigger. The input data for training ML models for AI/ML based positioning by the LMF is defined in clause 6.x.2.

To support AI/ML based positioning as defined in TS 23.273 [39], the LMF may request or subscribe to trained ML model(s) for UE positioning from the NWDAF containing MTLF.

The LMF provides the following input parameters in the Nnwdaf_MLModelProvision_Subscribe or Nnwdaf_MLModelInfo_Request service operation as defined in clause 7.5 and clause 7.6:

-	Indication for requesting a ML model for UE positioning;

-	[OPTIONAL] Target of ML Model Reporting: SUPI of the target UE for positioning, group of UEs (i.e. a list of Internal Group Ids) or any UE (i.e. all UEs under specified conditions (e.g. within an AoI, or served by a combination of S-NSSAI and DNN));

-	[OPTIONAL] ML Model Filter Information optionally including:

-	S-NSSAI;

-	DNN;

-	Service type (see TS 22.071 [35]), which identifies the type of the service which requests UE positioning;

-	LCS QoS, which includes Location accuracy, response time and/or LCS QoS Class;

-	one or more positioning methods;

-	Area of Interest.

-	[OPTIONAL] ML Model Target Period: indicates the time window for which the ML model are to be used for UE positioning.

-	[OPTIONAL] NF consumer information: identifies the vendor of LMF.

NOTE 1:	NF consumer information such as Vendor ID is defined in Stage 3.

-	[OPTIONAL] ML Model Interoperability Information, as described in clause 6.2A.2.

-	Other input parameters (excluding A list of Analytics IDs) as specified in clause 6.2A.2.

The NWDAF containing MTLF provides the output information as specified in clause 6.2A.2 (with the difference that no Analytics ID is indicated) about the trained ML model for UE positioning to the LMF, using the Nnwdaf_MLModelProvision_Subscribe response / Nnwdaf_MLModelProvision_Notify or Nnwdaf_MLModelInfo_Request response as defined in clause 7.5 and clause 7.6.

Editor's note:	The details of the input parameters and output information for requesting and provisioning the trained ML model for UE positioning are FFS.

6.x.2	Input data

To train the ML model for AI/ML based positioning, the NWDAF containing MTLF collects UE positioning related input data as listed in Table 6.x.2-1.

Table 6.x.2-1: Data collected by NWDAF for training the ML model for UE positioning

| Information | Source | Description |
| --- | --- | --- |
| UE ID | LMF | SUPI of the target UE. |
| UE location | LMF | Location of the UE as defined in TS 23.273 [39]. |
| UE velocity | LMF | Velocity of the UE as defined in TS 23.273 [39]. |
| Timestamp | LMF | A time stamp when UE position is measured. |
| Quality indicator | LMF | Quality indicator of UE position. |
| LCS Accuracy | LMF | Accuracy of UE position, see TS 23.273 [39]. |
| LCS response time | LMF | Response time for UE positioning, see TS 23.273 [39]. |
| LCS QoS Class | LMF | LCS QoS class as defined in clause 4.1b of TS 23.273 [39]. |
| Positioning method | LMF | Positioning method used for UE positioning, see TS 23.273 [39]. |
| Measurement data from the UE | LMF | UE positioning related measurement data (e.g. channel measurement, quality indicator and time stamp of the channel measurement) reported by the UE. |
| Measurement data from the NG-RAN | LMF | UE positioning related measurement data (e.g. channel measurement, quality indicator and time stamp of the channel measurement) reported by the NG-RAN. |
| PRU positions (0..max) | LMF | Position(s) and corresponding measurement data of the PRU(s), see clause 6.17.4 of TS 23.273 [39]. |
| > PRU position |  | PRU position, which includes the position of the PRU and related data (e.g. time stamp, quality indicator). |
| > PRU positioning measurement data |  | PRU positioning related measurement data (e.g. channel measurement, quality indicator and time stamp of the channel measurement) reported by the NG-RAN or UE (i.e. PRU). |
| MBSR position | LMF | MBSR position, which includes the location and optionally velocity of the MBSR that the UE connects to, see clause 5.9 of TS 23.273 [39], and related data (e.g. time stamp, quality indicator). |
| MBSR positioning measurement data | LMF | MBSR positioning related measurement data (e.g. channel measurement, quality indicator and time stamp of the channel measurement) reported by the MBSR. |
| Service type | LMF | Identifies the type of the service which requests the UE positioning, see TS 22.071 [35]. |
| UE location | AMF | UE location information, e.g. cell ID or TAI. |
| UE position | AF | UE position information obtained via application layer, e.g. GNSS positioning information. |

Editor's note:	The details of the input data are FFS.

6.x.3	Procedures

Figure 6.x.3-1: Procedure for NWDAF providing ML model for UE positioning

1.	The Consumer NF (i.e. LMF) discovers and selects an NWDAF containing MTLF, and sends an ML model request/subscribe (Indication of requesting a ML model for UE positioning, Target of ML model Reporting, ML model Filter Information, ML Model Reporting Information) to the NWDAF containing MTLF by invoking a Nnwdaf_MLModelInfo_Request or a Nnwdaf_MLModelProvision_Subscribe.

2.	The NWDAF containing MTLF collects UE positioning related input data from the LMF by invoking Nlmf_EventExposure_Subscribe (Event ID = UE positioning information, Event Filter information, Target of Event Reporting).

3.	The LMF obtains measurement data from the UE and NG-RAN for UE positioning.

Editor's note:	It is FFS how the LMF obtains measurement data from the UE and NG-RAN for UE positioning. Coordination with RAN WGs is needed.

4.	The LMF provides the measurement data from the UE and NG-RAN and other data as defined in Table 6.x.2-1 to the NWDAF containing MTLF.

5-6.	The NWDAF containing MTLF may collect UE positioning related input data from the AF as defined in Table 6.x.2-1 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service (Event ID = UE positioning information, Event Filter information, Target of Event Reporting) as defined in TS 23.502 [3].

7-8.	The NWDAF containing MTLF may collect UE positioning related input data as defined in Table 6.x.2-1 from the AMF.

9.	The NWDAF containing MTLF performs the ML Model (re)training for UE positioning using the collected data.

NOTE:	Step 2-9 can be performed before step 1, i.e. there can be an existing trained ML model for UE positioning for the UEs including the target UE in the NWDAF containing MTLF.

10.	The NWDAF containing MTLF provides the trained ML model for UE positioning to the LMF using either Nnwdaf_MLModelInfo_Request response or Nnwdaf_MLModelProvision_Subscribe response / Nnwdaf_MLModelProvision_Notify, depending on the service used in step 1.

11.	The LMF performs inference using the provisioned ML model and input data, and determines the position of the target UE.

* * * End of Changes * * * *