3GPP TSG-SA2 Meeting #164	S2-2407568

# 19 – 23 August 2024, Maastricht, Netherlands							(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1115 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI2: Vertical Federated Learning Registration, Discovery and Sample Alignmnet where FL Server is AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-07-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following CR adds description for Vertical Federated Learning Registration, Discovery, Sample Alignment and Selection of FL clients to participate in vertical federated learning where FL Server is AF taking into account the agreed Principles P#2.1 through P#2.5 in clause 8.2 of 3GPP TR 23.700-84 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding support of registration and discovery for vertical federated learning where FL Server is an AF in a new sub-clause under clause 6.2C.2.1. For selecting candidate FL clients a new VFL interoperability information field is proposed to be included to allow the FL server to select compatible FL clients. The procedure also includes an optional functionality at the NEF to perform sample alignment based on the Data Set requirements (i.e. Features/Samples) available at the AF acting as FL server |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing details on Registration, Discovery, Sample Alignment and Selection of FL clients for vertical federated learning. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.1.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ FIRST CHANGE *****************************

##### 6.2C.2.1.y	Support for Vertical Federated Learning where FL Server is AF

Figure 6.2C.2.1.y-1: Registration and Discovery procedure for Vertical Federated Learning where AF is FL Server

1.	NWDAF containing MTLF as FL Client NWDAF registers to NRF with its NF profile as described in steps 1-3 Figure 6.2C.2.1.y-1.

2.	If NEF is involved FL Server AF sends an FL Preparation Request including Analytic ID and Data Set Requirement which includes Features (Event IDs) and Samples needed to train a model.

Editor's Note: Where a new service or existing service is needed is FFS.

3-5	The FL Server AF (if NEF is not involved) or the NEF discovers other NWDAF(s) containing MTLF as FL Client NWDAF(s) from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML model required, FL capability Type (i.e. FL client), Service Area, NF type(s) of data sources from which the FL Client NWDAF is able to collect data for local model training, Time Period of Interest, VFL Interoperability Indicator.

Steps 6 to 11 are the sample alignment procedure

6.	FL Server AF (if NEF is not involved) or NEF sends Federated Learning preparation request to the FL Client NWDAF(s), using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service with the ML Preparation Flag, to check if the FL Client NWDAF(s) can meet the ML model training requirement (e.g. Analytics ID, Available data requirement, Availability time requirement (time span needed for the FL process), etc.). Available data requirement includes a list of Event IDs of the local data for training.

7.	The candidate NWDAF FL clients identify common samples and decide to join or not in the VFL process. Example criteria used by FL Client NWDAF(s) may be based on its data availability and time availability, computation and communication capability and VFL Interoperability information.

8.	FL Client NWDAF(s) invokes Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTraining_Subscribe response service operation or Nnwdaf_MLModelTrainingInfo_Request response service operation to indicate to the FL Server NWDAF whether it will join the FL procedure and may include the reason in the response message if it cannot join the FL process. The FL client NWDAF(s) include in the response the available data (i.e. Event IDs as a list of Features and Samples) if they decide to join the VFL process.

9.	If NEF is involved the NEF may optionally align the samples based on the Data Set Requirements sent by the FL Server AF.

10.	If NEF is involved the NEF sends an FL Preparation Request response including a list of FL Clients and corresponding features and aligned samples.

11.	FL Server AF determines the final list of FL Client NWDAF(s) to be involved in the VFL procedures. The FL Server AF assigns a VFL process ID corresponding to the VFL process and determines initial contribution weights from each VFL participant according to the data availability and/or importance of the feature for the VFL process.

************************************ END OF CHANGES ****************************************