3GPP TSG-SA2 Meeting #164	S2-2407574

# 19 – 23 August 2024, Maastricht, Netherlands							(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1119 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI2: Support of distributed inference for a model trained using VFL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | 3GPP SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-07-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following CR adds description for Vertical Federated Learning inference taking into account Principle#2.5 in clause 8.2 of 3GPP TR 23.700-84 P#2.5: For VFL inference process: P#2.5.1: NWDAF acting as VFL server scenario, the NF consumer of analytics will obtain the required output based on the VFL inference process coordinated by VFL server, which is generated via the VFL inference process between VFL sever and corresponding AFs or NWDAF acting as VFL client(s): - For NWDAF acting as VFL server, NWDAF triggers the VFL inference phase after receiving the subscription or request for analytics and delivers the analytics to the NF consumer. P#2.5.2: For the AF acting as a VFL server, the AF acting as VFL server can also start an inference process with corresponding NWDAF acting as VFL client(s). The AF may be triggered to start the inference by a 5GC consumer (i.e. NWDAF containing AnLF). Interactions are via NEF if AF is untrusted. P#2.5.3: Before performing the VFL inference, the NWDAF acting as VFL server determines corresponding AF(s) and/or NWDAF(s) as VFL client(s) for the inference process based on the same identifier used in the VFL training process. Related details will be specified in the normative phase. P#2.5.4: The VFL inference process may be controlled by a set of requirements, i.e. whether the determination if all VFL participants associated with the same identifier are needed in the VFL inference process may be based e.g. on accuracy requirements, the VFL signalling and load cost, contribution weights of each client, and temporal availability of output from VFL participants. P#2.5.5: When performing VFL model performance monitoring, inference data can be used for model retraining, which is aligned with R18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding descriptions for vertical federated learning inference procedure in a new sub-clause within clause 6.2C.2 that describes the vertical federated learning procedure where the FL server is the NWDAF and FL clients may be an NWDAF or AF. The changes proposed assume that the FL server and clients that were used during training are the same functions that are used for inference. Procedures are provided where the FL Server is NWDAF or AF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing details on vertical federated learning inference procedure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************************ FIRST CHANGE *****************************

##### 6.2C.2.x	Support for Vertical Federated Learning Inference

##### When the FL Server is an NWDAF the procedure is as follows:

Figure 6.2C.2.X-1: Distributed Inference when ML model is trained using VFL

1.	An NWDAF FL Server receives a request for analytics (e.g. Observed Service Experience Analytics) and includes analytics ID and analytics filters.

2.	The NWDAF FL Server determines the ML model requires vertical federated learning. If the NWDAF FL Server already has a trained ML model the NWDAF retrieves ML model information internally. Otherwise, the NWDAF FL Server  retrieves ML model information from the ADRF. ML model information includes the address of FL Client NWDAF(s)/AF(s), contributions weights, VFL process ID.

3.	The NWDAF FL Server sends an Inference Request to each participant of the VFL process. The request may include Analytics ID, filters such as the samples to do the inference calculation, VFL process ID. The Inference Request may be a subscription request or a one-time request. The request may also include an indication to provide accuracy report.

NOTE X: The request to an FL Client AF is sent via an NEF if the AF is untrusted.

4.	Each FL Client NWDAF or AF identifies the local ML model linked to the ML training process using the VFL process ID and computes an inference output using its local data. Each FL Client NWDAF or AF also provides a local accuracy metric of the local ML model by comparing predictions against real time data.

5.	The Inference output from each participant is sent to the NWDAF FL Server.

6.	If the NWDAF FL server has access to local data the NWDAF FL server trains its own local model.

7.	The NWDAF FL Server computes an aggregate inference output taking into account the contribution weights of each FL Client and also determines a global accuracy metric of the inference process taking into account each local accuracy metric received taking into account contribution weights from each FL client.

8.	The NWDAF FL Server prepares output data and determines accuracy of the analytics.

##### 9. The result is sent to the Analytics consumer.

When the FL Server is an AF the procedure is as in Figure 6.2C.2.X-1 with the following differences:

-	In step 1 the Analytics request may be received via an NEF if the AF is untrusted

-	In step 2 the FL Server AF identifies the FL clients based on local information. The procedure to retrieve ML model information is out of scope of 3GPP

-	In steps 3, 5 interaction with other AF FL clients is out of scope of 3GPP

************************************ END OF CHANGES ****************************************