3GPP TSG-SA WG2 Meeting #164	S2-2407591

# Maastricht, Netherlands, August 19 - 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1122 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NF Signalling Storm Analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following principles are concluded for KI#4 in clause 8.4 of TR 23.700-84 - A new Analytics ID to support Signalling storm Mitigation and Prevention caused by NFs signalling will be defined. - Analytics Filter includes information to assist the NWDAF for the signalling storm as defined above. - The following Input Data are agreed to be collected from NF, MDAF, AF and/or via NRF to support generation of the analytics by NWDAF: - Information related to control plane signalling exchange, including: - Request Type of control messages (and related posterior messages), message counts, success and failures, failure cause, fault information, response times for N1 and/or N2 interfaces and 5GC SBIs, taking information as described in clause 6.35.2.2 and table 6.39.1-1. - Information on NF contexts, including: - UE related number, state transition and timer information of a group of UEs as described in clause 6.35.2.2 and table 6.39.1-1; - NF load, status & resource usage as described in table 6.39.1-1. - The following Output Data from NWDAF are agreed to be included in Analytics output: - Statistics Analytics, including: - Target NF ID, cause of the signalling storm (i.e. signalling storm caused by UEs and signalling storm caused by NF), NF lists or a group of UEs, statistics corresponding to the input data. - Prediction Analytics, including: - Target NF ID, cause of the signalling storm, NF lists or a group of UEs, prediction corresponding to the input data, confidence level of the prediction. This CR provides the specification of the "new Analytics ID to support Signalling storm Mitigation and Prevention caused by NFs signalling" that includes input data, output analytics and the corresponding procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new sub-clause for NF Signalling Storm Analytics ID |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The concluded principles in the study phase are not implemented in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change (All New Text) **********

## 6.X	NF Signalling Storm analytics

### 6.X.1	General

This clause specifies the procedure for an NWDAF to provide analytics of signalling storm caused by NFs. This analytics ID may be used by other NFs and/or OAM to trigger appropriate signalling storm mitigation action.

The NWDAF collects information of control plane signalling between NFs on a specified Reference point and/or particular services, and NF context information, and then provides statistics and prediction of signalling storm cause by NFs.

The service consumer may be a NF (e.g., AMF, SMF, PCF), SCP, or OAM.

The consumer of the analytics includes in the request:

-	Analytics ID = "NF Signalling Storm".

-	Target of Analytics Reporting: NF Instance ID, NF ID.

-	Analytics Filter Information including:

-	Reference point(s) between NFs to be monitored for signalling storm;

-	Services provided by an NF to be monitored for signalling storm;

Editor's Note: The lists of supported Reference points and or services for signalling storm analytics are FFS.

-	Reporting Thresholds on the output parameters.

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Optional Preferred level of accuracy of the analytics.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

### 6.X.2	Input Data

NWDAF may collect input data from the NFs as the service producer or service consumer identified by the Analytics filter parameters. The detailed data are described in Table 6.X.2-1.

Table 6.X.2-1: Input data from 5GC related to NF control plane signalling

| Information | Source | Description |
| --- | --- | --- |
| NFs ID | Service Producer and Consumer NFs | The NF instance ID of the service producer and consumer NFs. |
| Service Operation information | Service Producer and Consumer NFs | The information of the reference point and corresponding service operation to be monitored. |
| Service Operation Counters | Service Producer NF / OAM (NOTE 1) | The counters related to the requests received by the service producer NF. |
| > Number of requests |  | The total number of requests of the service operation received in a time window. |
| > Number of pending requests |  | The number of requests of the service operation which are received but not processed and responded yet. |
| > Number of rejected requests |  | The number of rejected requests. |
| Service Operation Counters | Service Consumer NF / OAM (NOTE 1) | The counters related to the requests sent by the service consumer NF |
| > Number of requests |  | The total number of requests of the service operation sent in a time window. |
| > Number of pending requests |  | The number of requests of the service operation which has been sent but no response received yet. |
| > Number of rejected requests |  | The number of rejected requests. |
| NF Context data | Service Producer and Consumer NFs | The information about the service consumer and/or producer NFs context. |
| > NF load | NRF | The load of the NF instance(s) in the NF profile. |
| > NF status & resource usage | OAM | The usage of assigned virtual resources currently in use for the NF instance(s) as defined in clause 5.7 of TS 28.552 [11]. |
| Unexpected operational event | Service Producer |  |
| > Exceptional Load |  | NF load crossed a threshold, or it is increasing unexpectedly. |
| > Exceptional requests |  | Number of requests crossed a threshold, or it is increasing unexpectedly |
| NOTE 1: The already supported counters/KPIs by OAM defined per NF in TS 28.552 [X] are collected from OAM; other remaining counters/KPIs are collected from the corresponding NFs. |  |  |

NWDAF subscribes to the network data from 5GC NF(s) in the Table 6.X.2-1 by invoking Nnf_EventExposure_Subscribe service operation.

### 6.X.3	Output Analytics

The NWDAF provides the analytics (i.e. statistics and predictions) of control plane signalling. The output analytics are shown in Table 6.X.3-1 and Table 6.X.3-2.

Table 6.X.3-1: Output statistics of NF Signalling Storm Analytics

| Information | Description |
| --- | --- |
| Source NF(s) ID | The NF ID of the source NF(s) |
| Destination NF ID | The NF ID of the destination NF |
| Reference Point & Service Operation(s) signalling storm information | The information of the reference point and service operation(s) which are impacted by signalling storm. |
| >Exceptions ID | The ID of detected abnormality including: - Too many requests; - Too many pending requests; - Too many rejected requests |
| > Number of requests | The max/min/average of the number of requests |
| > Number of pending requests | The max/min/average of the number of requests that are pending |
| > Number of rejected requests | The max/min/average of the number of requests rejected |

Table 6.X.3-1: Output prediction of NF Signalling Storm Analytics

| Information | Description |
| --- | --- |
| Source NF(s) ID | The NF ID of the source NF(s) |
| Destination NF ID | The NF ID of the destination NF |
| Reference Point & Service Operation(s) signalling storm information | The information of the reference point and service operation(s) which are impacted by signalling storm. |
| >Exceptions ID | The ID of detected abnormality including: - Too many requests; - Too many pending requests; - Too many rejected requests |
| > Number of requests | The max/min/average of the number of requests |
| > Number of pending requests | The max/min/average of the number of requests that are pending |
| > Number of rejected requests | The max/min/average of the number of requests rejected |
| Confidence | Confidence of the prediction |

### 6.X.4	Procedures

Figure 6.X.4-1 shows the procedure where the NWDAF provides NF signalling storm analytics, in the form of statistics or predictions or both, to the analytics consumer.

Figure 6.X.4-1: Procedure for NF signalling storm analytics

1.	The analytics consumer sends Nnwdaf_AnalyticsSubscription_Subscribe(Analytics ID="NF Signalling Storm", Target of Analytics Reporting, Analytics Filter Information = "Reference point" and/or "Service", Analytics Reporting Information = ("Reporting Thresholds")) to the NWDAF.

2.	The NWDAF subscribes to the service consumer NF to collect the service operation counters and unexpected operational events via the corresponding Nnf_EventExposure_Subscribe service operation.

3.	The NWDAF subscribes to the service producer NF to collect the service operation counters and unexpected operational events via the corresponding Nnf_EventExposure_Subscribe service operation.

4.	The NWDAF using Nnrf_Management_NfStatus subscribes to NF load and status information from NRF about the service provider and service consumer NFs for the specified references point and/or service.

5.	The NWDAF subscribes to OAM services to get the resource usage and service operation information of the service producer and consumer NFs following the procedure captured in Clause 6.2.3

6.	The NWDAF derives the requested analytics considering the input parameters of the request including the reporting threshold.

7.	The NWDAF sends Nnwdaf_AnalyticsSubscription_Notify (NF signalling storm analytics, Subscription Correlation Id, Confidence).

8.	The NWDAF continues monitoring signallings between the NFs via collecting the service operation counter, NF load and other inputs specified in Table 6.X.2-1.

9.	The NWDAF derives new analytics taking into account the most recent data collected.

10.	In the case of detecting/predicting signalling storm, the NWDAF provides a notification using Nnwdaf_AnalyticsSubscription_Notify (NF signalling storm analytics, Subscription Correlation Id, Confidence).

********** End of Changes **********