SA WG2 Meeting #164	S2-2407596

# Maastricht, Netherlands, 19 – 23 August, 2024                  (revision of S2-240xxxx)

Source: 	CATT

Title: 	Discussion and conclusion proposals for AIoT Device ID component and usage

Document for: 	Approval

Agenda Item: 	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper discussed AIoT Device ID component and usage and made some conclusion proposals.

1	Discussion

Following the discussion in FS_AmbientIoT study item so far, it is understood that both EPC and 3GPP defined Device ID are necessary to support AIoT Services operations as well as AIoT Device authentication.

In general, the following components are necessary to compose the AIoT Device ID:

- EPC (used for AIoT Service operation)

- 3GPP defined Device ID (MCC+MNC+Device identifier, used for authentication of AIoT Device)

In general, the AIoT Device ID is used as followings (as illustrated in the Figure below)

- AF includes EPC and mask in the Inventory Request message to AIoTF via NEF

- AIoTF includes EPC and mask in the Inventory Request message to A-RAN

- A-RAN includes EPC and mask in the Initial Trigger Message

- AIoT Device includes its EPC and 3GPP defined Device ID in the Inventory Response message to AIoTF

- AIoTF performs authentication of AIoT Device with AUSF/UDM using 3GPP defined Device ID

- AIoTF includes EPC in the Inventory Response message to AF via NEF

Figure: Illustration of Usage of AIoT Device ID

2	Proposal

It is proposed to update TR 23.700-13 as follows.

# * * * * First change * * * *

8.1	Conclusions for Key Issue #2: Identification, Subscription, Registration and Connection management

For Key Issue #2, the following principles are concluded:

-	AIoT Device ID is composed of EPC (used for AIoT Service operation) and 3GPP defined Device ID (MCC+MNC+Device identifier, used for authentication of AIoT Device).

# * * * * End of Changes * * * *