3GPP TSG-WG SA2 Meeting #164	S2-2407620

Maastricht, NL, 19 - 23 August, 2024

Source:	Ericsson

Title:	KI#4, Update to conclusions

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: Resolves Editor’s Notes of Solution #26 in TR 23.700-70

# 1. Introduction/Discussion

Interim agreements for KI#4 introduce the following:

“For the multiplexed media flows which cannot be differentiated by using the legacy IP Packet Filter, the AF may provide additional Packet Filter in the "AF session with required QoS" procedure to the NEF/PCF.”

The additional Packet Filter is used by the UPF in DL direction and by the UE in the uplink direction respectively to map the media flows on the QoS Flows that are configured with the appropriate QoS parameters.

It can be assumed that in case of demanding QoS requirements for particular media flows, such QoS Flows will use the GBR resource type potentially being mapped on dedicated DRBs, i.e., these media flows will be configured with dedicated RAN resources. It is fair to assume that there is a limit to how many such dedicated resources can be supported by a given RAN node. Hence the admission of a QoS Flow using a GBR resource type can potentially impact admission of additional QoS Flows with GBR resource type, e.g., for other type of services, and thus have overall implication on the perceived capacity of a RAN node.

If the UE does not support the additional Packet Filters, all multiplexed data flows matching the legacy packet filter will be handled by the QoS rule with highest precedence. All the traffic matching the legacy packet filter will be mapped  to a single QoS Flow associated with that QoS rule. Consequently, different media flows will not be mapped to the dedicated QoS flows additionally configured. Hence the resources configured for these additional QoS Flows, reducing the capacity of the RAN node, will not be used. This might result in rejection of other QoS Flows demanding high QoS level. The bottom line is that the lack of knowledge of the UE capability wrt support of the ‘additional Packet Filter’ will negatively impact the RAN node capacity.

Furthermore, to avoid causing the same issues as described above when considering the DL direction, the SMF should also consider whether UPF supports additional Packet Filters before allocating QoS flows for the downlink direction.

To avoid waste of resources and thus impacting the RAN capacity stemming from network being unaware of the UE support level for the ‘additonal Packet Filter’ it is proposed to address the Editors note:

Editor's note:	Whether there is a need for a new UE capability indication for UL is FFS (e.g. for legacy UE handling and network handling).

With the additional agreement:

A new UE capability indicating support for additional Packet Filter as described in 2a required to map multiplexed media flows which cannot be differentiated by using the legacy IP Packet Filter will be introduced. The SMF receives the UE capability indication during PDU session establishment, selects the UPF and sends to the PCF an indication of whether the PDU session is capable of handling additional packet filters for multiplexed data flows (as a combination of UE and UPF support). The PCF makes use of such indication to handle the QoS requirements for media flows and additional packet filters received from the AF, so that PCC rules and QoS attributes are adapted, when required, using legacy packet filters avoiding the waste of resources in 5GC and RAN. In addition, the PCF notifies the AF that the requested QoS requirements for media flows and additional packet filters cannot be fulfilled, including information about the allocated QoS parameters to the service data flows.

# 2. Text Proposal

It is proposed to introduce the following changes vs. TR 23.700-70.

# * * * * First change * * * *

## 8.4	Interim agreements for Key Issue #4

The following aspects are concluded as principles for normative work:

1.	The AF may provide QoS requirements for media flows and additional packet filters as part of "AF session with required QoS". The detectable media flows that require different QoS treatment are mapped to distinct QoS Flows. If the AF does not provide any QoS requirement for the media flows or it provides the same QoS requirement, then the media flows can be mapped to the same QoS Flow. For the multiplexed media flows which cannot be differentiated by using the legacy IP Packet Filter, the AF may provide additional Packet Filter in the "AF session with required QoS" procedure to the NEF/PCF.

2.	In order to uniquely identify each media flow, the additional packet filter is used together with the legacy packet filter. The additional Packet Filter is used to differentiate the media flow among multiple media flows that share the same legacy packet filter in uplink (in UE) and/or downlink (in UPF) in the following case:

a)	For the media flows that are transported in (S)RTP, (S)RTCP and other associated protocols that are multiplexed into a single UDP/IP traffic flow as specified in IETF RFCs 5761, 5764, 7983, 8872, 8843 and 9443. The additional Packet Filter SHALL be the Synchronization Source (SSRC), Payload Type (PT) and RTP-M header fields.

3.	The PCC rule and N4 rules are enhanced to support the additional Packet Filter along with legacy packet filter.

4.	A new UE capability will be introduced, indicating support for additional Packet Filter that is required to map multiplexed media flows that cannot be differentiated by using the legacy IP Packet Filter as described in 2a. The SMF receives the UE capability indication during PDU session establishment, selects the UPF and sends to the PCF an indication of whether the PDU session is capable of handling additional packet filters for multiplexed data flows (as a combination of UE and UPF support). The PCF makes use of such indication to handle the QoS requirements for media flows and additional packet filters received from the AF, so that PCC rules and QoS attributes are adapted, when required, using legacy packet filters.

If the PCF does not support handling of additional packet filters for multiplexed data flows, the PCF rejects the QoS requirements for media flows and additional packet filters received from the AF

- If the PCF supports handling of additional packet filters for multiplexed data flows, but the PDU session is not capable of handling additional packet filters, then the PCF derives the QoS parameters for a single QoS flow with legacy packet filters, according to the AF request and operator policies, and provides the information to the SMF within the PCC rules following existing (legacy) procedures. In addition, the PCF notifies the AF that the requested QoS requirements cannot be fulfilled, including information about the allocated QoS parameters to the service data flows

- If the PCF and the PDU session support handling of additional packet filters, then the PCF derives the PCC rules and QoS information according to the AF request and and operator policies, and provides the information to the SMF within PCC rules enhanced to support the additional Packet Filter along with legacy packet filter.

5. The SMF may provide the received additional Packet Filter along with legacy packet filter to the UPF in the N4 rules. The PDI in PDR is extended to support the additional Packet Filter.

6.	The SMF provides QoS rules for the uplink traffic for the UE. QoS rules are enhanced to include additional Packet Filter when required (additional packet filter is not expected to be interpreted by a legacy UE; in case of legacy UE, it will use legacy packet filter). If supported by the UE, the UE uses both the legacy packet filter and the additional packet filter for QoS Flow mapping.

NOTE:	It is recommended that CT1 specifies additional packet filter in a backward compatibility manner so that legacy UE can match the XRM traffic to a QoS flow with the QoS requirement defined in r18 XRM.

# * * * * End of changes * * * *