SA WG2 Meeting #164	S2-2407682

Maastricht, NL, 18 - 23 August, 2024

Source:	vivo

Title:	Update to KI#4 conclusion

Document for:	Approval

Agenda Item: 	19.3.1

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract of the contribution: This paper proposes update of conclusion on KI#4.

1	Discussion

During SA2#163, there is an EN on whether there is a need for a new UE capability for network to provide the R19 QoS rule:

Editor's note:	Whether there is a need for a new UE capability indication for UL is FFS (e.g. for legacy UE handling and network handling).

For resolving this EN, there are two options:

Option-1: No new capability

Option-2: A new capability

To analyse the two options, the following aspects are discussed:

A). Legacy definition of packet filter

As per TS 24.501, a packet filter contains the following parts:

A packet filter = [packet filter direction][identifier 1][length of component 1][ component 1]...[identifier n][length of component n][ component n]

,wherein:

- packet filter direction: indicates the packet filter is for UL only, DL only, both DL and UL;

- packet filter content: identifier + length + content, wherein identifier indicates the content is IP address, port number, etc, and the content is the corresponding IP address, port number, etc.

For example, a packet filter can contain {direction, IP#1, Port#1}.

A QoS rule contains a list of packet filters. A QoS rule is considered to be matched if one of the packet filters in the list matched. A packet filter is considered to be matched only if all components of a packet filter match the data packet. That is, if a packet filter contains {UL, IP#1, Port#1}, and a data packet is {IP#1, Port#2}, then the data packet can NOT match this packet filter.

Observation 1: A packet filter is considered to be matched only if all components of a packet filter match the data packet.

A UE may be provisioned with multiple QoS rules. The QoS rule are matched one by one according to the precedence of the QoS rule.

It is highlighted that a single R19 new component can NOT be used separately, i.e. only SSRC can NOT identify a media flow. The R19 new components should be used along with legacy components (e.g. IP, Port, ...). For example, a R19 Packet filter = {direction, SSRC} can NOT be used to identify a media flow, and a R19 Packet filter = {direction, IP#1, Port#1,SSRC} can be used to identify a media flow.

If a QoS rule contains a component unknown to the legacy UE (i.e. receiving R19 packet filter for the legacy UE), the UE will request SMF to delete the unknown QoS rule (see below):

6.4.1.3 UE-requested PDU session establishment procedure accepted by the network ... the UE shall verify the authorized QoS rules and the authorized QoS flow descriptions provided in the PDU SESSION ESTABLISHMENT ACCEPT message for different types of errors as follows: ... d) Syntactical errors in packet filters: 1) When the rule operation is "Create new QoS rule" and two or more packet filters in the resultant QoS rule would have identical packet filter identifiers. 2) When there are other types of syntactical errors in the coding of packet filters, such as the use of a reserved value for a packet filter component identifier. If the QoS rule is the default QoS rule, the UE shall initiate a PDU session release procedure by sending a PDU SESSION RELEASE REQUEST message with 5GSM cause #45 "syntactical errors in packet filter(s)". Otherwise, the UE shall send a PDU SESSION MODIFICATION REQUEST message to delete the QoS rule with 5GSM cause #45 "syntactical errors in packet filter(s)".

Observation 2: From legacy UE perspective, the R19 additional packet filter (i.e. SSRC, PT) occupies the reserved value of packet filter component.

Observation 3: The legacy UE will request SMF to delete the unknown QoS rule containing R19 additional packet filter component (i.e. SSRC, PT,...). The UL traffic will be mapped to the default QoS flow which doesn’t support PDU Set QoS handling. i.e. the UL of r18 UE cannot use r18 PDU Set QoS handling when additional packet filter is applied.

For example, a packet flow contains two media flows

(Media flow-1 = IP#1, Port#1, SSRC#1;

Media flow-2 = IP#1, Port#1, SSRC#2).

The UE is provisioned with two QoS rules

(QoS rule -1 = IP#1, Port#1, SSRC#1;

QoS rule -2 = IP#1, Port#1, SSRC#2).

For legacy UE receiving the two QoS rules (QoS rule -1 = IP#1, Port#1, SSRC#1; QoS rule -2 = IP#1, Port#1, SSRC#2), the legacy UE will request SMF to remove the two QoS rules.

B). Whether a new capability is needed?

Option-1: No new capability.

To provide the additional packet filter to UE and avoid the UL of the UE fall back to default QoS flow, there are several options can be considered if not UE capability indication by r19 UE:

a)	Option 1-1: List of QoS rules containing:

-	Multiple R19 QoS rules (packet filter = {R18 (e.g., IP)+ R19 ( e.g. SSRC)}); and

-	A R18 QoS rule (i.e. packet filter = {R18 (e.g., IP)}).

-	The precedence of R19 QoS rules should be higher than R18 QoS rule.

In this option,

-	If UE supports R19 QoS rules, R19 QoS rule will always be used as the first precedence, and R18 QoS rule will not be used.

-	If UE NOT support R19 QoS rules, R19 QoS rule will be requested to be removed from the UE. Hence R18 QoS rule will be used as the first precedence.

Evaluation

-	Cons: SMF needs to additionally allocate a r18 QoS rule used for UL only.

b)	Option 1-2: List of QoS rules containing:

-	Multiple R19 QoS rules e.g.

QoS rule1 (packet filter 1= R18 (e.g., IP), packet filter 2= R19 (e.g., SSRC_1));

QoS rule2 (packet filter 1= R18 (e.g., IP), packet filter 2= R19 (e.g., SSRC_2));

QoS rule3 (packet filter 1= R18 (e.g., IP), packet filter 2= R19 (e.g., SSRC_3));

In this option,

-	If UE supports R19 QoS rules, UE can consider both packet filter 1 and packet filter 2 in the QoS rule.

-	If UE NOT support R19 QoS rules, R19 QoS rule will be requested to be removed from the UE. Hence R18 QoS rule will be used as the first precedence (supposed UE can ignore packet filter 2).

Evaluation,

Cons: The Legacy UE treat the following QoS rule with the same packet filter 1= R18 (e.g., IP). Although they are ugly configuration, the precedence of QoS rules can help to pick one, but the rest two QoS rules are not used and wastes the QoS rules resources.

- QoS rule1 (packet filter 1= R18 (e.g., IP), packet filter 2= R19 (e.g., SSRC_1))  QoS rule1 (packet filter 1= R18 (e.g., IP))

- QoS rule2 (packet filter 1= R18 (e.g., IP), packet filter 2= R19 (e.g., SSRC_2)) QoS rule2 (packet filter 1= R18 (e.g., IP))

- QoS rule3 (packet filter 1= R18 (e.g., IP), packet filter 2= R19 (e.g., SSRC_3))  QoS rule3 (packet filter 1= R18 (e.g., IP))

-	For a r19 UE, for one QoS rule, Option 1-2 requires to use two packet filters together for matching one data flow, which breaks the legacy mechanism.

Legacy UL QoS flow mapping only uses one packet filter (with multiple packet filter components), two packet filters used together need to introduce new match logics in the UE.

Besides, if more additional packet filter is introduced in the future release, obeying Option1-2 design, multiple packet filters are used together to filter a data flow, which always introduces new match logics, make the QoS flow mapping more and more complexity and the scalability becomes vulnerable.

b)	Option 1-3: List of QoS rules containing:

-	Multiple R19 QoS rules (packet filter = {R18 (e.g., IP)+ R19 ( e.g. SSRC)}); and

-	one R19 QoS rules (packet filter1 = {R18 (e.g., IP)+ R19 ( e.g. SSRC)}, packet filter2 = {R18 (e.g., IP)});

In this option,

-	If UE supports R19 QoS rules, R19 QoS rule will always be used as the first precedence, and R18 QoS rule will not be used.

-	If UE NOT support R19 QoS rules, R19 QoS rule will be requested to be removed from the UE (including R18 packet filter). Hence the UE will use default QoS rule, and a R18 UE can NOT ensure a PDU set QoS.

Evaluation,

-	Option 1-3 is NOT feasible as all R19 QoS rules will be requested to be removed by the legacy UE, leading to the result that the UL of legacy UE can only use default QoS rule.

Option-2: A new SM capability.

Proposal: Option 1-1 > Option 2

2	Proposal

It is proposed to include the following changes in TR 23.700-70 v1.0.0.

* * * * Start of Changes * * * *

8.4	Interim agreements for Key Issue #4

The following aspects are concluded as principles for normative work:

1.	The AF may provide QoS requirements for media flows and additional packet filters as part of "AF session with required QoS". The detectable media flows that require different QoS treatment are mapped to distinct QoS Flows. If the AF does not provide any QoS requirement for the media flows or it provides the same QoS requirement, then the media flows can be mapped to the same QoS Flow. For the multiplexed media flows which cannot be differentiated by using the legacy IP Packet Filter, the AF may provide additional Packet Filter information in the "AF session with required QoS" procedure to the NEF/PCF.

2.	In order to unique identify each media flow, the legacy packet filter component is extended to include the additional Packet Filter information. The additional Packet Filter information is used to differentiate the media flow among multiple media flows that share the same legacy packet filter both in uplink (in UE) and downlink (in UPF) in the following case:

a)	For the media flows that are transported in (S)RTP, (S)RTCP and other associated protocols that are multiplexed into a single UDP/IP traffic flow as specified in IETF RFCs 5761, 5764, 7983, 8872, 8843 and 9443. The additional Packet Filter information SHALL be the Synchronization Source (SSRC), Payload Type (PT) and RTP-M header fields.

3.	The PCC rule and N4 rules are enhanced to support the additional Packet Filter along with legacy packet filter.

4.	The SMF may provide the received additional Packet Filter information along with legacy packet filter to the UPF in the N4 rules. The PDI in PDR is extended to support the additional Packet Filter information.

5.	In order to make a legacy UE matching the XRM traffic to a QoS flow with the QoS defined in R18 XRM, the SMF configures a R18 QoS rule containing legacy packet filter with the legacy packet filter component for the QoS flow with PDU set QoS in addition to multiple QoS rules with the packet filter containing the additional packet filter component for identifying different media flows. The precedence of QoS rule with R19 packet filter is higher than QoS rule with the legacy packet filter. If supported by the UE, the UE uses the packet filter containing both the legacy packet filter component and the additional packet filter component for QoS Flow mapping.

NOTE:	It is recommended that CT1 specifies additional packet filter in a backward compatibility manner so that legacy UE can match the XRM traffic to a QoS flow with the QoS requirement defined in r18 XRM.

* * * * End of Changes * * * *