3GPP SA WG2 Meeting #164	S2-2407683

19-23 August 2024, Maastricht, NL

Title:	    [DRAFT] LS on supporting additional packet filter information in QoS rule

Response to:

Release:	Rel-19

Work Item:	FS_XRM Ph2

Source:	SA2

To:	CT1

Cc:

# Contact Person:

Name:	Xiaowan Ke

Tel. Number:

E-mail Address:	xiaowan(dot)ke(at)vivo(dot)com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 would like to introduce additional packet filter information to support "Traffic detection and QoS flow mapping for multiplexed data flows" in clause 8.4 of TR 23.700-70. The additional packet filter information (e.g. Synchronization Source (SSRC) as defined in RFC 3550) is used alone with existing packet filter components (e.g. IP, Port) to differentiate the media flow among multiple media flows which share the same IP 5-tuple.

SA2 used to discuss the following options to introduce the additional packet filter information:

- Option-1: Extend the packet filter component to include the additional packet filter information. In this case, only one packet filter is used to filter a data flow as the legacy, i.e. the packet filter containing both the existing packet filter components and additional packet filter information.

- Option-2: Introduce an additional new packet filter containing only the additional packet filter information in the QoS rule. In this case, two packet filters are used to filter a data flow, i.e. legacy packet filter (e.g. IP, Port) and another new packet filter (e.g. SSRC) together.

Some companies in SA2 believe that, in Option-1, Pre-R19 UE will request to remove the QoS rule with the packet filter containing the additional packet filter information. And some companies in SA2 believes that Option-2 violates the existing QoS flow mapping logic and doubts the benefit of the new logic.

SA2 kindly asks CT1 to provide feedback on the following aspects:

- Whether the above companies’ understanding is correct or not.

- How the additional packet filter information will be introduced in the QoS rule from CT1 perspective.

2. Actions:

CT1:

ACTION: 	SA2 kindly asks CT1 to provide feedback on above.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #165		14-18 October 2024	Hyderabad, IN

TSG-SA2 Meeting #166		18-22 November 2024	Orlando, FL, US