3GPP TSG-SA2 Meeting #164	S2-2407727

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0236 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1 Local offloading management policy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is concluded in KI#1 that, for Session Breakout model, the local I-SMF for local offloading management can obtain the local offloading management policy form the SMF. Accordingly, the SMF needs to receives the local offloading management policy from the PCF with indicating that the local offloading management is supported for a given PDU Session. Additionally, it is concluded that SMF may forward AF provided traffic influence information to I-SMF. The AF provided traffic influence information is provided as SM policy info from PCF to SMF, which sequentially provides to the local I-SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add policy information provisioning for Local Offloading Management |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Policy information cannot be provisioned to local I-SMF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

6.X.Y	Policy information provisioning for Local Offloading Management

For Local Offloading Management scenarios, the local I-SMF is configured with or receives the Local Offloading Management policy information from PCF via SMF during the procedure for establishing PDU Session supporting the Local Offloading Management. If the PCF receives a Local Offloading Management support indication from the SMF during the SM Policy Association procedure specified in clause of 4.16.4 of TS 23.502 [3], the PCF may provide Local Offloading Management Policy. The Local Offloading Management policy information contains IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN and shall select local UPF(s) as specified in clause 6.4 of TS 23.503 [4].

For Local Offloading Management scenario, the SMF may forward AF provided traffic influence information, which is received from PCF as part of SM policy, to the local I-SMF.

>>>> End of Changes <<<<