3GPP SA WG2 Meeting #164	S2-2407738

Maastricht, NL, August 19 - 23, 2024														(was S2-240xxxx)

Source:	Ericsson

Title:	KI#1, #2, #3, New Solution: Harmonized RAN-CN solution using AIOTF for Topology 2

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: The contribution discusses and proposes a solution using AIOTF for topology 2 with a harmonized RAN-CN interface for KI#1, 2 and 3.

1. Introduction

Ambient IoT devices are IoT devices powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g. using a capacitor). It can have, e.g., lower complexity, smaller size, reduced capabilities and lower power consumption than previously defined 3GPP IoT devices. The data rate of Ambient IoT devices is usually low.

This pCR proposes a solution using AIOTF for topology 2 with harmonized RAN-CN architecture. In this solution, AIOTF delivers device NAS message as non-UE associated signalling towards NG-RAN and NG-RAN forwards to intermediate UEs.

As solution#30, the device NAS layer is also maintained between AIOTF and AIoT device. However, in this solution, the device NAS message is delivered from AIOTF to NG-RAN over NGAP, and from NG-RAN to intermediate UE over Uu AS layer, instead of over UE AIOT NAS layer as in solution#30.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-13 v0.4.0:

* * * Start of Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | Key Issue #1 | Key Issue #2 | Key Issue #3 |
| #1 |  | X |  |
| #2 |  | X |  |
| #3 | X | X | X |
| #4 | X | X | X |
| #5 | X | X | X |
| #6 | X | X | X |
| #7 | X | X |  |
| #8 | X | X |  |
| #9 | X | X | X |
| #10 | X | X |  |
| #11 |  |  | X |
| #12 | X | X | X |
| #13 |  | X |  |
| #14 |  | X |  |
| #15 |  | X | X |
| #16 |  | X |  |
| #17 | X | X | X |
| #18 |  |  | X |
| #19 | X |  | X |
| #20 | X | X | X |
| #21 | X | X | X |
| #22 | X | X | X |
| #23 |  |  | X |
| #24 |  | X |  |
| #25 |  | X |  |
| #26 |  |  | X |
| #27 |  |  | X |
| #28 |  |  | X |
| #29 | X | X |  |
| #30 | X | X | X |
| #31 |  | X | X |
| #32 | X |  | X |
| #33 | X | X | X |
| #34 |  | X |  |
| #X | X | X | X |

* * * Next of Change (ALL TEXTS ARE NEW) * * * *

## 6.X	Solution #X: Harmonized RAN-CN solution using AIOTF for Topology 2

### 6.X.1	Description

This solution proposes a complementary solution to Solution #8. It enables the support of Ambient IoT devices for topology 2 with harmonized RAN-CN architecture, which addresses KI#1, KI#2 and KI#3. Together with Solution #8 which focus on Topology 1, the solutions can support AIoT devices for both Topologies.

In this solution, NG-RAN performs NG Setup towards AIOTF besides AMF. AIOTF is responsible for generating the device NAS message and delivering as non-UE associated signalling towards NG-RAN. It is NG-RAN who is in charge of the intermediate UE for the Ambient IoT operations, including selecting intermediate UEs (optionally based on the candidate UE list form the AIOTF), sending operation commands towards intermediate UEs and forwards the results from intermediate UEs towards the AIOTF. As the licensed spectrum is owned by MNO, it is proposed to let NG-RAN provide the radio resource information towards the intermediate UEs about the spectrum information for the over-the-air interface between Intermediate UEs and AIoT devices.

#### 6.X.1.1	Reference Architecture

The Figure 6.X.1.1-1 illustrates the architecture for harmonized RAN-CN solution using AIOTF for topology 2:

Figure 6.X.1.1-1: System Architecture of harmonized RAN-CN solution using AIOTF for Topology 2

This solution focuses on Topology 2, which works together with Solution #8 addressing Topology 1.

The functional entities defined in Solution #8 and TS 23.501 [4] are reused with the differences:

-	AIOTF: Distribute AIoT device NAS messages to NG-RAN.

-	NG-RAN: Determine Intermediate UEs, allocate radio resources for communication between Intermediate UEs and AIoT devices, forwards device NAS messages to Intermediate UEs, and forwards the response to AIOTF.

#### 6.X.1.2	Protocol Stack

The Figure 6.X.1.2-1 illustrates the protocol stack:

Figure 6.X.1.2-1: Protocol Stack for harmonized RAN-CN solution using AIOTF for Topology 2

As solution #8, within the protocol stack:

-	AIOT device NAS layer: The NAS protocol between AIOTF and AIoT devices.

-	AIOT AS layer: The AS protocol layers between UE reader and AIoT devices, including physical layer, MAC layer, etc.

-	App layer: The application layer protocol between AIoT devices and AF.

In NG-RAN, the AIoT service operation information received in NGAP will be mirrored to Uu AS layer.

### 6.X.2	Procedures

NOTE:	The message names in the procedures below are descriptive. It is assumed that the names are updated with corresponding SBI based names where applicable during the normative phase.

#### 6.X.2.1	NG Setup

The NG-RAN triggers NG Setup procedure not only towards the AMF, but also towards the AIOTF. Via the NG connection, the AIOTF delivers AIoT device NAS messages towards the NG-RAN, as well as receives AIoT device NAS messages from the NG-RAN.

#### 6.X.2.2	AIoT Service Authorization for Intermediate UE

The procedures for AIoT service authorization for Intermediate UEs are the same as described in clause 6.30.2.1 in solution #30.

To ensure the Intermediate UE is reachable when performing AIOT service operations:

-	The NG-RAN does not release the authorized Intermediate UEs to RRC_IDLE state (UEs can be released to RRC_INACTIVE when needed), or

- The NG-RAN may release the authorized Intermediate UEs to RRC_IDLE state. And in this case, the Intermediate UEs need to be paged back by CN before the AIoT service operation.

#### 6.X.2.3	AIOTF Keep Track of Intermediate UE

The procedures to enable AIOTF to keep track of Intermeidate UEs are the same as described in clause 6.30.2.2 in solution #30.

#### 6.X.2.4	Application Inventory

The Inventory procedure can be initiated by the AF to discover one or more AIoT devices in a specific area via Intermediate UEs.

Figure 6.X.2.4-1: Inventory Procedure

1.	AF sends Inventory Request to the AIOTF, which is the same as step 1 - 4 in Figure 6.8.2.1-1 in Solution #8.

2.	The AIOTF may generate the candidate UE list inside the intended area, if the NG-RAN may release the intermediate UEs to RRC_IDLE state.

3.	The AIOTF sends enable UE reachability request with the UE list towards the AMF to page the intermediate UEs inside the intended area. The AMF checks the CM states of those intermediate UEs, and triggers paging towards the NG-RAN(s) for those UEs in CM-IDLE state.

4.	The AIOTF discovers NG-RANs based on the intended area and sends Inventory Request towards the NG-RAN with the optional candidate UE list, area info, device info, inventory strategy and location required. The IDLE UEs which cannot be paged back are removed from candidate UE list generated in step 2.

5.	The NG-RAN checks whether the inventory request can be performed. The NG-RAN triggers RAN paging for the intermediate UEs in RRC_INACTIVE state and then selects intermediate UEs. If candidate UE list is provided by the AIOTF, the NG-RAN should select intermediate UEs within the candidate UE list. Otherwise, the NG-RAN can select among the authorized intermediate UEs.

6.	The NG-RAN determines radio resource for the AIoT service operation between the intermediate UEs and AIoT devices, and sends inventory request towards determined intermediate UEs, together with the determined radio resources.

NOTE 1:	It’s up to RAN2 to determine the Uu AS layer impacts for inventory request delivery.

7.	The Intermediate UE initiates inventory based on device information as well as the inventory strategy information provided by the AF. The Intermediate UE may provide reader identity information to enable the AIoT devices to understand they are read by which Intermediate UE. Considering the mobility of the Intermediate UE, the reader identity information can be a combination of a UE ID and the location information or the RAN ID that serves the Intermediate UE.

8.	The AIoT Device reports the device ID in an AIOT device NAS message to the Intermediate UE. If the Inventory procedure indicates delta inventory only, and the AIoT Device has performed the inventory procedure towards this UE reader in the same location or served by the same RAN node, it should skip the reporting. The device capability index can be provided by AIoT device optionally.

9.	The Intermediate UE forwards AIOT device NAS message towards the NG-RAN, including the AIoT Device ID and optional device capability index. The NG-RAN forwards the AIoT device NAS message towards the AIOTF.

The Intermediate UE may further provide location information (ULI) of the device, if requested from the AIOTF and allowed by local policy. The Intermediate UE may further provide an end indicator to inform the NG-RAN that will inform AIOTF whether it is the last inventory response for the inventory round, e.g. based on timeout in Intermediate UE.

NOTE 2:	It's up to RAN WG3 to determine the details and enhancements of the location information, and the ability to provide an end indicator from a reader is assumed to be aligned between UE as reader and RAN as reader.

10.	The AIOTF validates the concealed device ID via interacting with AUSF and UDM. The AIOTF may further get the device capability information from the device capability profile stored in the UDM based on the device capability index provided by the device.

11.	The AIOTF together with AUSF and UDM, triggers authentication and authorization procedures (i.e. like the authentication request/response between UE and network) towards the AIoT device. In Topology 2, the message between the AIOTF and the AIOT device is transmitted via the NG-RAN and the Intermediate UE.

Based on the device capability information from UDM, if the AIoT device is capable of storing received parameters for a longer period, step 12 - step 14 are executed i.e. the AIOT device is then considered registered:

12.	The security mode negotiation and security parameter exchanges are performed (i.e. like the security mode command/complete between UE and network). In Topology 2, the message between the AIOTF and the AIOT device is transmitted via the NG-RAN and the Intermediate UE.

NOTE 3:	Further details of the security procedures are to be defined by SA WG3.

13.	The AIOTF may further allocate CN device ID and sends it to the AIoT device.

14.	The AIOTF registers with the UDM (UECM) for the AIoT device access.

15.	The AIOTF may further aggregate the reported device IDs and send them to AF via the NEF, as step 14 - 16 in Figure 6.8.2.1-1.

Compared with solution#30, this solution is different in step 2-6 and step 9:  the AIOTF pages candidate intermediate UEs in the intended area and delivers the device NAS message over NGAP (e.g., inventory request) towards NG-RANs in the intended area. NG-RAN determines intermediate UEs inside the area and delivers the device NAS message over Uu AS layer (e.g. inventory request) towards intermediate UEs. The device response is sent from intermediate UEs to NG-RAN and forwarded to AIOTF. In step 11-13, the device NAS message delivery follows the same principle.

#### 6.X.2.5	Periodic Inventory Procedure

The periodic inventory may be requested by the AF, so that network can trigger the inventory periodically without the request from the AF directly.

Considering the movement of the Intermediate UEs which are not fixed, and the inventory is based on area information, the periodic inventory should be initiated by the CN (i.e., the AIOTF), which follows the instructions from the AF. When the period expires in the AIOTF, the AIOTF pages the Intermediate UEs based on the area information from the AF and the location information for the Intermediate UEs, and then triggers inventory procedure, as step 2-15 in Figure 6.X.2.4-1.

#### 6.X.2.6	Command Procedure

The command procedure is initiated by the AF to request one or more specific AIoT devices or a group of AIoT devices in an area to execute the command. The command can be read, write, enable, disable, or execution request from the AF. The command result from the devices may or may not be required.

Figure 6.X.2.6-1: Command Procedure

1.	The AF sends Command Request to the NEF, containing the command to be executed, area information, device information, optional inventory strategy information, and optional report aggregation info.

-	The command is to be executed in the AIoT devices. It contains the command (e.g. read, write, execute, etc.) together with the command parameters. The command specific parameters (e.g. what to be executed) are to be included in an app layer container between AF and AIOT devices.

-	The area information could be the external geographical area information.

-	The device information could be device ID, device group ID, and/or external device type.

-	The location required indicates whether the AF requests the location information of the AIoT devices provided.

-	The report aggregation info indicates whether the reports need to be aggregated or not for a specific aggregation period, and whether the reports are needed after the aggregation period.

2.	The NEF authorizes the request from the AF and perform the area translation to translate external area information to the internal area information. Within the authorization, the NEF further check whether the AF is authorized to get the location information of the device. The NEF map the external device type to internal device type (i.e. device 1, device 2a, device 2b, etc.).

NOTE 1:	It is up to RAN to determine whether the device type is useful or not e.g. to optimize AS functionality such as spectrum usage, number of repetitions, etc.

Depending on whether area information is provided, step 3a or step 3b is performed:

3a.	If area information is provided, the NEF sends NRF query with internal area information to query AIOTFs serving the area.

3b.	If area information is not provided, and device ID(s) are provided, the NEF query the serving AIOTF from UDM.

4.	The NEF sends the Command Request to the AIOTFs with the command, internal area information, device information, device type, and optional location required information.

5.	The AIOTF allocates a Transaction ID for the command, which is unique per command. The AIOTF creates the AIOT Device NAS request message for the devices, based on the command information from the AF. The AIOTF get the registered devices, which match the device information. If no registered devices, step 6 - 14 are skipped.

6.	The AIOTF sends enable UE reachability request towards the AMF to page the intermediate UEs for the registered devices, if the NG-RAN may release the the intermediate UEs to RRC_IDLE state. The AMF check the CM states of those intermediate UEs, and triggers paging towards the NG-RAN(s) for those UEs in CM-IDLE state.

7.	The AIOTF creates an AIOT device NAS request message containing the Command. The AIOTF sends the NG-RAN with the AIOT device NAS request message, registered devices together with UEs, and the Transaction ID.

8.	The NG-RAN checks whether the command request can be performed. The NG-RAN performs RAN paging for the intermediate UEs in RRC_INACTIVE state. The NG-RAN determines radio resource for the AIoT service operation between Intermediate UEs and the AIoT devices, and sends the Command Request towards the Intermediate UEs for the registered devices with the AIOT device NAS message, radio resource information, registered devices, and the Transaction ID.

NOTE 2:	It’s up to RAN2 to determine the Uu AS layer impacts for command request delivery.

9.	The Intermediate UE initiates command delivery to transfer the AIOT device NAS request messages based on CN allocated device IDs of the registered devices, using dedicated signalling. Transaction ID is also included.

NOTE 3:	RAN signalling details, including how the command is sent, are to be further studied by RAN WGs.

10.	The AIoT Device executes the command, creates an AIOT device NAS response message containing the command result and delivers the AIOT device NAS response message, if the result needs to be sent back. The application specific result can be included in an app layer container.

11.	The Intermediate UE forwards the AIOT device NAS response message to the NG-RAN, and the NG-RAN forwards to the AIOTF. The Intermediate UE may further provide location information (ULI) of the device, if requested from the AIOTF and allowed by local policy.

NOTE 4:	It's up to RAN WG3 to determine the details and enhancements of the location information.

12.	The AIOTF gets the command result from the AIOT Device NAS response message. The AIOTF may perform aggregation, based on the report aggregation information provided by the AF. Within the aggregation period, the AIOTF will buffer the command results reported from the AIoT devices. The AIOTF may stop buffering and send report immediately, if it receives end indicator from the Intermediate UE in step 11. When the aggregation period expires, the AIOTF sends the report. For those command result after the aggregation period, if it is needed by the AF, the AIOTF sends the report. Otherwise, it will be dropped.

13.	The AIOTF sends Command Response or Notification Request towards the NEF for the command result or the aggregated command result information.

14.	The NEF sends Command Response or Notification Request towards the AF for the command result or the aggregated command result information.

If the AIOTF determines all required devices have executed the command (e.g. when AF provide the device ID(s) and those devices have executed the command in step 9 - 10), the following steps are skipped.

15.	For those unregistered devices(s), the AIOTF initiate Inventory procedure as step 2 - 14 in Figure 6.X.2.4-1 (step 12 - 13 will be skipped, if the device is not able to persistent received information). In the inventory the Transaction ID is delivered, so that the registered devices which have executed command do not respond this inventory request from the Intermediate UE.

16.	The AIOTF creates an AIOT device NAS request message containing the Command. The AIOTF sends the NG-RAN with the AIOT device NAS request message, the respond device together with the serving UE.

17.	The NG-RAN checks whether the command request can be performed. The NG-RAN determines radio resource which is used between AIoT Devices and Intermediate UE. The NG-RAN sends the AIOT device NAS request message and the respond device.

18.	The Intermediate UE initiates command delivery for the AIOT Device NAS request message to the device.

19.	The AIoT Device executes the command, creates an AIOT Device NAS response message containing the command result and delivers the AIOT Device NAS response message to the intermediate UE, if the result needs to be sent back. The application specific result can be included in an app layer container.

20. The Intermediate UE forwards the AIOT device NAS response message to the NG-RAN, and the NG-RAN forwards to the AIOTF. The Intermediate UE may further provide location information (ULI) of the device, if requested from the AIOTF and allowed by local policy.

21.	As step 13 - 14, the AIOTF get the command result from the AIOT Device NAS response message. It may perform the aggregation and send to NEF. The NEF delivers to the AF.

Compared with solution#30, this solution is different in step 6-8 and step 11:  the AIOTF pages intermediate UEs and delivers the device NAS message over NGAP (i.e., command request) towards NG-RANs. NG-RAN delivers the device NAS message over Uu AS layer (e.g., command request) towards intermediate UEs. The device response is sent from intermediate UEs to NG-RAN and forwarded to AIOTF. In step 16-17 and step 20, the device NAS message delivery follows the same principle.

### 6.X.3	Impacts on services, entities and interfaces

The following NFs are impacted on top of the impacts of solution #8:

-	NG-RAN:

-	NGAP association with AIOTF.

-	Determine radio resources between Intermediate UE and AIoT devices.

-	Receive request from AIOTF, (optional) determine Intermediate UEs, forward to Intermediate UEs.

-	Receive response from Intermediate UEs and forward to AIOTF.

-	Keep Intermediate UEs in RRC_CONNECTED or RRC_INACTIVE state (optional)

-	Ambient IoT Function (AIOTF):

-	NG-RAN connectivity.

-	Routing the request from AF (via NEF) to NG-RAN, for DO-DTT/DT traffic types.

-	Routing the response from NG-RAN to AF (via NEF) for DO-DTT traffic type.

-	Page Intermediate UEs via AMF (optional)

-	AMF: Authorization of Intermediate UEs and create UE context towards AIOTF.

-	UDM: Enhance authorization information for Intermediate UEs.

-	Intermediate UE:

-	Authorization of Intermediate UEs

-	Receive the request and radio resource information from NG-RAN and perform Ambient IoT operations (e.g. inventory, command, etc.) on the proper spectrum.

-	Receive responses from AIoT devices and send to NG-RAN.

* * * End of Change * * * *