SA WG2 Meeting #164	S2-2407751

Maastricht, Netherlands, 19 August – 23 August, 2024	(revision of S2-24xxxxx)

Source:	LG Electronics

Title:	KI#3: EN resolution about UAV not supporting Rel-19 NTZ mechanism

Document for:	Approval

Agenda Item:	19.10.1

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract of the contribution: This paper proposes to resolve EN in KI#3 description.

Discussion

TR 23.700-59v1.0.0 includes the following EN in clause 5.3.1 for KI#3 description that needs to be resolved.

Editor's note:	What is requirement to 3GPP system with regard to aerial UEs not supporting 3GPP Rel-19 NTZ mechanism and how to handle these aerial UEs need to be addressed during the study and conclusion of KI#3 and coordination with RAN WGs.

Pre-SA2#164 NWM Discussion for KI#3 was progressed including the following question.

Question#3-7: "Editor’s note: What is requirement to 3GPP system with regard to aerial UEs not supporting 3GPP Rel-19 NTZ mechanism and how to handle these aerial UEs need to be addressed during the study and conclusion of KI#3 and coordination with RAN WGs." in clause 5.3.1 needs to be resolved/finalized. Do you agree that SA2 can assume that aerial UEs not supporting 3GPP Rel-19 NTZ mechanism are capable of complying with NTZ regulation using methods not provided by 3GPP (e.g. Application layer configuration) ? If you cannot agree with the above assumption, please provide what kind of NTZ enforcement by NW is needed for these aerial UEs.

Observation: According to feedback provided by interested companies, it is observed that no majority view can be identified because some companies agreed with the above assumption while some companies commented that reject from network is needed for non-supporting UAV UE in NTZ.

Proposal: It is proposed that SA2 is handling this issue during normative phase to decide how to describe/resolve aspect on UAV UEs not supporting 3GPP Rel-19 NTZ mechanism, for example,

1)	"UAV UEs not supporting 3GPP Rel-19 NTZ mechanism(s) are assumed to comply with NTZ regulation using methods outside of 3GPP scope (e.g. application layer configuration)." can be described; or

2)	any existing mechanisms to reject UAV UEs not supporting 3GPP Rel-19 NTZ mechanism in NTZ by core network (e.g. Registration Reject by AMF, Attach/TAU Reject by MME) can be described.

Capturing both 1) and 2) can be also considered, e.g., 1) as NOTE, 2) as informative annex.

Proposal

It is proposed to agree the following changes into TR 23.700-59.

* * * * Start of 1st Change * * * *

5.3	Key Issue #3: Support of No Transmit Zones

5.3.1	Description

This key issue relates to the introduction by CEPT ECC Decision 22(07) [3] of No Transmit Zones for aerial UEs. The ECC Decision asserts that a mechanism is necessary to ensure that aerial UEs respect no-transmit zones in order to protect incumbent radio systems from potential interference from aerial UEs.

Since the ECC Decision does not identify any specific RAT, NTZs can be supported by both LTE and NR.

This key issue addresses the following aspects:

-	How to ensure an aerial UE respects no-transmit zones, including:

-	whether a mobile network cells overlapping completely or partially with the NTZ and using the restricted frequency bands of the NTZ;

-	whether mechanisms are needed to differentiate aerial UEs that support functions defined for NTZs in Rel-19 and aerial UEs that don't;

-	what if any, specific aerial UE behaviour when the aerial UE approaches, enters, or exits the NTZ.

-	Whether and how to enable configuration of NTZ information in the aerial UE.

-	Whether to allow the enforcement of no-transmit zone(s) for both aerial UEs in connected mode and aerial UEs in idle mode and if yes then how.

NOTE:	Any potential solutions developed shall be coordinated with RAN WGs or progressed together with RAN WGs input.

For aerial UEs supporting NTZ regulations, it is assumed that they are configured with NTZ information, so that these aerial UEs will be aware of the presence of NTZs.

* * * * Start of Next Change * * * *

8.3.X	Aspect on UAV UEs not supporting 3GPP Rel-19 NTZ mechanism

NOTE:	How to describe/resolve aspect on UAV UEs not supporting 3GPP Rel-19 NTZ mechanism will be decided during normative phase, for example:

1)	"UAV UEs not supporting 3GPP Rel-19 NTZ mechanism(s) are assumed to comply with NTZ regulation using methods outside of 3GPP scope (e.g. application layer configuration)." can be described; or

2)	any existing mechanisms to reject UAV UEs not supporting 3GPP Rel-19 NTZ mechanism in NTZ by core network (e.g. Registration Reject by AMF, Attach/TAU Reject by MME) can be described.

Capturing both 1) and 2) can be also considered, e.g., 1) as NOTE, 2) as informative annex.

* * * * End of Changes * * * *