3GPP TSG-SA2 Meeting #164	S2-2407755

# 19 - 23 August, 2024, Maastricht, Netherlands									revision of S2-xxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1133 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of VFL model training with successive VFL client information sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The conclusions for VFL training documented in clause 8.2 of TR 23.700-84, including: P#2.4.5: VFL clients may also provide intermediate results (e.g. gradient information, loss information) to other VFL clients as instructed by the VFL server . During VFL training, the VFL server and clients will coordinate to perform forward computation and backwards propagation. Coordination between VFL clients to share intermediate results are supported by multiple VFL algorithms, e.g. split VFL. Rather than only deploying the VFL server to aggregate all the intermediate results from a huge number of VFL clients, the intermediate results shared between VFL clients can reduce the computational load of the server and also improve the VFL computation efficiency. This CR specifies the procedures for VFL model training with successive VFL client information sharing by invoking NEF and NWDAF service operations. New NEF services are introduced to deliver model training request, intermediate results, and model training termination request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce procedure for training with successive VFL client information sharing with detailing the NEF and NWDAF service operations and the parameters included in the services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - VFL cannot be fully supported by 5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 6.2x, new 6.2x.1, new 6.2x.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes (new text) * * *

## 6.2X	Vertical Federated Learning

### 6.2X.1	General

This clause specifies support for VFL in the 5GC.

### 6.2X.2	Procedures for VFL training

The procedure in Figure 6.2x.2-1 to support VFL model training with AF or NWDAF acting as the VFL server and successive VFL client information sharing is described step by step below.

Figure 6.2x.2-1: Procedure for VFL training with successive VFL client information sharing

0.	VFL server (i.e. AF or NWDAF) and VFL clients (i.e. NWDAF) register to NRF. The registration may include their NF profiles, Analytics ID(s), Address information of NWDAF, Service Area, VFL capability type information (i.e. VFL server and VFL client type), VFL client coordination capability, VFL computational capability (i.e. acting as VFL server and/or VFL client ), and Time interval supporting VFL. The latter parameter can be the same as Time interval supporting FL described in clause 5.2.

The VFL server and clients are discovered via NRF by invoking the Nnrf_NFDiscovery_Request service operation. During the discovery, the VFL server may include the requirements on the VFL clients and server in the discovery request, e.g. the location of the VFL clients and server, the (minimum) available capacity of the VFL clients and server, VFL client coordination capability.

NOTE 1:	The initial selection of VFL clients by the VFL server may happen in step 0. The selection of VFL clients by the VFL server may also be finalized in step 2.

NOTE 2:	The details of sample and/or feature alignment are out of scope of this procedure.

1.	The VFL server determines to initiate the VFL model training based on its internal logic and sends a VFL preparation request to the VFL client NWDAF(s) (via NEF if the VFL server AF is untrusted AF).

If the VFL server is trusted AF or NWDAF, the VFL server invokes Nnwdaf_MLModelTrainingInfo_Request or Nnwdaf_MLModelTraining_Subscribe service operation towards the VFL client NWDAF(s).

For untrusted VFL server AF, the AF invokes new service operation Nnef_MLModelTrainingInfo_Request or Nnef_MLModelTraining_Subscribe request towards the NEF. Then the NEF forwards the model preparation request to the corresponding VFL client NWDAF(s) by invoking Nnwdaf_MLModelTrainingInfo_Request or Nnwdaf_MLModelTraining_Subscribe service operation.

Editor's note:	The details of the new NEF service operations, including Nnef_MLModelTrainingInfo_Request, Nnef_MLModelTraining_Subscribe, Nnef_MLModelTraining_Notify, Nnef_MLModelTraining_Unsubscribe, etc., are FFS.

The VFL server may include ML Preparation Flag to check if the VFL clients can meet the ML model training requirements in the VFL preparation request. The VFL server may also include Analytics ID and VFL process ID, the target samples (e.g. UE ID, application ID, etc.), optional target features, ML Model Interoperability information, Available data requirement, Availability time requirement, required NWDAF capacity for the VFL process etc.

The VFL clients may respond to the VFL server by indicating whether they will join the VFL operation and may include the reason in the response message if it cannot join the VFL operation, e.g. due to no sufficient  capacity to perform VFL model training, not able to coordinate with other VFL client NWDAF(s), etc.

2.	The VFL server may performed VFL client selection or refinement based on the responses received by from the VFL clients.

3.	The VFL requests the VFL clients to start the VFL Model training process where intermediate training results are shared and coordinated by the VFL server, facilitating a collaborative approach to model refinement across the VFL clients.

The VFL server may trigger the VFL training by invoking a ML Model Training request towards VFL client #1 (NWDAF) (via NEF if the AF is untrusted).

3a.	For untrusted VFL server AF, the AF invokes new service operation Nnef_MLModelTraining_Subscribe request or Nnef_MLModelTrainingInfo_Request towards the NEF for VFL training request. Then the NEF forwards the received VEL training request to the selected VFL client NWDAF(s) by invoking Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service operation.

3b.	If the VFL server is trusted AF or NWDAF, the VFL server invokes Nnwdaf_MLModelTraining_Subscribe  request or Nnwdaf_MLModelTrainingInfo_Request towards the selected VFL client NWDAF(s).

The VFL server may indicate the Analytics ID and a VFL process ID associated to the VFL training, target samples (e.g. UE ID, application ID, etc.), optional target features, ML Model Interoperability information, Available data requirement, Availability time requirement, the VFL clients that participate the VFL process, the order of VFL clients for performing local model computation to the VFL clients.

Editor's note:	The complete list of parameters to support the successive VFL client information sharing is FFS.

4a.	VFL client #1 (NWDAF) triggers the local model computation to calculate the intermediate results. The VFL client may collect data for model training if the local data is not sufficient.

4b.	VFL client#1 notifies the intermediate results to the next VFL client #2 based on the order configured by the VFL server , by invoking the Nnwdaf_MLModelTrainingInfo_Request or Nnwdaf_MLModelTraining_Subscribe service operation.

VFL client#1 may indicate the remaining VFL clients that participate the VFL training process, the order of the remaining VFL clients for performing local model computation and other information as detailed in Step 3 to VFL client #2 (NWDAF).

Alternatively, the full list of the VFL clients that participate the VFL training process and the order of the VFL clients to perform VFL training might be indicated to each client by the server, e.g. in step 1 or in parallel with step 3, And. In this case, each VFL client can work out the next VFL client to interact with based on the VFL client list, the order, and the ID of the previous VFL client.

4c – 4e. Repeat Step 4a and 4b until the VFL training procedure propagates to the last VFL client based on the order configured by the VFL server.

5.	The VFL client #N NWDAF notifies the intermediate results to the VFL server.

5a.	If the VFL client is untrusted AF, the VFL client #N NWDAF notifies the intermediate results to the VFL server via NEF by invoking new service operation Nnef_MLModelTrainingInfo_Request response or Nnef_MLModelTraining_Notify based on the service operation used in Step 3. The NEF forwards the intermediate results to the VFL server by invoking Nnwdaf_MLModelTrainingInfo_Request response or Nnwdaf_MLModelTraining_Notify service operation based on the service operation received from VFL client.

5b.	If the VFL client is trusted AF or NWDAF, the VFL client #N invokes Nnwdaf_MLModelTrainingInfo_Request response or Nnwdaf_MLModelTraining_Notify based on the service operation used in Step 3.

6.	The VFL server performs further VFL computation and aggregation for local model (e.g. calculates the loss using the labels) by using the intermediate results received from VFL client #N. The VFL server updates its local model.

7.	In the subsequent iteration, the VFL server triggers the backwards propagation VFL computation for model update and refinement in each successive VFL clients, in the reversed order of VFL forward computation in step 4a-4e. VFL server sends VFL model refinement request to the last client in the previous iteration, e.g. VFL client #N.

7a.	 If the VFL client is untrusted AF, the VFL server invokes new service operation Nnef_MLModelTraining_Notify towards NEF. Then the NEF forwards the request to the VFL client by invoking Nnwdaf_MLModelTraining_Notify.

7b.	If the VFL client is trusted AF or NWDAF, the VFL server invokes Nnwdaf_MLModelTraining_Notify service operation toward VFL client #N.

The VFL server may include the intermediate results (e.g. the loss which will be used by the VFL client to compute gradient for model refinement and update), and optionally the VFL clients that participate in the VFL training process and the order of VFL clients for performing local computation for model refinement to the VFL client #N.

NOTE 3:	the order of VFL clients to perform model refinement in step 8a-8e might be reserved order of forward computation in step 4a-4e. The VFL clients that participate in the forward computation and backward propagation are the same. In this case, the VFL clients may derive the order of the backward propagation based on the ID of the previous VFL client.

8a.	VFL client #N performs VFL local computation to refine the local model based on the intermediate results provided by the VFL server.

8b.	VFL client #N deliveries the model refinement results to the subsequent VFL client based on the order configured by the VFL server or derived by itself by invoking Nnwdaf_MLModelTraining_Notify.

8c. – 8e. Repeat Step 8a and 8b until the VFL local computation for model refinement procedure propagates to the VFL client which is the first VFL client to perform the forward computation in step 4a, e.g. VFL client #1.

9.	VFL client #1 notifies the model refinement results to the VFL server.

9a.	 If the VFL client is untrusted AF, the VFL client #1 NWDAF notifies the intermediate results to the VFL server via NEF by invoking new service operation Nnef_MLModelTraining_Notify. The NEF forwards the model refinement response to the VFL server by invoking Nnwdaf_MLModelTraining_Notify service operation.

9b.	If the VFL client is trusted AF or NWDAF, VFL client #1 invokes Nnwdaf_MLModelTraining_Notify.

10.	The VFL server performs further VFL computation and model to update local model by using the intermediate results received from VFL client #1.

Steps 3-10 will be repeated until the VFL server determines to terminate the VFL model training.

11.	The VFL server determines to terminate the VFL model training process based on its internal logic, e.g. if the model is converged.

If the VFL server is untrusted AF, the VFL server invokes new service operation Nnef_MLModelTraining_Unsubscribe service operation towards NEF, and then the NEF forwards the request to the corresponding VFL client(s) by invoking Nnwdaf_MLModelTraining_Unsubscribe service operation.

If the VFL server is trusted AF or NWDAF , the VFL server sends VFL model training termination request by invoking Nnwdaf_MLModelTraining_Unsubscribe service operation toward NWDAF.

Step 11 may also happen after Step 6. In this case, Step 7-10 will be skipped.

Editor's note:	The VFL inference procedure with successive VFL client information sharing is FFS.

* * *End of Changes * * *