3GPP TSG WG2 Meeting #164	S2-2407768

19 - 23 August 2024, Maastricht, Netherlands	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5454 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Handling of Headers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Vodafone, ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The specification of KI#3 outcome Enhancement of UPF handling of headers according to approved WID: SP-240995 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes to incorporate the agreed enhancements for KI#3 as in WID, as follows: - Enhance Nnef_TrafficInfluence service to accommodate requests from AFs for handling of Headers/Tags according to Table 6.1.2.1-1 from TR 23.700-63. - Enhance the procedure in clause 4.3.6 of TS 23.502 to provision the handling information to the SMF/UPF - A new data subset in the Application data set is provisioned in the UDR. - NEF, PCF, SMF and UPF are enhanced to support the handling of Headers/Tags - UPF profile in NRF is enhanced with the new UPF capability to support the corresponding UPF selection and discovery - Enhancing Nupf_EventExposure for Header/Tag reporting. - Enhancing N4 PFCP rules to instruct reporting of headers/tags handling upon AF request according to option 2 in TR 23.700-63. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Handling of Headers Enhancements are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.x (new), 5.6.x.1 (new), 5.6.x.2 (new), 5.8.2.8.x (new), 5.8.5.6, 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.503 CR 4877, TS 23.502 CR 1329 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change (all new) * * * *

### 5.6.x	Support for Handling of Headers/Tags

#### 5.6.x.1	General

Handling of Headers/Tags is an optional feature that allows exchange of information in-band in the traffic path between the application (in the UE or in the application server) and the 5GS.

Handling of Headers/Tags enhances Header Enrichment (see clause 5.8.5.6) by allowing more actions than just insertion, that can be applied in uplink and in downlink directions. Handling of Headers/Tags is available for request by the AF, and it supports Header/Tag handling related notifications e.g. upon detection of a header/tag.

The content of this clause applies to non-roaming and to Home Routed roaming scenario, i.e. to cases where the involved entities (AF, PCF, SMF, UPF) belong to the Home PLMN and the AF has an agreement with the Home PLMN.

The PCF controls handling of Headers/Tags in UPF by provisioning header handling information in the PCC Rules as described in clause 6.3.1 of TS 23.503 [45]. The PCF derives the header handling information in the PCC rule as described in 6.1.3.x of TS 23.503 [45] based on the AF request. SMF instructs UPF by means of FAR (see clause 5.8.5.6).

The UPF performs Handling of Headers/Tags (i.e. detection, insertion, replacement, or removal of headers/tags) and notifies the UP Handling of Header/Tag events to SMF, to NEF/AF or to both as described in clause 5.8.2.8.x.

How the handling of Headers/Tags is performed in UPF is implementation specific. As an example, UPF can use user plane encapsulation protocols that can be defined by other standard organizations.

#### 5.6.x.2	Application Function influence on Handling of Headers/Tags

It is assumed that a service level agreement (SLA) exists between the operator and a third party. The mechanism(s) (e.g. protocol layer, type of encryption, etc.) to exchange information in-band in the traffic path using handling of headers/tags is agreed as part of the SLA. The reference to a system pre-configuration(s) corresponding to the mechanism to be applied for the AF request is also part of the SLA. The AF is assumed to have this reference and to be able to include it in the AF requests.

In the non-roaming scenario, Handling of Headers/Tags, and Application Function influence on Service Function Chaining (as defined in clause 5.6.16) and Application Function influence on traffic routing (as defined in clause 5.6.7) can be applied simultaneously.

In the Home Routed roaming scenario, Handling of Headers/Tags and Application Function influence on Service Function Chaining (as defined in clause 5.6.16) can be applied simultaneously.

The AF requests are sent to the PCF via N5 (in the case of requests targeting specific on-going PDU Sessions of individual UE(s), for an AF allowed to interact directly with the 5GC NFs) or via the NEF. The AF requests that target existing or future PDU Sessions of multiple UE(s) or of any UE are sent via the NEF and may target multiple PCF(s), as described in clause 6.3.7.2. The PCF(s) transform(s) the AF requests into policies that apply to PDU Sessions. When the AF has subscribed to UP Handling of Header event notifications, such notifications are sent by UPF either directly to the AF or via an NEF.

When the request is sent via NEF, Nnef_TrafficInfluence service is used. The procedures for AF to request Handling of Headers/Tags are described in clause 4.3.6 in TS 23.502 [3]. For AF interacting with PCF directly or via NEF, the AF requests may contain the information as described in the Table 5.6.x.2-1:

Table 5.6.x.2-1: Information element contained in AF request

| Information Name | Applicable for PCF or NEF (NOTE 1) | Applicable for NEF only | Category |
| --- | --- | --- | --- |
| Traffic Description | Defines the target traffic on which to apply handling of headers, represented by the combination of DNN and optionally S-NSSAI and application identifier or traffic filtering information. | The target traffic can be represented by AF-Service-Identifier, or by the combination of DNN and optionally S-NSSAI. | Mandatory |
| Target UE Identifier(s) | Indicates the UE(s) that the request is targeting, i.e. an individual UE, a group of UE represented by Internal Group Identifier(s) (NOTE 2), or any UE accessing the combination of DNN, S-NSSAI | GPSI can be applied to identify the individual UE, or External Group Identifier can be applied to identify a group of UEs. | Mandatory |
| Spatial Validity Condition | Indicates that the request applies only to the traffic of UE(s) located in the specified location, represented by areas of validity. | The specified location can be represented by geographical area. | Optional |
| Temporal Validity Condition | Time interval(s) when the request applies or duration(s). | N/A | Optional |
| AF transaction identifier | The AF transaction identifier refers to the AF request. | N/A | Mandatory |
| Handling of Headers Control information (UL/DL) |  |  |  |
| Header/Tag Handling Reference | A reference that points to predefined configuration which defines in UPF how to perform header handling for this request (e.g. the protocol). | N/A | Mandatory |
| Header/Tag Handling Additional Data | Additional data to complement the UPF predefined configuration for the Header Handling Reference. (NOTE 3) | N/A | Optional |
| Header/Tag Handling Action(s) (NOTE 4) | The header handling action(s) requested on the target traffic. | N/A | Optional |
| Header/Tag Handling Condition (NOTE 4) | It may be included to indicate if a requested action is to be performed always or once. | N/A | Optional |
| Header/Tag Handling Direction (NOTE 4) | Indicates if the action applies to UL or DL direction. | N/A | Optional |
| Header/Tag Information (NOTE 4) | AF input to construct the headers to perform the Header Handling Action(s). | N/A | Optional |
| Information on AF subscription to UP Handling of Header event | Indicates whether the AF subscribes to reporting of handling of header/tag execution and the parameters of this subscription. | N/A | Optional |
| NOTE 1: When the AF request targets existing or future PDU Sessions of multiple UE(s) or of any UE and is sent via the NEF, as described in clause 6.3.7.2, the information is stored in the UDR by the NEF and notified to the PCF by the UDR. NOTE 2: Internal Group ID can only be used by an AF controlled by the operator and only towards PCF. NOTE 3: The NEF, PCF and SMF do not need to understand the Header/Tag Handling Additional Data. NOTE 4: This information element is not needed if the predefined configuration for the Header/Tag Handling Reference includes that information. |  |  |  |

For each information element mentioned above as part of Handling of Header control information in the AF request, a detailed description follows:

1)	Header/Tag Handling Reference:

Reference agreed as part of the SLA that points to a predefined configuration in UPF which defines the header/tag handling mechanism (e.g. protocol layer, type of encryption, etc) and how the Header/Tag Information is used to construct the header(s)/tag(s). The predefined configuration may also include Header Handling Actions and Conditions (i.e. the requested actions are implicit in the Header Handling Reference that AF provides).

2)	Header/Tag Handling Additional Data:

This additional data is not standardised, but it can be interpreted by UPF based on SLA. It is sent transparently by NEF, PCF and SMF to UPF. In UPF, it complements the UPF predefined configuration for the Header Handling Reference.

3)	Header/Tag Handling Action (s):

The action(s) that AF may request on the target traffic. One or more of the following can be requested:

- Detect. It is used to request detection of certain header/tag. Detect only considers the header/tag name. An AF/NF can subscribe to UP Handling of Header/Tag event to be notified upon detection. The report includes any value associated with the header/tag.

- Remove. It can be used to request removal of certain header. Remove only considers the header/tag name.

- Replace. It can be used to request replacement of certain header value. The header/tag value to be replaced is detected by the header/tag name. The header/tag name is not modified, and the header/tag value is replaced.

- Insert. It can be used to add certain header/tag and when applicable, the associated value.

When multiple actions are requested on the target traffic, the enforcement of the Header Handling Actions is done in the following order: Detect, Remove, Replace, and Insert.

NOTE 5:	Header/Tag name is an agreed identifier between the operator and 3rd-party and is settled as part of the SLA. For example, it can take the actual name of an HTTP the header if HTTP is used.

NOTE 6:	The execution order is relevant to build a request that has the intended impact on the traffic. For example, a header that is removed cannot be replaced.

The AF may include explicit actions in the request, or they can be implicit in the Header Handling Reference, that is, included in the predefined configuration.

For each Header Handling Action explicitly requested, the corresponding Header/Tag Information (see below) is also provided. When applicable, Header Handling Condition(s) (see below) may also be provided.

4)	Header/Tag Handling Condition:

How to apply the action. It may have one of the following values: always or the first match of the action.

NOTE 8:	In addition, Spatial Validity Condition and Temporary Validity Condition parameters in the request can be used to indicate that the request applies only in certain UE location (s) or during certain time interval respectively.

A Header /Tag Handling Condition may be provided for a Header Handling Action explicit in the request. For actions implicit in the Header/Tag Handling Reference, the header/tag Handling Condition is included in the predefined configuration if applicable.

5) 	Header/Tag Handling Direction:

Header Handling Direction indicates to which direction the Header/Tag handling Control information applies to. Either uplink (UL) or downlink (DL).

6)	Header/Tag Information:

The Header/Tag Information contains input information to construct the Header(s) for the Header Handling Actions. AF may provide the Header/Tag Information using one or more of the following:

-	Header/Tag Name and Header/Tag Value: Strings that UPF uses to build header name and header value according to the syntaxis of the protocol selected. For example, AF provides Header/Tag Name = path, and Header/Tag Value= domainX.com. Header/Tag Names are not standardised and they are based on local configuration and SLA.

To detect a header/tag, the Header/Tag Name is required and Header/Tag Value is optional.

To remove a header/tag, the Header/Tag Value is not needed. Depending on the action, AF may provide the Tag Name only, for example, Tag Name = Token, to request to remove an HTTP header with header name e.g. X-Token.

In addition, AF may request a Header/Tag Name without requesting a Header/Tag Value. This is the case of information only known by the 5GC functions also on a per PDU basis. In such a case, the 5GC shall provide the actual Header/Tag Value. E.g. AF provides Tag Name = RAT and SMF, based on session information and predefined configuration, sends to UPF Tag Name= RAT and Tag Value=NR.

Editor’s Note: it is FFS whether some use case requires additional alternatives for Header/Tag information, for example a Header Reference.

Header/Tag Information is provided for every explicit action included in the request. Header/Tag Information is provided for implicit actions (preconfigured in the system for the Header Handling Reference) if needed.

7)	Information on AF subscription to UP Handling of Header event:

The AF may subscribe to receive reporting of handling of header execution from UPF (directly or via NEF), that is, when requested detection, removal, replacement or insertion of a header is performed by UPF. The AF subscription information may includes:

-	Notification endpoint (including correlation id) of the consumer of the subscription.

-	Reporting Frequency. The actions can be requested to be notified only once for the PDU session, periodically (in this case, a reporting period is also provided) or whenever the action is enforced. In the later case correlation applies to avoid reporting for each PDU.

-	Reporting Suggestion Information as defined in clause 5.8.2.17 to reduce the UPF performance impacts.

The SMF may have a pre-configuration for the Header/Tag Handling Reference received from PCF. SMF checks the Header/Tag information received and adds Header/Tag values if needed according to its local configuration and session information before sending the header/tag handling information to UPF.

NOTE 9: SMF can only add the Header/Tag values when it has the information.

The UPF checks whether it has the pre configuration for the Header/Tag Handling Reference received from SMF, and whether any Tag values still need to be determined by UPF. UPF performs handling of headers/tags as described earlier in this clause.

NOTE 10:	It is left for UPF implementation how to solve any misalignment between the header/tag handling information received from SMF and the predefined configuration corresponding to the Header Handling Reference.

The PCF sends the Information on AF subscription to notification of UP Handling of Header event to SMF if any has been received. The SMF sends the Notification target address for receiving event notifications to UPF. If SMF determines based on local configuration that it shall receive the notifications, it instructs UPF to duplicate the reporting and subscribes to the UPF event exposure service.

# * * * * 2nd change (all new) * * * *

#### 5.8.2.8.x 	Handling of Headers/Tags

Compared to Header Enrichment, Handling of Headers/Tags allows more actions than insert and Handling of Headers/Tags can be applied in uplink and in downlink. UPF is instructed to perform Handling of Headers/Tags by SMF upon request by the AF via PCF, and it is not limited to HTTP headers. In addition, it supports header handling related notifications e.g. upon detection of a header.

When UPF receives a request for Handling of Header/Tag from SMF in FAR, it contains a Header Handling Reference which points to a predefined configuration in UPF which includes the details of the protocol where header/tag handling shall be performed (e.g. HTTP). The mechanism used to perform Handling of Headers/Tags in the UPF is left for UPF implementation and it is established by SLA between operator and third party. In addition to the Header/Tag Handling Reference, UPF may receive Header/Tag Handling Additional Data to complement the predefined configuration.

The Handling of Header/Tag request may contain explicit header handling actions to perform on the traffic (there may be one or more of Detect, Remove, Replace and Insert) and conditions. Header/Tag Actions and Header/tag condition can also be included in the predefined configuration (that is, they are implicit in the Header Handling Reference). To enforce the actions requested, UPF uses the information received in the Handling of Headers Control information, see clause 5.6.x.

The Handling of Header/Tags request may include a request to UPF to report when a header handling action is performed. UPF event exposure service is used for reporting. The AF may request to receive Handling of Headers/Tags reports (directly or via the NEF). By local policy the SMF may determine it should receive Handling of Headers/Tags reports, irrespective of if the AF has requested Handling of Headers/Tags reports. In this case the SMF subscribes to the UPF event exposure service for receiving Handling of Headers/Tags reports, either implicitly via PFCP as specified in TS 29.244 [65] or via Nupf_EventExposure Service. If both the AF and the SMF have subscribed to the UPF event exposure service for receiving Handling of Header/Tags reports the UPF sends the notifications to both the AF (optionally via the NEF) and the SMF.

Handling of Headers/Tags is further described in clause 5.6.x. Handling of Headers/Tags is further described in clause 5.6.x.

# * * * * 3rd change * * * *

#### 5.8.5.6	Forwarding Action Rule

The following table describes the Forwarding Action Rule (FAR) that defines how a packet shall be buffered, dropped or forwarded, including packet encapsulation/decapsulation and forwarding destination.

Table 5.8.5.6-1: Attributes within Forwarding Action Rule

| Attribute | Description | Comment |
| --- | --- | --- |
| N4 Session ID | Identifies the N4 session associated to this FAR. | NOTE 9. |
| Rule ID | Unique identifier to identify this information. |  |
| Action | Identifies the action to apply to the packet | Indicates whether the packet is to be forwarded, duplicated, dropped or buffered. When action indicates forwarding or duplicating, a number of additional attributes are included in the FAR. For buffering action, a Buffer Action Rule is also included and the action can also indicate that a notification of the first buffered and/or a notification of first discarded packet is requested (see clause 5.8.3.2). For drop action, a notification of the discarded packet may be requested (see clause 5.8.3.2). |
| Network instance (NOTE 2) | Identifies the Network instance associated with the outgoing packet (NOTE 1). | NOTE 8. |
| Destination interface (NOTE 3) (NOTE 7) | Contains the values "access side", "core side", "SMF", "N6-LAN", "5G VN internal". | Identifies the interface for outgoing packets towards the access side (i.e. down-link), the core side (i.e. up-link), the SMF, the N6-LAN (i.e. the DN), or to 5G VN internal (i.e. local switch). |
| Outer header creation (NOTE 3) | Instructs the UP function to add an outer header (e.g. IP+UDP+GTP, VLAN tag), IP + possibly UDP to the outgoing packet. | Contains the CN tunnel info, N6 tunnel info or AN tunnel info of peer entity (e.g. NG-RAN, another UPF, SMF, local access to a DN represented by a DNAI) (NOTE 8). Any extension header stored for this packet shall be added. The time stamps should be added in the GTP-U header if QoS Monitoring for packet delay is enabled for the traffic corresponding to the PDR(s). |
| Send end marker packet(s) (NOTE 2) | Instructs the UPF to construct end marker packet(s) and send them out as described in clause 5.8.1. | This parameter should be sent together with the "outer header creation" parameter of the new CN tunnel info. |
| Transport level marking (NOTE 3) | Transport level packet marking in the uplink and downlink, e.g. setting the DiffServ Code Point. | NOTE 8. |
| Forwarding policy (NOTE 3) | Reference to a preconfigured traffic steering policy or http redirection (NOTE 4). | The Forwarding policy refers to a preconfigured forwarding behaviour in UPF, which may be related to: - N6-LAN steering to steer the subscriber's traffic to the appropriate N6 Service Functions deployed by the operator; - local N6 steering to enable traffic steering in the local access to the DN according to the routing information provided by an AF as described in clause 5.6.7; - a Redirect Destination and values for the forwarding behaviour (always, after measurement report (for termination action "redirect")). |
| Metadata (NOTE 10) | Metadata the UPF needs to add to traffic sent over a SFC. | The metadata information is associated with a TSP ID related to N6-LAN steering. |
| Request for Proxying in UPF | Indicates that the UPF shall perform ARP proxying and / or IPv6 Neighbour Solicitation Proxying as specified in clause 5.6.10.2. | Applies to the Ethernet PDU Session type. |
| Container for header enrichment (NOTE 2) | Contains information to be used by the UPF for header enrichment. | Only relevant for the uplink direction. |
| Buffering Action Rule (NOTE 5) | Reference to a Buffering Action Rule ID defining the buffering instructions to be applied by the UPF (NOTE 6) |  |
| Handling of Headers/Tag policy | Contains the information that UPF needs to perform handling of headers as described in 5.6.x. It may indicate that the UPF shall report when a header/tag handling action is performed (NOTE 11). |  |
| NOTE 1: Needed e.g. if: - UPF supports multiple DNN with overlapping IP addresses; - UPF is connected to other UPF or NG-RAN node in different IP domains; - UPF “local switch” and N19 forwarding is used for different 5G LAN groups. NOTE 2: These attributes are required for FAR action set to forwarding. NOTE 3: These attributes are required for FAR action set to forwarding or duplicating. NOTE 4: The TSP ID is preconfigured in the SMF and used to determine the Forwarding Policy included in the FAR according to the description in clause 5.6.7 and clause 6.1.3.14 of TS 23.503 [45] for local N6 steering and in clause 5.6.16 and clause 6.1.3.14 of TS 23.503 [45] for N6-LAN steering. The Forwarding Policy action is enforced before the Outer header creation actions. NOTE 5: This attribute is present for FAR action set to buffering. NOTE 6: The buffering action rule is created by the SMF and associated with the FAR in order to apply a specific buffering behaviour for UL/DL packets requested to be buffered, as described in clause 5.8.3 and clause 5.2.4 of TS 29.244 [65]. NOTE 7: The use of “5G VN internal” instructs the UPF to send the packet back for another round of ingress processing using the active PDRs pertaining to another N4 session of the same 5G VN group. NOTE 8: When in architectures defined in clause 5.34, a FAR is sent over N16a from SMF to I-SMF, the FAR sent by the SMF may indicate that the I-SMF is to locally determine the value of this attribute in order to build the N4 FAR rule sent to the actual UPF controlled by the I-SMF. This is further defined in clause 5.34.6. NOTE 9: In the architecture defined in clause 5.34, the rules exchanged between I-SMF and SMF are not associated with a N4 Session ID but are associated with a N16a association. NOTE 10: The use of Metadata is described in clause 5.6.16. How the UPF transforms the Metadata into actual information sent with the traffic (e.g. in the encapsulation header) is based on local policies related with the Forwarding Policy and not specified. NOTE 11: When the UPF is requested to report when a header/tag handling action is performed the UPF sends the events via Nupf_EventExposure_Notify service operation to the notification endpoint (including correlation ID) of the Target of Reporting NF/AF (observe that the SMF also can be a subscriber). |  |  |

# * * * * 4th change * * * *

### 6.2.3	UPF

The User plane function (UPF) includes the following functionality. Some or all of the UPF functionalities may be supported in a single instance of a UPF:

-	Anchor point for Intra-/Inter-RAT mobility (when applicable).

-	Allocation of UE IP address/prefix (if supported) in response to SMF request.

-	External PDU Session point of interconnect to Data Network.

-	Packet routing & forwarding (e.g. support of Uplink classifier to route traffic flows to an instance of a data network, support of Branching point to support multi-homed PDU Session, support of traffic forwarding within a 5G VN group (UPF local switching, via N6, via N19)).

-	Packet inspection (e.g. Application detection based on service data flow template and the optional PFDs received from the SMF in addition).

-	User Plane part of policy rule enforcement, e.g. Gating, Redirection, Traffic steering).

-	Lawful intercept (UP collection).

-	Traffic usage reporting.

-	QoS handling for user plane, e.g. UL/DL rate enforcement, Reflective QoS marking in DL.

-	Uplink Traffic verification (SDF to QoS Flow mapping).

-	Transport level packet marking in the uplink and downlink.

-	Downlink packet buffering and downlink data notification triggering.

-	Sending and forwarding of one or more "end marker" to the source NG-RAN node.

-	Functionality to respond to Address Resolution Protocol (ARP) requests and / or IPv6 Neighbour Solicitation requests based on local cache information for the Ethernet PDUs. The UPF responds to the ARP and / or the IPv6 Neighbour Solicitation Request by providing the MAC address corresponding to the IP address sent in the request.

-	Packet duplication in downlink direction and elimination in uplink direction in GTP-U layer.

-	NW-TT functionality.

-	High latency communication, see clause 5.31.8.

-	ATSSS Steering functionality to steer the MA PDU Session traffic, refer to clause 5.32.6.

NOTE: Not all of the UPF functionalities are required to be supported in an instance of user plane function of a Network Slice.

-	Inter PLMN UP Security (IPUPS) functionality, specified in clause 5.8.2.14.

-	Event exposure, including exposure of network information, i.e. the QoS monitoring information, as specified in clause 5.8.2.18, events as specified in clause 5.2.26.2 of TS 23.502 [3], exposure of data collected for analytics, as specified in clause 5.2.26.2 of TS 23.502 [3] and exposure of the TSC management information as specified in clause 5.8.5.14

-	Exposure of the UE information, e.g. UE IP address translation information as specified in clause 5.2.26.3 of TS 23.502 [3] and clause 4.15.10 of TS 23.502 [3] if Network address translation (i.e. NAT) functionality of the UE IP address is deployed within UPF.

-	Support PDU Set Handling as defined in clause 5.37.5.

-	Handling of Headers as defined in clause 5.8.2.8.x.

# * * * * End of changes * * * *