3GPP TSG-SA2 Meeting #164	S2-2407806

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1138 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Vertical Federated Learning (VFL) between NWDAF(s) and AF(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, CATT, ZTE, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on conclusions for KI#2: 5GC Support for Vertical Federated Learning in clause 8.2 of TR23.700-84, this CR aims to introduce the VFL between NWDAF(s) and AF(s) in 5GC. TS 23.288 provides NWDAF specification support for HFL but no VFL support is available, and the existing procedures defined for HFL could be enhanced to support VFL, as mentioned in clause 5.2.4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 5.X is added to specify general introduction for vertical federated learning between AF(s) and NWDAF(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL between NWDAF(s) and AF(s) is not introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

5.X	Verticle Federated Learning (VFL) between NWDAF(s) and AF(s)

Vertical Federated Learning (VFL) is a federated learning technique wherein the local data set in different participant for local model training have different feature spaces for the same samples (e.g. UE IDs). Due to the data privacy and data heterogeneity, huge amount data of 5GC and AF cannot be fully utilized. Therefore, VFL between NWDAF(s) and AF(s) could be introduced to address the above problems, enabling the joint ML model training without any local data sharing based on the data of different features.

For Vertical Federated Learning supported by NWDAF(s) and AF(s), either the NWDAF or the AF can act as VFL server and initiate VFL training or inference process with the VFL client(s).The NWDAF as VFL server will select candidate NWDAF(s) and /or AF(s) (via NEF profile for untrusted AF) as VFL client(s). The AF (via NEF in case of untrusted AF) as VFL server will select candidate NWDAF(s) as VFL client(s).

VFL related new functionalities include:

-	VFL Server: An NWDAF or AF that integrates local training results for the local ML model update in VFL training process. It also coordinates the VFL training process by discovering and selecting VFL clients. In VFL inference process, The VFL server aggregates local inference results from VFL clients to generate the final VFL inference result and sends the final VFL inference result to the consumer. Only one VFL server exists for each VFL process.

-	VFL Client: An NWDAF or AF that holds the local dataset and performs local training and inference as asked by VFL Server. There can be multiple VFL Clients in VFL training and inference.

-	VFL process is associated with an analytics ID (i.e. VFL training of models for analytics ID, VFL inference for an analytics ID) when VFL server is the NWDAF.

-	VFL process is associated with internal AF process (i.e. VFL training of models for internal AF process, VFL inference for internal AF process) when VFL server is AF.

-	5GC shall support vertical federated learning (VFL), i.e. a federated learning technique without exchanging/sharing local data set or ML models, in the following scenarios:

-	VFL among NWDAFs in a single PLMN.

-	VFL between AF and NWDAF(s) in a single PLMN.

The NWDAFs as VFL Server determines based on internal logic and the operator's policy whether or not to use VFL to provide a particular analytics ID.

-	NWDAF and AF are the only NFs that may act with any of the above VFL functionalities, i.e. VFL Server and VFL client.

# >>>>END OF CHANGES<<<<