3GPP SA WG2 Meeting #164	S2-2407817

Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024	Revision of S2-2406313

Source: 	China Mobile

Title: 	New solution on support of Ambient IoT services for supply chain for KI#1 and KI#2

Document for: 	Approval

Agenda Item: 	19.14.1

Work Item / Release:	FS_AmbientIoT /Rel-19

Abstract: This contribution is to provide a new solution to address the KI#1 and KI#2 for Ambient IoT services for supply chain.

1. Introduction/Discussion

This contribution is proposed as a solution to address KI#1 and KI#2 for Ambient IoT services for supply chain such as automated supply chain distribution and fresh food supply chain depicted in TR 22.840.

- Text Proposal

It is proposed to capture the following changes in TR 23.700-13.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | Key Issue #1 | Key Issue #2 | Key Issue #3 |
| #1 |  | X |  |
| #2 |  | X |  |
| #3 | X | X | X |
| #4 | X | X | X |
| #5 | X | X | X |
| #6 | X | X | X |
| #7 | X | X |  |
| #8 | X | X |  |
| #9 | X | X | X |
| #10 | X | X |  |
| #11 |  |  | X |
| #12 | X | X | X |
| #13 |  | X |  |
| #14 |  | X |  |
| #15 |  | X | X |
| #16 |  | X |  |
| #17 | X | X | X |
| #18 |  |  | X |
| #19 | X |  | X |
| #20 | X | X | X |
| #21 | X | X | X |
| #22 | X | X | X |
| #23 |  |  | X |
| #24 |  | X |  |
| #25 |  | X |  |
| #26 |  |  | X |
| #27 |  |  | X |
| #28 |  |  | X |
| #29 | X | X |  |
| #30 | X | X | X |
| #31 |  | X | X |
| #32 | X |  | X |
| #33 | X | X | X |
| #34 |  | X |  |
| #X | X | X |  |

# * * * * Second change (all new) * * * *

6.X	Solution #X: New solution for support of Ambient IoT services for supply chain

6.X.1	Description

This solution resolves Key Issue KI#1 and KI#2 for Ambient IoT services for supply chain such as automated supply chain distribution and fresh food supply chain as depicted in TR 22.840.

During the whole process of production and sales, the Ambient IoT devices are transported to different companies, areas or countries. The devices need to access local network where it is located. If the serving operator restricts access to the devices from another operator without SLA agreement, the devices cannot work normally. To solve this problem, a wildcard can be configured in the field of Operator ID for devices applied for supply chain services based on SLA agreement between the third Ambient IoT AF and serving operator.

As the Ambient IoT Devices for supply chain services possibly move anywhere, it is difficult to authenticate and authorize the devices in the serving network. That is because the serving network cannot acquire the credentials or subscription data if there is no roaming agreement between the home network and the serving network. What’s more, the serving network is not aware of the home network if the information of home PLMN is not contained in the device ID or signaling message. Thus, for supply chain service the credential holder can be placed in the third AF. To support the supply chain service or registration for the whole serving zone, the AF should sign SLA with the Operator. It is supposed that AF can enforce authentication and authorization by itself or with the help of other AF.

In order to keep a competitive solution compared to the RFID solution, it is possible to trigger not only one or a few ambient IoT devices defined by the TID list but also numerous unregistered ambient IoT devices in the whole serving zone in the AF-triggered registration procedure. Then, it is possible to perform an automatic AF-triggered registration procedure for unregistered ambient IoT devices in the whole zone during the middle of the night without human resources involved. However, there is no need for ambient IoT devices to keep dedicated status info.

The principles are depicted below:

-  A new network function named Ambient IoT NF may be adopted to manage Ambient IoT devices and procedures. If not, this relevant function can be supported by AMF.

-	In 5GC, each Ambient IoT device has a unique device ID that contains the Operator ID, Group ID and Instance ID. For Devices applied in supply chain, the field of Operator ID can be filled with a wildcard to represent the Device Type of Supply Chain based on SLA agreement between the third Ambient IoT AF and serving operator, i.e. 999.

-  The device ID is kept internally in 5GS and ambient IoT devices for access control, identification, and route steering purposes. It will not be exposed to the third AF.

-  Before registration, the Ambient IoT device and Credential holder are pre-configured with the Ambient IoT default Device ID which contains Operator ID only, TID, and credentials.

- X.2	Procedures

#### 6.X.2.1 Procedure of AF triggered registration for Topology 1

The following figure presents a procedure of AF triggered registration for Topology 1 for supply chain devices.

8. Performs access control based on the Operator ID List and device status. Determine credential holder based on the default Device ID

AMF/New

Ambient IoT NF

6. The NG-RAN Reader activate AIoT devices; Matched devices perform registration procedures with the default device ID, TID, and Credential, without Operator ID.

3. AF Triggered Registration Request (Operator ID list filled with a wildcard, TID list,TA list…)

Ambient AF

fd

UDM

NEF

NG-RAN

Reader

Ambient IoT

Devices

1. AF Triggered Registration Request (TID list, Operator ID list filled with a wildcard, Location, …)

0.a pre-configuration

4.Obtain targeted NG-RAN Reader based on TA lists

5. AF Triggered Registration Request (TID list…)

10. Store Device ID, TID, Status ...

11. AF triggered Registration Response (TID，Status, …)

2.AF authentication and authorization

7. AF Triggered Registration Response ( Default Device ID, TID, and credential, …)

Credential Holder

0.b pre-configuration

re-configuration

9. Authentication based on TID and credential and Produce real Device ID

Figure 6.X.2-1 AF triggered Registration Procedure for Topology 1

0.a	The Ambient IoT devices are pre-configured with default Device ID and credentials. TID (See GS1 TDS Release 2.1 [10]) is configured by AIoT manufacturers in advance.

0.b Meanwhile, the third credential holder is also pre-configured with the TID, default credentials, status (inactive), and so on.

- AF sends an AF Triggered Registration Request to NEF with the following parameters: Transaction ID, TID  list, Operator ID list, location, AF ID, time, periodical indication, and so on.

-	Operator ID list: For supply chain service, this field is filled with a wildcard representing the Device Type of Supply Chain. With the information, all the inactive devices applied in supply chain can access the network no matter which operator it belongs to..

-	TID list: The information can be used to define the targeted AIoT devices. If the TID list is carried, it means AF wants to trigger one or a few AIoT devices defined by the TID list to perform the registration. If not, it means AF wants to trigger all Ambient IoT devices in this location to perform the registration.

- NEF performs the below actions towards the third Ambient AF.

-	Perform authentication to the third Ambient AF to decide whether it is allowed to access 5GS or not.

-	Check the authorization to determine whether the third Ambient AF is allowed to perform an AF triggered registration operation or not.

-	Check the authorization to determine whether supply chain service and multiple operators’ device access are allowed or not.

-	Converting location information to TA List information.

-      The NEF obtains serving AMF or Ambient IoT NF based on the TA list.

3.	The NEF sends AF Triggered Registration Request to the AMF/Ambient IoT NF, including the Transaction ID, Operator ID list, TID list ID, TA list, time, periodical indication, and so on. For supply chain service, the Operator ID list field is filled with a wildcard representing the Device Type of Supply Chain.

4.	The AMF/Ambient IoT NF selects the NG-RAN reader based on the TA list.

5.	The AMF/Ambient IoT NF forwards the AF triggered registration request to the NG-RAN reader, including the TID list, time, periodical indication, and so on.

6.	NG-RAN activates one or a few targeted ambient IoT devices. The matched AIoT devices respond with the default device ID, TID, and credentials. If the TID list is not carried by the NG-RAN reader, all AIoT devices execute the registration procedure.

7.	The NG-RAN reader forwards the registration messages with the default device ID, TID, and credentials to AMF/Ambient IoT NF.

- Upon receiving the registration message from NG-RAN, AMF/New AIoT NF performs access control based on the Operator ID List, the status of devices and default Device ID. AMF/New AIoT NF also selects the credential holder owned by the third AF.

- Perform an authentication operation based on TID as username and credentials between AMF/New AIoT NF and the credential holder. Once authentication is successful, a Real device ID with a unique and non-zero instance ID will be produced.

- 5GC stores the new device ID, TID, status (active), serving NG-RAN or AIoTNF info, and so on in UDM, AMF/New AIoT NF. The status information will be used in the Ambient IoT service procedures to verify the Ambient IoT device.

- The AMF/New AIoT NF returns AF Triggered Registration Response to the AF via NEF.

6.X.2.2	Procedure for AF triggered registration for Topology 2

The registration may be triggered by AF via the UE reader. The UE reader interacts with AMF /Ambient IoT NF via NG-RAN, which could be regarded as an AF triggered registration procedure for topology 2.

AMF/New

Ambient IoT NF

7. UE Readers activates AIoT devices. Matched devices respond with the default device ID, TID, and credentials. If the TID list is not carried, all devices in the UE reader’s serving area respond.

4. AF Triggered Registration Request (Operator ID list, TID list, location list, …)

5. Determine targeted UE readers

Ambient AF

UDM

NEF

UE

Reader

Ambient IoT

Devices

2. AF Triggered Registration Request (TID list, Operator ID list filled with a wildcard…)

1.UE Reader registration with UE reader type indication

6. AF Triggered Registration Request (TID List, …)

9. AF triggered Registration Response (TID, Status, …)

3. AF authentication and authorization

8. AMF/New AIoTNF performs access control based on the Operator ID List and device status. It also determines credential holder based on the default Device ID. Credential holder does authentication based on TID and credential and produces new Device ID. UDM and AMF/ New AIoT NF store New Device ID, TID and Status.

Credential Holder

The following figure presents a procedure of network triggered registration for Topology 2 for supply chain devices.

Figure 6.X.2-2: AF triggered Registration Procedure for Topology 2

1. The UE Reader performs registration with the fixed or mobile type of UE reader, precise location, AF ID, and Store ID.

2. AF sends the AF Triggered Registration Request to NEF with the following parameters: Transaction ID, TID  list, Operator ID list, location, AF ID, Store ID, Aggregation indication information, time, periodical indication, UE GPSI, and so on. For supply chain service, the field of Operator ID list is filled with a wildcard representing the Device Type of Supply Chain. With this information, all the devices applied in supply chain can access the network no matter which operator it belongs to.

If UE GPSI is included in the message, the third AF wants to perform an AF-triggered registration procedure with a designated UE reader defined by UE GPSI. If not, 5GS determines the involved UE readers based on input info from the third AF, such as location, AF ID, store ID, and so on. Considering UE reader posture, such as UE reader antenna pointing direction, only fixed-type UE readers will be involved in the procedure.

- NEF will perform the below actions towards the third Ambient AF

-	Perform authentication to the third Ambient AF to decide whether it is allowed to access 5GS or not.

-     Check the authorization to determine whether the third Ambient AF is allowed to perform an AF-triggered registration operation.

-	Check the authorization to determine whether supply chain service and multiple operators’ devices access are allowed or not.

-	Converting location information to TA List information.

-      The NEF obtains serving AMF or Ambient IoT NF based on the TA list.

- The NEF sends AF Triggered Registration Request to the AMF/Ambient IoT NF, including the Transaction ID, Operator ID list, TID list ID, TA list, AF ID, Store ID, aggregation indication, time, periodical indication, UE GPSI and so on. For supply chain service, the Operator ID list field is filled with a wildcard representing the Device Type of Supply Chain.

- If UE GPSI isn't included in the message, the AMF/Ambient IoT NF selects the fixed-type UE readers based on the Location list, AF ID, and Store ID. Alternatively, if UE GPSI is carried in the message, the AMF/Ambient IoT NF selects the designated UE reader based on UE GPSI.

- First, the AMF/Ambient IoT NF sets up the connection with targeted UE readers. Then, it forwards the AF-triggered registration request to the UE reader with the TID list, time, periodical indication, and so on. For supply chain service, the Operator ID list field is filled with a wildcard.

- The UE readers activate one or a few targeted AIoT devices to perform registration. The matched devices respond with the default device ID, TID, and credentials. If the TID list is not carried in the message, all AIoT devices in the UE readers’ serving area execute the registration procedure.

- AMF/New AIoT NF performs access control based on the Operator ID List, the status of devices and default Device ID. AMF/New AIoT NF Determines the credential holder owned by the third AF based on the default Device ID. The credential holder authenticates the devices based on the TID and credential. If authentication is successful, 5GS produces a new Device ID. At the same time, new Device ID, TID, Status (active), serving NG-RAN or AIoTNF info, and so on are stored in UDM and AMF/ New Ambient IoT NF.

- The AMF/New AIoT NF returns AF Triggered Registration Response to the AF via NEF.

6.x.3	Impacts on services, entities, and interfaces

This solution impacts the following entities.

NEF:

-	Capability to authentication and authorization to the third ambient IoT AF.

-	Capability to support verification on operator ID list.

AMF/Ambient IoT NF:

-	The AMF/Ambient IoT NF selects the NG-RAN reader and/or UE reader based on the TA list, Location, AF ID, and store ID.

-   Capability to aggregate the Ambient IoT devices' response.

UDM/Credential holder:

-	The UDM stores the profile information of Ambient IoT devices.

-   Authenticating the ambient IoT device and producing new device IDs

NG-RAN Reader:

-	The NG-RAN reader performs paging and receives responses from Ambient IoT devices in Topology 1.

UE Reader:

-	The UE reader performs paging and receives responses from Ambient IoT devices in Topology 2.

Ambient IoT device:

-	The ambient IoT device stores the default AIoT device ID and credentials.

-  Performing registration procedure once it is matched during the paging procedure.

# * * * * End of changes * * * *