3GPP TSG-WG2 Meeting #164	 S2-2407819

# Maastricht, Netherlands, 19 August – 23 August, 2024                   (revision of S2-2xxxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0124 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support multiple USS serving different geographical areas corresponding to the UAV flight path |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The CR intends to support multiple USS serving different geographical areas corresponding to the UAV flight path based on the conclusion of clause 8.1.2 in TR 23.700-59. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the descriptions and procedures for supporting multiple USS serving different geographical areas corresponding to the UAV flight path. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new functionality of supporting multiple USS serving different geographical areas corresponding to the UAV flight path can’t be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

5.X

5.x.1	Functional Description

3GPP network provides the functionality to support the scenario of multiple USS serving different geographical areas corresponding to the UAV flight path via the service exposure towards USS. The USS changeover is supported for the pre-mission flight planning and during UAV flight.

5.x.2	Procedures

5.x.2.1	Procedure for pre-mission flight planning for USS changeover

Figure 5.x.2.1-1: Pre-mission flight planning for USS changeover

1.	[Optional] The steps 1-2 in clause 5.x.2.1 Pre-mission Flight Planning with assistance of 5GC are performed.

2.	If the serving USS (i.e. USS 1) determines that the waypoint(s) within the flight path for specific UAV is outside its service area, it selects the suitable candidate target USS (e.g. USS 2), and triggers communication with each of the candidate target USS to determines candidate border-crossing point(s) for the UAV.

NOTE 1:	How a serving USS determines the target USS and a list of candidate border-crossing points between the serving USS and potential target USS is out of scope of the 3GPP.

3.	The serving USS sends a Pre-mission flight planning assistance request to the UAS NF/NEF. The request may include 3GPP UAV ID (e.g. GPSI), information of the flight path, a list of candidate border-crossing point(s), service area of USS (e.g. serving USS and neighboring USS), and candidate flight path(s) (if available).

4.	After receiving the Pre-mission flight planning assistance request from the serving USS, the NEF/UAS NF may map the parameters (e.g. candidate border-crossing point(s), service area(s)) to 3GPP identifiers (e.g. a list of Cell IDs, gNB IDs or TAIs).

5.	The NEF/UAS NF determines the relevant NFs and service needed to collect the required information for the serving USS for UAV flight planning (e.g. NWDAF analytics service for Movement Behaviour analytics, GMLC service for Ranging/Sidelink Positioning location, AMF for UAV's presence in bordering cells/TAs, AMF service for UAV's deviation for the expected/assigned trajectory).

The NEF/UAS NF can discover the AMF(s) serving the cell IDs/TAIs comprising the candidate border-crossing point(s) as described in clause 5.3.4.4 of TS 23.501 [2] to subscribe UAV's presence.

The NEF/UAS NF invokes service operations towards the identified NFs.

6.	The NEF/UAS NF sends the Pre-mission flight planning assistance response to the serving USS based on the collected information. This information includes the best matched route among the ones provided from the USS/UTM and candidate USS border-crossing point(s).

7.	Based on the Pre-mission flight planning assistance response, USS 1 selects a target USS from a list of pre-selected suitable candidate target USS in step 2 and starts communication to request the target USS  to perform the flight planning for the UAV.

NOTE 2: How to perform the flight planning between the serving USS and target USS is out of scope of the 3GPP.

5.x.2.2	Procedure for USS changeover during UAV flight

Figure 5.x.2.2-1: USS changeover during UAV flight

1a.	The serving USS (i.e. USS 1) requests UAV presence monitoring events whether the UAV location is in the border-crossing point(s) via NEF/UAS NF by invoking Nnef_EventExposure_Subscribe service operation.

The NEF/UAS NF maps the border-crossing point(s) into an area of interest that is represented by a list of Cell IDs/TAIs, and subscribes relevant AMF as described in clause 5.3.3.

1b.	If the UAV moves into a border-crossing point (i.e. border cell/TA), the AMF notifies the NEF/UAS NF about the event.

2.	If the NEF/UAS NF receives the notification about UAV moving into a border cell/TA, the NEF/UAS NF may determine the UAV has left based on current location (e.g. presence in a bordering cell/TA of a new USS), or is about to leave the service area of the serving USS via collecting the required information (e.g. NWDAF analytics service for Movement Behaviour analytics, GMLC service for Ranging/Sidelink Positioning location, AMF service for UAV’s deviation for the expected/assigned trajectory).

3.	If the NEF/UAS NF determines that the UE has left or is about to leave service area served by the serving USS, the NEF/UAS NF sends notification to serving USS that the UAV has left or is about to leave the service area (i.e. the USS changeover is needed for the UAV) by invoking Nnef_EventExposure_Notify service operation.

4.	The serving USS communicates with the target USS (i.e. USS 2) to perform the changeover for the UAV.

NOTE 1: Interaction between serving USS and target USS is out of scope of the 3GPP. It is expected that a serving USS pass the target USS the identifiers of UAV (e.g. 3GPP UAV ID) and UAV context etc.

5. The serving USS informs the UAV about the changeover to the target USS and provides target USS address, and optionally triggers the exposure services towards 5GC NFs similar to what the previous serving USS had before the changeover.

6.	If the AMF serving a border cell/TA on target USS’side notifies a serving NEF/UAS NF about the UAV presence in the border cell/TA from target USS’s side, and the information transferred in step 4 has been completed, the target USS may respond to the notification conforming to the network about the completion of the changeover for the UAV.

7a. The UAV triggers UUAA procedure as described in clause 5.2.2 or 5.2.3. The CAA Level UAV ID (same CAA Level UAV ID for the UAV accorss USS 1 and USS 2) and target USS address are provided by UAV.

7b. The target USS initiates UUAA Re-authorization procedure via NEF/UAS NF as described in clause 5.2.4.3 with following modifications:

In step 1, the target USS sends a Naf_Authentication_Notification request to UAS NF for re-authentication of the UAV. The USS includes target USS address, CAA Level UAV ID (same CAA Level UAV ID for the UAV accorss USS 1 and USS 2) transparently delivered to the UAV. And the USS informs the NEF/UAS NF that the UAV Re-authorization is due to USS changeover.

>>>> End of Changes <<<<